Conventional & Special Core Analysis

Draft Final Report

Client: Waha Oil Company

Well: T1-31 (IRIS PROSPECT - EAST CON31)

Report Number: 02/PET1-1&2/24 PETROLEUM RESEARCH CENTER (PRC) SERVICES – CONSULTANCY – TRAINING GARGARISH ROAD KM7 TRIPOLI, LIBYA INFO@PRCLIBYA.COM TEL: +218 21 486 0022 FAX: +218 21 483 0031

PRC Work Order No: 190/PET1-1&2/24

Date of Issue: January 15, 2026

Prepared by: –

    - Eng. Muhanad S. Ikaybah

    - Eng. Aisha Al-Shaafi

Team Work: –

- Eng. Muad Abdullah

- Tech. Omar Alhaknoun

- Eng. Moataz Elheshri

Approved by: - Ahmed El-Naami

Head of Reservoir Rock Section

All copyrights are reserved to Waha Oil Company. Neither this report nor any part may be reproduced or transmitted in any means, electronic or mechanical, including photocopying, microfilming and recording or by any information storage and retrieval system, nor the included data and results used, without the written permission of Waha Oil Company.

التسجيل  :                                                                                                                                          التاريخ :29//071447ه

الملف   :                                                                                                                                        الموافق :15/01/2026م

RE: Conventional and Special Core Analysis

for Well #: T1-31

Dear Mohamed Ammar Haman \ Manager, Exploration Department

With reference to your letter dated on July 03, 2024, concerned with your request to carry out conventional and special core analysis for well No. T1-31, we thus hereby enclose a digital copy of the draft final report of the conventional and special core analysis which contains all the results performed on plugs from the subject well.

We hope that the report meets your requirements. Your remarks and suggestions shall be highly appreciated.

Muhieddin Ibrahim Hamza

Manager, Petroleum Engineering Research Dept.

Muhieddin Ibrahim Hamza

Manager, Petroleum Engineering Research Dep.

Table of Contents

|  | Contents | Pages |
| --- | --- | --- |
| 1 | Summary | 1 - 2 |
| Part One: Conventional Core Analysis |  |  |
| Section One: Whole Core Samples Analysis |  |  |
| 2 | Methods and Experiments (Tests According to API RP 40, Recommended Practice for Core Analysis) | 3 - 6 |
| 3 | Whole Core Samples Analysis Results | 7 |
| Section Two: Plug Samples Analysis |  |  |
| 4 | Methods and Experiments (Tests According to API RP 40, Recommended Practice for Core Analysis) | 8 - 14 |
| 5 | Conventional Core Analysis Results and Profiles | 15 - 20 |
| 6 | Statistical Analysis | 21 - 37 |
| 7 | Probe Permeability Results and Profile | 38 - 42 |
| 8 | Comparison between Probe & Routine Permeability Results | 43 |
| Part Two: Special Core Analysis |  |  |
| 9 | Porosity and Permeability as a Function of Overburden Pressure | 44 - 56 |
| 10 | Pore Volume Compressibility | 57 - 69 |
| 11 | Specific Water Permeability as Function of Throughput | 70 - 94 |
| 12 | Formation Resistivity Factor (Ambient Conditions) | 95 - 99 |
| 13 | Formation Resistivity Index | 100 - 114 |
| 14 | Formation Resistivity Factor (Overburden Pressure) | 115 - 143 |
| 15 | Oil-Water (Drainage Cycle) Capillary Pressure by Using Centrifuge Method | 144 - 148 |
| 16 | Water-Oil (Imbibition Cycle) Capillary Pressure by Using Centrifuge Method | 149 - 153 |
| 17 | Mercury Injection Capillary Pressure | 154 - 210 |
| 18 | Unsteady State Water-Oil Relative Permeability (At Reservoir Conditions) | 211 - 241 |
| 19 | Effective Oil Permeability at Initial Water Saturation under at Overburden Pressure | 242-243 |
| 20 | Determination of Young’s Modulus, Poisson’s ratio & Acoustic Velocity | 243-261 |
| 21 | Conclusions | 262-264 |
| 22 | References | 265 |

Summary

Summary

This report presents the results of the conventional core analysis for well No. T1-31. This well is located in the concession 31 and the core was recovered from Gir Formation.

A Total of (28 Horizontal and 10 Vertical) one and half inch diameter core plugs and 4 full diameter core samples were selected by Waha representative from one full diameter core, having a total core length of 30 feet, from well T1-31, these plugs were drilled by our staff in the presence of the Waha Oil Company representative. In addition to that, (24 horizontal) one-inch diameter sidewall plugs were received in our laboratory.

Conventional core analysis measurements have been done on core samples taken from the mentioned well, as per scope of work, these measurements were conducted by Petroleum Research Center laboratories and finally the results were processed and interpreted.

- The first part of this report contains the conventional core analysis which includes:

- The measurement of porosity, grain density, horizontal permeability (Ka, max and 90o) and vertical permeability under ambient conditions for four whole core samples which were received from Waha Oil Company representative.

- The measurement of porosity, grain density and air permeability for twenty-eight horizontal core plug samples of 1.5 inches and twenty-four horizontal sidewall plug samples of 1 inches which were selected from above mentioned well by Waha Oil Company representative. In addition to that, ten vertical core plug samples of 1.5 inches were selected to perform the measurement of air permeability. After that, the statistical analysis was calculated, then Probe permeability was measured.

- The hydraulic flow unit was not performed, because most of samples have very low permeability or fractured.

- The second part of the report includes the special core analysis which includes:

- Firstly, the porosity and permeability at various overburden pressures (5 pressures) were measured and the pore volume compressibility was determined from the data derived for each overburden pressure.

- After that the specific water permeability as function of throughput was measured.

- The electrical properties of the selected plug samples were measured, which include formation resistivity factor, cementation factor, formation resistivity index and saturation exponent.

- The oil-water and water-oil capillary pressure by ultra-speed centrifuge, (at ambient conditions) was carried out and the calculations were made using Hassler technique.

- Twelve samples were selected for the measurements of high pressure (up to 60,000 psi) mercury injection capillary pressure, drainage & imbibition, and pore size distribution.

- Two of one and half inch diameter core plugs were selected to measure water-oil relative permeability at reservoir conditions by using SMAX system, and five one-inch diameter sidewall plugs were selected also to measure water-oil relative permeability at reservoir conditions but without X-ray scan. Both tests were carried out by using mineral oil at reservoir temperature and overburden pressure. Relative permeability relationships at reservoir conditions were calculated from collected data, sample and fluid parameters by using Sendra software.

- After that the effective oil permeability was measured after establishment of initial water saturation.

- Finally, rock mechanics test was conducted on three samples, including one sample at unconfining and another two samples at confining stresses.

Part One

Conventional Core Analysis

Section One

Whole Core Samples Analysis Results

Methods and Experiments

Tests According to API RP 40, Recommended Practice for Core Analysis

ANALYTICAL PROCEDURES AND EQUIPMENT SPECIFICATION

Sample Preparation: -

Four whole core samples were cut from the core recovered from well T1-31 as requested by Waha Oil Company representative.

The whole core samples were extracted of hydrocarbons using toluene, leached of salt using methanol, and oven dried at 80 oC. Silver Nitrate solution was used to ensure the removal of salts existing in the pore spaces.

POROSITY AND GRAIN DENSITY MEASUREMENTS: -

- Core analysis laboratory of Petroleum Research Centre used a double cell helium expansion gas porosimeter for the sample’s measurement of porosity and grain density. The porosimeter operates using the principle of Boyle's Law.

- In the instrument, a sealed reference chamber is filled with helium gas at room temperature to a pressure of 450 psig. A sample is placed in a sealed chamber with plastic fillers, which is connected to the reference chamber by a two-way valve.

- This valve when opened, allows the gas in the reference chamber to expand into the combined volume of the two chambers. From Boyle's Law, the volume of the sample chamber can be calculated if the volume of the reference chamber, dead volume and fillers volume are known. This is the operation principle of the porosimeter.

- Before using the porosimeter to analyse a sample, the instrument must be calibrated. This was done by running a series of plastic blanks in the instrument, to build up a graph of blank volume versus the final reading, and the R2 was checked. Then the dead volume of the lines is measured.

- The grain volume is calculated by the following formula:

Grain volume = Total volume of fillers + Reading of the fillers – Fillers volume (with sample)

– Reading of the sample (with fillers)

- Bulk volumes were measured by Length and Diameter.

The formula for the calculation of the Bulk Volume is:

Bulk volume (cc) = πR2L

R (cm) = D/2

Where:

- π = 3.14

- R = Sample Radius (cm)

- L = Length of samples (cm)

- D = Diameter of samples (cm)

The porosity of a sample is calculated by using the following equations: -

Pore volume (cc)         = Bulk volume - Grain volume                                               (1)

Porosity (%)               =                                                                 (2)

Grain density (gm/cc) =                                                                     (3)

GAS PERMEABILITY MEASUREMENTS: -

- Gas permeability was measured using a calibrated steady state permeameter with Nitrogen as the flowing medium. The flow was allowed to stabilise before the readings were taken.

- To check the performance of the permeameter a full set of check samples of known permeability was run at the beginning of every day (one check plug for each orifice of the permeameter). After every set of samples analysed (20 samples maximum), check samples were again tested – one check sample specified to each orifice used in the analysis.

- The cleaned whole core samples were marked up with two axial planes with 90o between each and mounted in individual hydrostatic core holder with lateral stainless-steel mesh screens on both sides of the first plane. The overburden pressure was applied up to 450 psig.

- The horizontal gas permeability was determined by connecting the system to the permeability upstream and downstream to measure the flow rate. Using Darcy’s law, the permeability being determined through the first axis.

- The above steps were repeated with the other axial plane and the maximum permeability was reported in addition to the permeability value of the axis perpendicular to the maximum (i.e 90 degrees to MAX).

- The vertical gas permeability was measured by removing the lateral screens and connecting the two end faces of the sample to the permeameter.

- Nitrogen gas was flowed through each sample and the differential pressure (across the sample) was measured using a transducer. The permeability value was calculated by application of Darcy’s law.

- The vertical gas permeability is calculated by the following formula:

KGV =                                                    (4)

Where:

| KGV | = | Vertical gas permeability (mD) |
| --- | --- | --- |
| Pb | = | Barometric pressure (atm) |
| P1 | = | Upstream pressure (atm) |
| P2 | = | Downstream pressure (atm) |
| Qg | = | Gas flow rate (cc/sec.) |
| µg | = | Gas viscosity (cp) |
| L | = | Sample length (cm) |
| A | = | Cross-sectional area (sq.cm) |

- The horizontal gas permeability is calculated by the following formula:

KGH =                                                        (5)

Where:

| KGH | = | Horizontal gas permeability (mD) |
| --- | --- | --- |
| µg | = | Gas viscosity (cp) |
| Qg | = | Gas flow rate (cc/sec.) |
| S.F. | = | Shape Factor (Ref. API RP40, Secton6, Page 28) is 1 where each screen covers a 90o arc. |
| L | = | Sample length (cm) |
| P1 | = | Upstream pressure (atm) |
| P2 | = | Downstream pressure (atm) |

Whole Core Samples Analysis Results

Porosity and Permeability Results at Ambient Conditions for Well T1-31

| Sample No. | Depth (ft.in) | Shifted Depth (ft.in) | Air Permeability, mD No Klinkenberg correction |  |  | Air Permeability, mD With Klinkenberg correction |  |  | Porosity (%) | Density, (gm/cc) |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Max @ 400 | 90degs.@ 400psi | Vertical @ 400 | Max @ 400 | 90degs.@ 400psi | Vertical @ 400 |  | Bulk | Grain |  |
| 1 | 7686.10 - 7687.03 | 7694.02 -7694.07 | 1.20 | 0.480 | 0.280 | 0.825 | 0.312 | 0.176 | 13.82 | 2.47 | 2.71 |  |
| 2 | 7695.05 - 7695.10 | 7702.09 -7703.02 | 7.40 | 4.00 | 0.330 | 5.67 | 2.96 | 0.210 | 3.20 | 2.62 | 2.67 | Fractured |
| 3 | 7707.02 - 7707.07 | 7714.06 -7714.11 | 3.00 | 0.020 | 0.020 | 2.18 | 0.011 | 0.011 | 5.46 | 2.60 | 2.69 |  |
| 4 | 7712.08 - 7713.02 | 7720.00 -7720.06 | 0.110 | 0.420 | 0.010 | 0.066 | 0.27 | 0.005 | 4.58 | 2.60 | 2.68 |  |

| Client | : Waha Oil Company | Concession | : 31 | Report No | : 02/PET1-1&2/2024 |
| --- | --- | --- | --- | --- | --- |
| Well | : T1-31 | Formation | : Gir | Field | : |

Section Two

Plug Samples Analysis Results

Methods and Experiments

Tests According to API RP 40, Recommended Practice for Core Analysis

ANALYTICAL PROCEDURES AND EQUIPMENT SPECIFICATION

Sample Preparation: -

One full diameter core from well # T1-31 with total length of 30 feet was received in our laboratory. As per scope of work, a total and spectral core gamma was conducted producing logs on scale 1:200. Full diameter core samples and one and half inch diameter core plugs were drilled in horizontal and vertical directions as requested by Waha using diamond core bit with water as the bit coolant and lubricant. In addition to that, one-inch diameter sidewall plugs were received in our laboratory.

The samples were extracted of hydrocarbons using toluene, leached of salt using methanol, and oven dried at 80 oC. Silver Nitrate solution was used to ensure the removal of salts existing in the pore spaces.

POROSITY, PERMEABILITY AND GRAIN DENSITY MEASUREMENTS: -

The clean, dry samples were subjected to various analyses to determine porosity, permeability and grain density values where possible. The experimental procedures and equipment specifications are discussed below: -

1. Helium Gas Expansion Porosimeter: -

Core analysis laboratory of Petroleum Research Center use a twin cell helium expansion gas porosimeter for the plug samples grain volume measurement. The porosimeter operates using the principle of Boyle's Law.

A sealed reference chamber in the instrument is filled with helium gas at ambient temperature to a pressure of 100 psig. A sample is placed in another sealed chamber, connected to the reference chamber by a two-way valve. This valve when opened allows the gas in the reference chamber to expand into the combined volume of the two chambers. From Boyle's Law, the volume of the sample chamber can be calculated when the volume of the reference chamber, the initial pressure and the final pressure are known.

The instrument must be calibrated before using the porosimeter for analysis. This is done by running a series of stainless-steel blanks of known volumes, to build up a graph of blank volume versus the inverse of the final pressure.

The resulted calibration graph has to be entered into a computer program which performs a linear regression, producing an equation relating the grain volume of any sample run to the final pressure

reading. The porosity and the grain density were then calculated by determining the bulk volume, and the weight of the sample.

As a quality check, a suite of standard samples of known porosities and grain densities were measured every 20 samples.

The instrument used by our laboratory has the following specifications: -

Pressure indicator with remote pressure transducer

| 1- Range 2- Resolution | : | 0-100 psi 0.01 psi |
| --- | --- | --- |
| 3- Accuracy | : | + 0.1% of the full scale |

Precision Pressure Controller

| 1- Range | : | 0-100 psi |
| --- | --- | --- |
| 2- Resolution | : | Better than 0.01 psi |
| 3- Repeatability | : | Better than 0.02 psi |

2. Positive Displacement Pump: -

The positive displacement mercury pump is used to determine the bulk volume of each sample. The unit consists of a stainless steel high pressure pycnometer (sample container); it is attached to the volumetric pump cylinder. As the hand wheel connected to the pump is wound in or out, mercury is charged or withdrawn from the sample chamber. The volume of mercury displaced is measured linearly by a digital transducer connected to the pump. In practice, mercury is allowed into the pycnometer to a set reference mark, and the transducer set to zero. This process should be repeated several times to ensure accuracy. The mercury is subsequently released back into the pump and the sample is placed and sealed in the pycnometer. After that mercury is allowed back into the pycnometer up to the previously determined reference mark, and the display shown on the digital transducer noted. This value is later converted to the actual volume by using a simple correction factor derived from calibration volumes. The mercury is cleaned, and the pump reset to zero at least once every four samples, depending on the sample type.

3. Calculation of Porosity: -

The porosity of a plug sample is calculated by using the following equations: -

Pore volume (cc)         = Bulk volume - Grain volume

Porosity (%)                =

Grain density (gm/cc) =

4. Nitrogen Gas Permeability: -

The clean, dry sample is placed in the Hassler-type core holder, and an overburden pressure of 200 psig is applied to the cell, compressing the rubber sleeve around the sample. Nitrogen gas pressure is applied to one end face of the sample, whilst the other end face is open to atmospheric pressure, causing the gas to flow through the sample. The flow of gas is measured at the low pressure (downstream) end face of the sample. The flow rate is measured by passing the gas through one of three laminar flow orifices and measuring the differential pressure developed across the orifice.

The transducers and flow orifices are calibrated using a dead weight tester and soap film meter at least once every three months, or when any component is altered or replaced.

The equipment specifications are as follow: -

High pressure precision regulator

| 1- Range | : | 0-30 psi |
| --- | --- | --- |
| 2- Resolution | : | Better than 0.01 psi |

High pressure transducer and readout (upstream)

| 1- Range | : | 0-100 psi |
| --- | --- | --- |
| 2- Resolution | : | 0.01 psi |
| 3- Accuracy | : | + 0.1 % of the full range |

Low pressure transducer and readout (upstream)

| 1- Range | : | 0-15 psi |
| --- | --- | --- |
| 2- Resolution | : | 0.001 psi |
| 3- Accuracy | : | + 0.1 % of full scale |

The length of the sample, upstream and downstream pressures, flow rate, viscosity of nitrogen, barometric pressure and temperature are entered into Darcy's equation for gas permeability, and the permeability of the sample calculated.

Kg =                                                                                           (6)

Where:

| Kg | = | Gas permeability (mD) |
| --- | --- | --- |
| Pb | = | Barometric pressure (atm) |
| P1 | = | Upstream pressure (atm) |
| P2 | = | Downstream pressure (atm) |
| Qg | = | Gas flow rate (cc/sec.) |
| µg | = | Gas viscosity (cp) |
| L | = | Sample length (cm) |
| A | = | Cross-sectional area (sq.cm) |

5. Klinkenberg Permeability: -

The Klinkenberg permeability correction is calculated using the mathematical relationship between Kg and Kl as follows:

Kl = 0.68 (Kg) 1.06 ……………    (Empirical Correction)                           (7)

This empirical correction is valid for permeability ranges from 0.01 up to 174 mD and from 175 to 7500 mD using the following table:

| (Kg) Permeability Range, mD | (Kl) Klinkenberg Permeability, mD |
| --- | --- |
| 175-299 | Kg – 20 |
| 300-699 | Kg – 30 |
| 700-999 | Kg – 40 |
| 1000-1699 | Kg – 50 |
| 1700-2299 | Kg – 60 |
| 2300-5799 | Kg – 80 |
| 5800-7500 | Kg – 100 |

Internal Quality Control: -

As previously stated, all pressure gauges and transducers are regularly checked and calibrated using dead weight tester and wet soap flow meters. In order to maintain a rigid quality assurance programme, a series of four standard check plugs whose dimensions, porosity and grain density are known, are run every data set (maximum of 20 samples) to check repeatability.

The following criteria are used to check repeatability of all data by using the same equipment with different operator.

| Parameter | Acceptable Error |
| --- | --- |
| Grain volume | + 0.2 % |
| Grain density | + 0.02 gm/cc |
| Porosity | + 0.2 % |
| Permeability | + 4 % |

6. PROBE PERMEABILITY: -

The profile (Mini or probe) permeameter is used for localised measurement of permeability with a resolution of about 4 mm which assist in the accurate characterisation and quantification of permeability distribution in heterogeneous formations and also in thinly bedded formations, where the bed thickness is less than the diameter of the core plug. Here a probe is used to make many measurements of permeability along the surface of the (slabbed) core.

The Pressure-Decay Profile Permeameter (PDPK-400) can measure gas permeabilities reliably from 0.001 millidarcy to greater than 30 Darcy.  Permeabilities are slip corrected (Klinkenberg) and are also corrected for non-Darcian flow (Forchheimer Factor).

The Profile Permeameter Software (PROPERM) is a Windows™-based operating program. The software is designed for ease of use and features graphically oriented operator menus and step-by-step Action Command Boxes.

When a core sample is loaded onto the core rack, the probe assembly and core rack are then moved to a desired measurement location, the probe is lowered pneumatically, and its rubber tip seal is pressed against the sample with a controlled pressure. The exact position on the sample where the permeability measurement will be made is indicated by a laser beam that passes through the axis of the probe and projects on the sample as a tiny red dot.

The manifold and probe assembly comprise four accurately calibrated volumes that are initially charged with nitrogen. The probe tip is sealed against the sample with a pneumatic ram, the computer-controlled probe valve is opened and the pressure in the tank and probe assembly recorded as a function of time.

The slope of a preliminary semi-logarithmic pressure decay is determined between two predetermined pressures, or for a maximum elapsed time of 5 seconds.  A "maximum volume",

which will complete the permeability measurement within a maximum run time, is calculated from the preliminary pressure-decay slope along with the value of the current pressure and a final, minimum measurement pressure (about 0.15 psig).  The largest actual reservoir volume that is smaller than the calculated maximum volume is then selected by closing the appropriate valves.

As the flow path of the gas is complex (roughly hemispherical) and the core may not be cleaned, the profile permeability results are calibrated against permeabilities measured in homogeneous cleaned plugs.

Upon your request the permeability measurements have been taken on slabbed core measured six points per foot as possible, a total of 160 probe samples were measured on slabbed cores for well T1-31. The probe results are tabulated against depth in the pages 38-42.

Conventional Core Analysis

Results and Profiles

| Sample No. | Depth (ft.in) | Shifted Depth (ft.in) | Air Permeability, mD No Klinkenberg correction |  | Air Permeability, mD With Klinkenberg correction @200 psi |  | Porosity (%) | Density (gm/cc) |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | @200 psi |  |  |  |  |  |  |  |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |  | Bulk | Grain |  |
| Core # 2 (7685' – 7715') ft, Recovery 100%, Formation: Gir, lithology: Limestone |  |  |  |  |  |  |  |  |  |  |
| 1 | 7685.02 | 7692.06 | 0.640 | ---- | 0.424 | ---- | 16.86 | 2.42 | 2.71 |  |
| 2 | 7686.05 | 7693.09 | 0.561 | ---- | 0.368 | ---- | 16.09 | 2.44 | 2.72 |  |
| 3 | 7687.07 | 7694.11 | 0.223 | 0.115 | 0.139 | 0.069 | 13.95 | 2.48 | 2.72 |  |
| 4 | 7688.08 | 7696.00 | 0.004 | ---- | 0.002 | ---- | 2.18 | 2.63 | 2.67 |  |
| 5 | 7689.05 | 7696.09 | 0.003 | ---- | 0.001 | ---- | 3.03 | 2.63 | 2.69 |  |
| 6 | 7690.03 | 7697.07 | 0.005 | 0.004 | 0.002 | 0.002 | 5.36 | 2.61 | 2.70 |  |
| 7 | 7693.07 | 7700.11 | 1.172 | 0.006 | 0.805 | 0.003 | 7.39 | 2.57 | 2.69 | Fractured. + Broken |
| 8 | 7694.08 | 7702.00 | 0.005 | ---- | 0.002 | ---- | 6.47 | 2.59 | 2.70 | Short |
| 9 | 7695.11 | 7703.03 | 0.005 | ---- | 0.002 | ---- | 7.58 | 2.57 | 2.70 |  |
| 10 | 7696.09 | 7704.01 | 0.980 | 0.006 | 0.666 | 0.003 | 9.67 | 2.55 | 2.71 | Cemented Fracture. |
| 11 | 7697.04 | 7704.08 | 0.004 | ---- | 0.002 | ---- | 3.85 | 2.62 | 2.68 |  |
| 12 | 7698.07 | 7705.11 | 0.021 | ---- | 0.011 | ---- | 11.53 | 2.52 | 2.72 |  |
| 13 | 7699.09 | 7707.01 | 0.017 | 0.017 | 0.009 | 0.009 | 11.05 | 2.53 | 2.72 |  |
| 14 | 7700.03 | 7707.07 | 0.017 | ---- | 0.009 | ---- | 10.95 | 2.53 | 2.72 |  |
| 15 | 7701.08 | 7709.00 | 0.014 | ---- | 0.007 | ---- | 10.70 | 2.53 | 2.72 |  |
| 16 | 7702.04 | 7709.08 | 0.019 | 0.018 | 0.010 | 0.010 | 10.11 | 2.54 | 2.71 |  |
| 17 | 7703.03 | 7710.07 | 0.007 | ---- | 0.004 | ---- | 5.32 | 2.62 | 2.71 |  |
| 18 | 7704.05 | 7711.09 | 0.004 | ---- | 0.002 | ---- | 6.96 | 2.59 | 2.71 |  |

Conventional Core Analysis Results

Client:   Waha Oil Company                                Concession: 31                                                       Report No.:     02/PET1-1&2/24

Well  :      T1-31                                                      Formation: Gir

Conventional Core Analysis Results

| Client |  |  | : Waha Oil Company |  |  |  | Concession |  | : 31 |  |  |  | Report No. |  | : 02/PET1-1&2/24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well |  |  | : T1-31 |  |  |  | Formation |  | : Gir |  |  |  |  |  |  |  |
| Sample No. |  | Depth (ft.in) |  |  | Shifted Depth (ft.in) | Air Permeability, mD No Klinkenberg correction |  | Air Permeability, mD With Klinkenberg correction @200 psi |  |  | Porosity (%) | Density (gm/cc) |  |  | Remarks |  |  |
|  |  |  |  |  |  | @200 psi |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Horizontal | Vertical | Horizontal | Vertical |  |  | Bulk | Grain |  |  |  |  |
| 19 |  | 7705.08 |  |  | 7713.00 | 0.005 | 0.005 | 0.002 | 0.002 |  | 6.57 | 2.60 | 2.71 |  |  |  |  |
| 20 |  | 7706.11 |  |  | 7714.03 | 5.511 | ---- | 4.152 | ---- |  | 7.81 | 2.56 | 2.69 |  | Fractured. |  |  |
| 21 |  | 7707.08 |  |  | 7715.00 | 0.008 | ---- | 0.004 | ---- |  | 8.61 | 2.56 | 2.71 |  |  |  |  |
| 22 |  | 7708.06 |  |  | 7715.10 | 0.009 | 0.009 | 0.005 | 0.005 |  | 9.55 | 2.55 | 2.71 |  |  |  |  |
| 23 |  | 7709.03 |  |  | 7716.07 | 0.009 | ---- | 0.005 | ---- |  | 10.32 | 2.53 | 2.71 |  |  |  |  |
| 24 |  | 7710.06 |  |  | 7717.10 | 2.352 | ---- | 1.684 | ---- |  | 9.18 | 2.54 | 2.70 |  |  |  |  |
| 25 |  | 7711.07 |  |  | 7718.11 | 0.014 | 0.014 | 0.007 | 0.007 |  | 11.41 | 2.52 | 2.71 |  |  |  |  |
| 26 |  | 7712.04 |  |  | 7719.08 | 0.013 | ---- | 0.007 | ---- |  | 10.42 | 2.53 | 2.71 |  |  |  |  |
| 27 |  | 7713.06 |  |  | 7720.10 | 0.005 | ---- | 0.002 | ---- |  | 7.68 | 2.60 | 2.73 |  |  |  |  |
| 28 |  | 7714.11 |  |  | 7722.03 | 0.008 | 0.004 | 0.004 | 0.002 |  | 6.90 | 2.61 | 2.73 |  |  |  |  |

Conventional Core Analysis Results

| Client |  |  | : Waha Oil Company |  |  |  | Concession |  | : 31 |  |  | Report No. |  |  |  | : 02/PET1-1&2/24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well |  |  | : T1-31 |  |  |  | Formation |  | : Gir |  |  |  |  |  |  |  |  |
| Sample No. |  | Depth (ft.in) |  |  | Shifted Depth (ft.in) | Air Permeability, mD No Klinkenberg correction |  |  | Air Permeability, mD With Klinkenberg correction @200 psi |  |  | Porosity (%) |  |  | Density (gm/cc) |  |  | Remarks |
|  |  |  |  |  |  | @200 psi |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Horizontal | Vertical |  | Horizontal |  | Vertical |  |  |  | Bulk | Grain |  |  |
|  |  |  |  |  |  | Sidewall Plugs |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7500.00 |  |  | 7507.04 | 0.880 | ---- |  | 0.594 |  | ---- | 16.22 |  |  | 2.42 | 2.70 |  |  |
| 4 |  | 7660.00 |  |  | 7667.04 | 1.177 | ---- |  | 0.808 |  | ---- | 21.44 |  |  | 2.34 | 2.70 |  | Short |
| 6 |  | 7750.00 |  |  | 7757.04 | 0.030 | ---- |  | 0.017 |  | ---- | 12.40 |  |  | 2.52 | 2.74 |  |  |
| 7 |  | 7774.00 |  |  | 7781.04 | 0.319 | ---- |  | 0.203 |  | ---- | 13.53 |  |  | 2.48 | 2.71 |  |  |
| 8 |  | 7810.00 |  |  | 7817.04 | 0.135 | ---- |  | 0.081 |  | ---- | 11.89 |  |  | 2.52 | 2.72 |  | Short |
| 9 |  | 7837.00 |  |  | 7844.04 | 0.880 | ---- |  | 0.594 |  | ---- | 11.84 |  |  | 2.52 | 2.72 |  |  |
| 10 |  | 7876.00 |  |  | 7883.04 | 0.213 | ---- |  | 0.132 |  | ---- | 12.65 |  |  | 2.48 | 2.70 |  |  |
| 11 |  | 7902.00 |  |  | 7909.04 | 0.251 | ---- |  | 0.157 |  | ---- | 12.46 |  |  | 2.50 | 2.71 |  |  |
| 12 |  | 7915.00 |  |  | 7922.04 | 0.079 | ---- |  | 0.046 |  | ---- | 2.91 |  |  | 2.64 | 2.69 |  |  |
| 13 |  | 7945.00 |  |  | 7952.04 | 0.073 | ---- |  | 0.042 |  | ---- | 3.51 |  |  | 2.64 | 2.70 |  |  |
| 14 |  | 8000.00 |  |  | 8007.04 | 0.393 | ---- |  | 0.253 |  | ---- | 15.61 |  |  | 2.44 | 2.71 |  |  |
| 15 |  | 8030.00 |  |  | 8037.04 | 0.105 | ---- |  | 0.062 |  | ---- | 9.06 |  |  | 2.56 | 2.71 |  |  |
| 16 |  | 8150.00 |  |  | 8157.04 | 0.997 | ---- |  | 0.678 |  | ---- | 19.76 |  |  | 2.37 | 2.71 |  |  |
| 18 |  | 8225.00 |  |  | 8232.04 | 2.907 | ---- |  | 2.107 |  | ---- | 20.73 |  |  | 2.35 | 2.70 |  | Short |
| 19 |  | 8250.00 |  |  | 8257.04 | 1.366 | ---- |  | 0.946 |  | ---- | 19.21 |  |  | 2.38 | 2.71 |  | Broken edge |
| 20 |  | 8300.00 |  |  | 8307.04 | 0.781 | ---- |  | 0.523 |  | ---- | 16.62 |  |  | 2.43 | 2.71 |  | Broken edge |
| 21 |  | 8345.00 |  |  | 8352.04 | 0.121 | ---- |  | 0.072 |  | ---- | 5.83 |  |  | 2.61 | 2.71 |  |  |
| 22 |  | 8375.00 |  |  | 8382.04 | 0.092 | ---- |  | 0.054 |  | ---- | 3.20 |  |  | 2.64 | 2.69 |  |  |

Conventional Core Analysis Results

| Client |  |  | : Waha Oil Company |  |  |  | Concession |  | : 31 |  |  | Report No. |  |  | : 02/PET1-1&2/24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well |  |  | : T1-31 |  |  |  | Formation |  | : Gir |  |  |  |  |  |  |  |
| Sample No. |  | Depth (ft.in) |  |  | Shifted Depth (ft.in) | Air Permeability, mD No Klinkenberg correction @200 psi |  |  | Air Permeability, mD With Klinkenberg correction @200 psi |  |  | Porosity (%) |  | Density (gm/cc) |  |  | Remarks |
|  |  |  |  |  |  | Horizontal | Vertical |  | Horizontal |  | Vertical |  |  | Bulk | Grain |  |  |
| 23 |  | 8473.00 |  |  | 8480.04 | 0.282 | ---- |  | 0.178 |  | ---- | 15.92 |  | 2.43 | 2.70 |  | Short |
| 24 |  | 8566.00 |  |  | 8573.04 | 0.471 | ---- |  | 0.306 |  | ---- | 11.63 |  | 2.50 | 2.70 |  | Short |
| 25 |  | 8625.00 |  |  | 8632.04 | 0.095 | ---- |  | 0.056 |  | ---- | 1.02 |  | 2.58 | 2.60 |  |  |
| 26 |  | 8672.00 |  |  | 8679.04 | 0.419 | ---- |  | 0.270 |  | ---- | 13.98 |  | 2.46 | 2.70 |  |  |
| 29 |  | 8840.00 |  |  | 8847.04 | 0.726 | ---- |  | 0.484 |  | ---- | 17.11 |  | 2.42 | 2.71 |  |  |
| 30 |  | 8902.00 |  |  | 8909.04 | 0.453 | ---- |  | 0.294 |  | ---- | 14.50 |  | 2.45 | 2.70 |  |  |

Conventional Core Analysis Results and Profile

Client: Waha Oil Company                                                    Formation: Gir

Well  : T1-31                                                                            Core #       : 2

Conventional Core Analysis Results and Profile

(Sidewall Plugs)

Client: Waha Oil Company                                                     Formation: Gir

Well  : T1-31

Statistical Analysis

# Statistical Analysis of Conventional Core Analysis Data

# Client: Waha Oil Company

# Well #: T1-31

Formation : Gir

Core # 2 (7685' – 7715') ft, Recovery 100%, Formation : Gir, Lithology: Limestone

# The Following graphs represent the statistical analysis of conventional core analysis: -

- Horizontal Permeability Vs Porosity.

- Horizontal Permeability Vs Vertical Permeability.

- Frequency and Cumulative Frequency Distribution of Horizontal Permeability

- Frequency and Cumulative Frequency Distribution of Porosity.

- Frequency and Cumulative Frequency Distribution of Grain Density.

# All Porosities are reported in percentage.

# All Permeabilities are reported in milliDarcy.

# Horizontal Permeability vs. Porosity

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 28 |

# Horizontal Permeability vs. Vertical Permeability

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 10 |

Horizontal Permeability Frequency Distribution

# Client: Waha Oil Company

# WELL No: T1-31

# Formation: Gir

# HORIZONTAL PERMEABILITY ANALYSIS SUMMARY

| Number of samples | 28 |
| --- | --- |
| Minimum | 0.001 |
| Maximum | 4.152 |
| Arithmetic mean | 0.30 |
| Geometric mean | 0.015 |
| Harmonic mean | 0.004 |

GROUPING BY PERMEABILITY RANGE

| Permeability Range (mD) | Samples in range | Permeability Arith. (mD) | Permeability Geom. (mD) | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- | --- |
| 0-0.01 | 19 | 0.004 | 0.004 | 67.86 | 67.86 |
| 0.01-0.02 | 2 | 0.011 | 0.011 | 7.14 | 75.00 |
| 0.02-0.04 | 0 | 0.000 | 0.000 | 0.000 | 75.00 |
| 0.04-0.08 | 0 | 0.000 | 0.000 | 0.000 | 75.00 |
| 0.08-0.16 | 1 | 0.139 | 0.139 | 3.571 | 78.57 |
| 0.16-0.32 | 0 | 0.000 | 0.000 | 0.000 | 78.57 |
| 0.32-0.64 | 2 | 0.396 | 0.395 | 7.14 | 85.71 |
| 0.64-1.25 | 2 | 0.735 | 0.732 | 7.14 | 92.86 |
| 1.25-2.5 | 1 | 1.684 | 1.684 | 3.571 | 96.43 |
| 2.5-5 | 1 | 4.152 | 4.152 | 3.571 | 100.00 |

# Horizontal Permeability Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 28 |

# Porosity Frequency Distribution

# Client: Waha Oil Company

# WELL No: T1-31

# Formation: Gir

# POROSITY ANALYSIS SUMMARY

| Number of samples | 28 |
| --- | --- |
| Minimum | 2.18 |
| Maximum | 16.86 |
| Arithmetic mean | 8.84 |

GROUPING BY POROSITY RANGE

| Porosity Range (%) | Samples in range | Mean Porosity (%) | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- |
| 0-2 | 0 | 0.000 | 0.000 | 0.000 |
| 2-4 | 3 | 3.02 | 10.71 | 10.71 |
| 4-6 | 2 | 5.34 | 7.14 | 17.86 |
| 6-8 | 8 | 7.17 | 28.57 | 46.43 |
| 8-10 | 4 | 9.25 | 14.29 | 60.71 |
| 10-12 | 8 | 10.81 | 28.57 | 89.29 |
| 12-14 | 1 | 13.95 | 3.57 | 92.86 |
| 14-16 | 0 | 0.000 | 0.000 | 92.86 |
| 16-18 | 2 | 16.48 | 7.14 | 100.00 |

# Porosity Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 28 |

Grain Density Frequency Distribution

# Client: Waha Oil Company

# WELL No.: T1-31

# Formation: Gir

GRAIN DENSITY ANALYSIS SUMMARY

| Number of samples | 28 |
| --- | --- |
| Minimum | 2.67 |
| Maximum | 2.73 |
| Arithmetic mean | 2.71 |

GROUPING BY DENSITY RANGE

| Grain Density Range | Samples in range | Mean Grain Density G/C.C | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- |
| 2.60-2.65 | 0 | 0.00 | 0.000 | 0.000 |
| 2.65-2.70 | 8 | 2.69 | 28.57 | 28.57 |
| 2.70-2.75 | 20 | 2.71 | 71.43 | 100.00 |

# Grain Density Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 28 |

# Statistical Analysis of Conventional Core Analysis Data

# Client: Waha Oil Company

# Well #: T1-31

Formation : Gir

Sidewall Plugs

# The Following graphs represent the statistical analysis of conventional core analysis: -

- Horizontal Permeability Vs Porosity.

- Frequency and Cumulative Frequency Distribution of Horizontal Permeability

- Frequency and Cumulative Frequency Distribution of Porosity.

- Frequency and Cumulative Frequency Distribution of Grain Density.

# All Porosities are reported in percentage.

# All Permeabilities are reported in milliDarcy.

# Horizontal Permeability vs. Porosity

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 24 |

# Horizontal Permeability Frequency Distribution

# Client: Waha Oil Company

# WELL No: T1-31

# Formation: Gir

# HORIZONTAL PERMEABILITY ANALYSIS SUMMARY

| Number of samples | 24 |
| --- | --- |
| Minimum | 0.017 |
| Maximum | 2.107 |
| Arithmetic mean | 0.37 |
| Geometric mean | 0.200 |
| Harmonic mean | 0.102 |

GROUPING BY PERMEABILITY RANGE

| Permeability Range (mD) | Samples in range | Permeability Arith. (mD) | Permeability Geom. (mD) | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- | --- |
| 0-0.01 | 0 | 0.000 | 0.000 | 0.000 | 0.00 |
| 0.01-0.02 | 1 | 0.017 | 0.017 | 4.167 | 4.17 |
| 0.02-0.04 | 0 | 0.000 | 0.000 | 0.000 | 4.17 |
| 0.04-0.08 | 6 | 0.056 | 0.000 | 25.00 | 29.17 |
| 0.08-0.16 | 3 | 0.124 | 0.119 | 12.50 | 41.67 |
| 0.16-0.32 | 6 | 0.251 | 0.000 | 25.00 | 66.67 |
| 0.32-0.64 | 4 | 0.549 | 0.547 | 16.67 | 83.33 |
| 0.64-1.25 | 3 | 0.811 | 0.803 | 12.50 | 95.83 |
| 1.25-2.5 | 1 | 2.107 | 2.107 | 4.167 | 100.00 |

# Horizontal Permeability Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 24 |

# Porosity Frequency Distribution

# Client: Waha Oil Company

# WELL No: T1-31

# Formation: Gir

# POROSITY ANALYSIS SUMMARY

| Number of samples | 24 |
| --- | --- |
| Minimum | 1.02 |
| Maximum | 21.44 |
| Arithmetic mean | 12.63 |

GROUPING BY POROSITY RANGE

| Porosity Range (%) | Samples in range | Mean Porosity (%) | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- |
| 0-2 | 1 | 1.02 | 4.17 | 4.17 |
| 2-4 | 3 | 3.21 | 12.50 | 16.67 |
| 4-6 | 1 | 5.83 | 4.17 | 20.83 |
| 6-8 | 0 | 0.000 | 0.00 | 20.83 |
| 8-10 | 1 | 9.06 | 4.17 | 25.00 |
| 10-12 | 3 | 11.79 | 12.50 | 37.50 |
| 12-14 | 5 | 13.00 | 20.83 | 58.33 |
| 14-16 | 3 | 15.34 | 12.50 | 70.83 |
| 16-18 | 3 | 16.65 | 12.50 | 83.33 |
| 18-20 | 2 | 19.49 | 8.33 | 91.67 |
| 20-22 | 2 | 21.09 | 8.33 | 100.00 |

# Porosity Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 24 |

Grain Density Frequency Distribution

# Client: Waha Oil Company

# WELL No.: T1-31

# Formation: Gir

GRAIN DENSITY ANALYSIS SUMMARY

| Number of samples | 24 |
| --- | --- |
| Minimum | 2.60 |
| Maximum | 2.74 |
| Arithmetic mean | 2.70 |

GROUPING BY DENSITY RANGE

| Grain Density Range | Samples in range | Mean Grain Density G/C.C | Frequency (%) | Cumulative Frequency (%) |
| --- | --- | --- | --- | --- |
| 2.55-2.60 | 0 | 0.00 | 0.000 | 0.000 |
| 2.60-2.65 | 1 | 2.60 | 4.167 | 4.167 |
| 2.65-2.70 | 2 | 2.69 | 8.33 | 12.50 |
| 2.70-2.75 | 21 | 2.71 | 87.50 | 100.00 |

# Grain Density Frequency Distribution

| Client | : Waha Oil Company | Formation: Gir |
| --- | --- | --- |
| Well | : T1-31 | Number of Samples: 24 |

Probe Permeability

Results and Profiles

PROBE PERMEABILITY RESULTS

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #       : 2

| Core# 2 (7685’ 7715’), Formation: Gir, Lithology: Limestone |  |  |  |
| --- | --- | --- | --- |
| Sample No. | Depth (ft) | Shifted Depth (ft) | KL (Probe) (mD) |
| 1 | 7685.33 | 7692.67 | 0.085 |
| 2 | 7685.42 | 7692.76 | 0.080 |
| 3 | 7685.58 | 7692.92 | 0.186 |
| 4 | 7685.67 | 7693.01 | 0.118 |
| 5 | 7685.75 | 7693.09 | 0.073 |
| 6 | 7685.92 | 7693.26 | 0.056 |
| 7 | 7686.17 | 7693.51 | 0.130 |
| 8 | 7686.25 | 7693.59 | 0.120 |
| 9 | 7686.33 | 7693.67 | 0.149 |
| 10 | 7686.42 | 7693.76 | 0.147 |
| 11 | 7686.58 | 7693.92 | 0.145 |
| 12 | 7686.75 | 7694.09 | 0.138 |
| 13 | 7687.67 | 7695.01 | 0.129 |
| 14 | 7687.75 | 7695.09 | 0.118 |
| 15 | 7688.08 | 7695.42 | 0.015 |
| 16 | 7688.33 | 7695.67 | 0.012 |
| 17 | 7688.50 | 7695.84 | 0.025 |
| 18 | 7688.75 | 7696.09 | 0.010 |
| 19 | 7688.83 | 7696.17 | 0.006 |
| 20 | 7688.92 | 7696.26 | 0.006 |
| 21 | 7689.00 | 7696.34 | 0.007 |
| 22 | 7689.17 | 7696.51 | 0.002 |
| 23 | 7689.42 | 7696.76 | 0.008 |
| 24 | 7689.67 | 7697.01 | 0.011 |
| 25 | 7689.83 | 7697.17 | 0.018 |
| 26 | 7689.92 | 7697.26 | 0.013 |
| 27 | 7690.00 | 7697.34 | 0.019 |
| 28 | 7690.25 | 7697.59 | 0.019 |
| 29 | 7690.42 | 7697.76 | 0.032 |
| 30 | 7690.50 | 7697.84 | 0.044 |
| 31 | 7690.58 | 7697.92 | 0.015 |
| 32 | 7690.67 | 7698.01 | 0.012 |

PROBE PERMEABILITY RESULTS

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #       : 2

| Sample No. | Depth (ft)) | Shifted Depth (ft) | KL (Probe) (mD) |
| --- | --- | --- | --- |
| 33 | 7691.83 | 7699.17 | 0.029 |
| 34 | 7692.25 | 7699.59 | 0.015 |
| 35 | 7692.58 | 7699.92 | 0.036 |
| 36 | 7693.67 | 7701.01 | 0.015 |
| 37 | 7693.75 | 7701.09 | 0.018 |
| 38 | 7693.83 | 7701.17 | 0.017 |
| 39 | 7693.92 | 7701.26 | 0.017 |
| 40 | 7694.08 | 7701.42 | 0.020 |
| 41 | 7694.25 | 7701.59 | 0.017 |
| 42 | 7694.33 | 7701.67 | 0.011 |
| 43 | 7694.50 | 7701.84 | 0.010 |
| 44 | 7694.58 | 7701.92 | 0.015 |
| 45 | 7694.75 | 7702.09 | 0.021 |
| 46 | 7695.83 | 7703.17 | 0.023 |
| 47 | 7695.92 | 7703.26 | 0.009 |
| 48 | 7696.08 | 7703.42 | 0.006 |
| 49 | 7696.25 | 7703.59 | 0.004 |
| 50 | 7696.42 | 7703.76 | 0.007 |
| 51 | 7696.58 | 7703.92 | 0.012 |
| 52 | 7696.75 | 7704.09 | 0.013 |
| 53 | 7696.92 | 7704.26 | 0.016 |
| 54 | 7697.08 | 7704.42 | 0.024 |
| 55 | 7697.17 | 7704.51 | 0.031 |
| 56 | 7697.42 | 7704.76 | 0.048 |
| 57 | 7697.58 | 7704.92 | 0.029 |
| 58 | 7697.75 | 7705.09 | 0.037 |
| 59 | 7697.92 | 7705.26 | 0.042 |
| 60 | 7698.08 | 7705.42 | 0.046 |
| 61 | 7698.25 | 7705.59 | 0.046 |
| 62 | 7698.42 | 7705.76 | 0.045 |
| 63 | 7698.58 | 7705.92 | 0.034 |
| 64 | 7698.75 | 7706.09 | 0.032 |
| 65 | 7698.92 | 7706.26 | 0.045 |

PROBE PERMEABILITY RESULTS

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #        : 2

| Sample No. | Depth (ft) | Shifted Depth (ft) | KL (Probe) (mD) |
| --- | --- | --- | --- |
| 66 | 7699.08 | 7706.42 | 0.048 |
| 67 | 7699.25 | 7706.59 | 0.047 |
| 68 | 7699.42 | 7706.76 | 0.043 |
| 69 | 7699.58 | 7706.92 | 0.041 |
| 70 | 7699.75 | 7707.09 | 0.039 |
| 71 | 7699.92 | 7707.26 | 0.045 |
| 72 | 7700.08 | 7707.42 | 0.042 |
| 73 | 7700.17 | 7707.51 | 0.054 |
| 74 | 7700.42 | 7707.76 | 0.034 |
| 75 | 7700.58 | 7707.92 | 0.044 |
| 76 | 7700.75 | 7708.09 | 0.039 |
| 77 | 7700.83 | 7708.17 | 0.040 |
| 78 | 7700.92 | 7708.26 | 0.031 |
| 79 | 7701.08 | 7708.42 | 0.050 |
| 80 | 7701.17 | 7708.51 | 0.066 |
| 81 | 7701.33 | 7708.67 | 0.061 |
| 82 | 7701.50 | 7708.84 | 0.040 |
| 83 | 7701.75 | 7709.09 | 0.018 |
| 84 | 7701.92 | 7709.26 | 0.015 |
| 85 | 7702.08 | 7709.42 | 0.035 |
| 86 | 7702.25 | 7709.59 | 0.083 |
| 87 | 7702.33 | 7709.67 | 0.080 |
| 88 | 7702.67 | 7710.01 | 0.103 |
| 89 | 7702.75 | 7710.09 | 0.045 |
| 90 | 7702.92 | 7710.26 | 0.028 |
| 91 | 7703.08 | 7710.42 | 0.025 |
| 92 | 7703.17 | 7710.51 | 0.020 |
| 93 | 7703.42 | 7710.76 | 0.016 |
| 94 | 7703.50 | 7710.84 | 0.018 |
| 95 | 7703.75 | 7711.09 | 0.009 |
| 96 | 7703.92 | 7711.26 | 0.006 |
| 97 | 7704.08 | 7711.42 | 0.006 |
| 98 | 7704.17 | 7711.51 | 0.006 |
| 99 | 7704.25 | 7711.59 | 0.008 |

PROBE PERMEABILITY RESULTS

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #        : 2

| Sample No. | Depth (ft) | Shifted Depth (ft) | KL (Probe) (mD) |
| --- | --- | --- | --- |
| 100 | 7704.50 | 7711.84 | 0.034 |
| 101 | 7704.75 | 7712.09 | 0.021 |
| 102 | 7704.92 | 7712.26 | 0.021 |
| 103 | 7705.08 | 7712.42 | 0.020 |
| 104 | 7705.25 | 7712.59 | 0.022 |
| 105 | 7705.42 | 7712.76 | 0.033 |
| 106 | 7705.75 | 7713.09 | 0.023 |
| 107 | 7705.83 | 7713.17 | 0.020 |
| 108 | 7705.92 | 7713.26 | 0.030 |
| 109 | 7706.08 | 7713.42 | 0.003 |
| 110 | 7706.75 | 7714.09 | 0.003 |
| 111 | 7706.83 | 7714.17 | 0.003 |
| 112 | 7706.92 | 7714.26 | 0.004 |
| 113 | 7707.00 | 7714.34 | 0.006 |
| 114 | 7707.08 | 7714.42 | 0.010 |
| 115 | 7707.17 | 7714.51 | 0.012 |
| 116 | 7707.25 | 7714.59 | 0.012 |
| 117 | 7707.42 | 7714.76 | 0.021 |
| 118 | 7707.67 | 7715.01 | 0.028 |
| 119 | 7707.83 | 7715.17 | 0.032 |
| 120 | 7708.08 | 7715.42 | 0.018 |
| 121 | 7708.25 | 7715.59 | 0.020 |
| 122 | 7708.33 | 7715.67 | 0.030 |
| 123 | 7708.58 | 7715.92 | 0.036 |
| 124 | 7708.75 | 7716.09 | 0.032 |
| 125 | 7708.92 | 7716.26 | 0.039 |
| 126 | 7709.08 | 7716.42 | 0.031 |
| 127 | 7709.33 | 7716.67 | 0.027 |
| 128 | 7709.50 | 7716.84 | 0.036 |
| 129 | 7709.67 | 7717.01 | 0.034 |
| 130 | 7709.83 | 7717.17 | 0.039 |
| 131 | 7709.92 | 7717.26 | 0.035 |
| 132 | 7710.08 | 7717.42 | 0.026 |
| 133 | 7710.25 | 7717.59 | 0.032 |

PROBE PERMEABILITY RESULTS

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #        : 2

| Sample No. | Depth (ft) | Shifted Depth (ft) | KL (Probe) (mD) |
| --- | --- | --- | --- |
| 134 | 7710.33 | 7717.67 | 0.035 |
| 135 | 7710.58 | 7717.92 | 0.030 |
| 136 | 7710.75 | 7718.09 | 0.021 |
| 137 | 7710.92 | 7718.26 | 0.045 |
| 138 | 7711.08 | 7718.42 | 0.036 |
| 139 | 7711.25 | 7718.59 | 0.043 |
| 140 | 7711.33 | 7718.67 | 0.062 |
| 141 | 7711.42 | 7718.76 | 0.045 |
| 142 | 7711.67 | 7719.01 | 0.018 |
| 143 | 7711.83 | 7719.17 | 0.025 |
| 144 | 7712.08 | 7719.42 | 0.027 |
| 145 | 7712.17 | 7719.51 | 0.024 |
| 146 | 7712.42 | 7719.76 | 0.044 |
| 147 | 7712.50 | 7719.84 | 0.025 |
| 148 | 7712.58 | 7719.92 | 0.021 |
| 149 | 7713.17 | 7720.51 | 0.031 |
| 150 | 7713.25 | 7720.59 | 0.030 |
| 151 | 7713.50 | 7720.84 | 0.021 |
| 152 | 7713.67 | 7721.01 | 0.022 |
| 153 | 7713.75 | 7721.09 | 0.025 |
| 154 | 7713.83 | 7721.17 | 0.018 |
| 155 | 7714.08 | 7721.42 | 0.019 |
| 156 | 7714.25 | 7721.59 | 0.026 |
| 157 | 7714.42 | 7721.76 | 0.024 |
| 158 | 7714.58 | 7721.92 | 0.031 |
| 159 | 7714.75 | 7722.09 | 0.021 |
| 160 | 7714.92 | 7722.26 | 0.022 |

Comparison between Probe & Routine Permeability Results

Client: Waha Oil Company                                     Formation : Gir

Well   : T1-31                                                             Core #        : 2

Part Two

Special Core Analysis

Section One

Porosity and Permeability

As a function of overburden Pressure

Porosity and Permeability as A Function of Overburden Pressure: -

Four of one and half inch diameter plugs, and eight of one-inch diameter sidewall plugs representing Gir formation, were selected, as requested by Waha Oil Company, from the well # T1-31, to investigate the effect of overburden pressure on effective porosity and permeability. The clean dry samples were placed in a CMS-300 instrument which is fully automated permeameter and porosimeter used to determine porosity and permeability. The effective porosity and permeability were measured, as requested by Waha Oil Company, at pressure values of 1000, 2000, 3500, 4839 and 5200 psi for one and half inch diameter plugs, and 1000, 2500, 4000, 5182 and 6000 psi for one-inch diameter sidewall plugs.

The base porosity was measured without overburden pressure, while base permeability was measured at 200 psi. The measurements and the calculations of these Petrophysical properties are based on the techniques described by API, Recommended practice 40 (Reference 1). The results are presented in tables 2.1.1 through 2.1.12 and in graphs 2.1.1 through 2.1.12.

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 1 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7685.02 : 7692.06 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 16.86* |  | 1.000 |  |
| 200 | 16.82 | 0.640** | 0.998 | 1.000 |
| 1000 | 16.68 | 0.634 | 0.989 | 0.991 |
| 2000 | 16.54 | 0.629 | 0.981 | 0.982 |
| 3500 | 16.38 | 0.622 | 0.972 | 0.973 |
| 4839 | 16.28 | 0.619 | 0.965 | 0.966 |
| 5200 | 16.25 | 0.618 | 0.964 | 0.965 |

Table (2.1.1)

*    Base porosity

** Base permeability

Figure (2.1.1)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 2 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7686.05 : 7693.09 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 16.09* |  | 1.000 |  |
| 200 | 16.08 | 0.561** | 0.999 | 1.000 |
| 1000 | 16.02 | 0.555 | 0.996 | 0.989 |
| 2000 | 15.96 | 0.549 | 0.992 | 0.978 |
| 3500 | 15.88 | 0.542 | 0.987 | 0.967 |
| 4839 | 15.81 | 0.538 | 0.983 | 0.959 |
| 5200 | 15.79 | 0.537 | 0.982 | 0.957 |

Table (2.1.2)

*    Base porosity

** Base permeability

Figure (2.1.2)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 3 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7687.07 : 7694.11 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 13.95* |  | 1.000 |  |
| 200 | 13.92 | 0.223** | 0.998 | 1.000 |
| 1000 | 13.80 | 0.220 | 0.989 | 0.988 |
| 2000 | 13.69 | 0.218 | 0.981 | 0.977 |
| 3500 | 13.55 | 0.215 | 0.972 | 0.965 |
| 4839 | 13.46 | 0.214 | 0.965 | 0.958 |
| 5200 | 13.44 | 0.213 | 0.964 | 0.956 |

Table (2.1.3)

*    Base porosity

** Base permeability

Figure (2.1.3)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 24 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7710.06 : 7717.10 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 9.18* |  | 1.000 |  |
| 200 | 9.15 | 2.352** | 0.996 | 1.000 |
| 1000 | 9.03 | 1.591 | 0.983 | 0.676 |
| 2000 | 8.91 | 1.303 | 0.971 | 0.554 |
| 3500 | 8.78 | 1.182 | 0.957 | 0.503 |
| 4839 | 8.70 | 1.148 | 0.948 | 0.488 |
| 5200 | 8.68 | 1.143 | 0.946 | 0.486 |

Table (2.1.4)

*    Base porosity

** Base permeability

Figure (2.1.4)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 4 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7660.00 : 7667.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 21.44* |  | 1.000 |  |
| 200 | 21.29 | 1.18** | 0.993 | 1.000 |
| 1000 | 20.80 | 1.13 | 0.970 | 0.962 |
| 2500 | 20.15 | 1.08 | 0.940 | 0.915 |
| 4000 | 19.74 | 1.05 | 0.921 | 0.888 |
| 5182 | 19.51 | 1.03 | 0.910 | 0.874 |
| 6000 | 19.39 | 1.02 | 0.905 | 0.868 |

Table (2.1.5)

*   Base porosity

** Base permeability

Figure (2.1.5)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 9 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7837.00 : 7844.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 11.84* |  | 1.000 |  |
| 200 | 11.79 | 0.88** | 0.996 | 1.000 |
| 1000 | 11.61 | 0.866 | 0.981 | 0.984 |
| 2500 | 11.37 | 0.847 | 0.961 | 0.963 |
| 4000 | 11.21 | 0.836 | 0.947 | 0.950 |
| 5182 | 11.12 | 0.829 | 0.939 | 0.942 |
| 6000 | 11.06 | 0.826 | 0.934 | 0.938 |

Table (2.1.6)

*   Base porosity

** Base permeability

Figure (2.1.6)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 16 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8150.00 : 8157.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 19.76* |  | 1.000 |  |
| 200 | 19.70 | 0.997** | 0.997 | 1.000 |
| 1000 | 19.48 | 0.976 | 0.986 | 0.979 |
| 2500 | 19.18 | 0.949 | 0.970 | 0.952 |
| 4000 | 18.97 | 0.933 | 0.960 | 0.936 |
| 5182 | 18.84 | 0.925 | 0.954 | 0.928 |
| 6000 | 18.77 | 0.921 | 0.950 | 0.923 |

Table (2.1.7)

*   Base porosity

** Base permeability

Figure (2.1.7)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 19 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8250.00 : 8257.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 19.21* |  | 1.000 |  |
| 200 | 19.09 | 1.37** | 0.994 | 1.000 |
| 1000 | 18.69 | 1.34 | 0.973 | 0.980 |
| 2500 | 18.16 | 1.30 | 0.945 | 0.955 |
| 4000 | 17.81 | 1.28 | 0.927 | 0.940 |
| 5182 | 17.62 | 1.27 | 0.917 | 0.932 |
| 6000 | 17.52 | 1.27 | 0.912 | 0.928 |

Table (2.1.8)

*   Base porosity

** Base permeability

Figure (2.1.8)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 20 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8300.00 : 8307.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 16.62* |  | 1.000 |  |
| 200 | 16.55 | 0.781** | 0.996 | 1.000 |
| 1000 | 16.31 | 0.776 | 0.981 | 0.993 |
| 2500 | 15.99 | 0.769 | 0.962 | 0.984 |
| 4000 | 15.77 | 0.765 | 0.949 | 0.979 |
| 5182 | 15.65 | 0.762 | 0.941 | 0.976 |
| 6000 | 15.57 | 0.761 | 0.937 | 0.974 |

Table (2.1.9)

*   Base porosity

** Base permeability

Figure (2.1.9)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 24 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8566.00 : 8573.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 11.63* |  | 1.000 |  |
| 200 | 11.54 | 0.471** | 0.992 | 1.000 |
| 1000 | 11.23 | 0.467 | 0.965 | 0.992 |
| 2500 | 10.82 | 0.462 | 0.931 | 0.981 |
| 4000 | 10.57 | 0.459 | 0.909 | 0.975 |
| 5182 | 10.43 | 0.457 | 0.897 | 0.971 |
| 6000 | 10.35 | 0.456 | 0.890 | 0.969 |

Table (2.1.10)

*   Base porosity

** Base permeability

Figure (2.1.10)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 29 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8840.00 : 8847.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 17.11* |  | 1.000 |  |
| 200 | 17.03 | 0.726** | 0.995 | 1.000 |
| 1000 | 16.75 | 0.699 | 0.979 | 0.963 |
| 2500 | 16.38 | 0.668 | 0.957 | 0.920 |
| 4000 | 16.13 | 0.650 | 0.943 | 0.895 |
| 5182 | 15.99 | 0.641 | 0.935 | 0.882 |
| 6000 | 15.91 | 0.636 | 0.930 | 0.876 |

Table (2.1.11)

*   Base porosity

** Base permeability

Figure (2.1.11)

POROSITY AND PERMEABILITY AS A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 30 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8902.00 : 8909.04 |

| Pressure (Psi) | Porosity (%) | Air permeability (mD) | Relative to base porosity | Relative to base permeability |
| --- | --- | --- | --- | --- |
| 0 | 14.50* |  | 1.000 |  |
| 200 | 14.45 | 0.453** | 0.997 | 1.000 |
| 1000 | 14.27 | 0.448 | 0.984 | 0.988 |
| 2500 | 14.04 | 0.441 | 0.968 | 0.973 |
| 4000 | 13.87 | 0.436 | 0.957 | 0.963 |
| 5182 | 13.78 | 0.434 | 0.950 | 0.958 |
| 6000 | 13.72 | 0.433 | 0.946 | 0.955 |

Table (2.1.12)

*   Base porosity

** Base permeability

Figure (2.1.12)

Section Two

P.V. Compressibility

PORE VOLUME COMPRESSIBILITY

The data derived from porosity at overburden pressure was used to calculate pore volume compressibility. Using the reduction in pore volume related to the hydrostatic net overburden pressure, a value of the hydrostatic pore volume compressibility was calculated for each overburden pressure. This value was converted to uniaxial pore volume compressibility for each overburden pressure using Teew’s equation shown below (Reference 2) and the value of 0.25 for the Poisson's ratio which was obtained from measured data. The results are presented in tables 2.2.1 through 2.2.12 and in figures 2.2.1 through 2.2.12.

Cpuni= 1/3 [ (1+ν) / (1-ν)]Cphyd

Where:

Cpuni    = uniaxial pore volume compressibility, pv/pv/psi

Cphyd   =  hydrostatic pore volume compressibility, pv/pv/psi

ν = Poisson’s ratio

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 1 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7685.02 : 7692.06 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 16.86* |  |  |
| 200 | 16.82 | 13.35 | 7.42 |
| 1000 | 16.68 | 10.92 | 6.07 |
| 2000 | 16.54 | 8.66 | 4.81 |
| 3500 | 16.38 | 6.41 | 3.56 |
| 4839 | 16.28 | 5.16 | 2.87 |
| 5200 | 16.25 | 4.91 | 2.73 |

Table (2.2.1)

*    Base porosity

Figure (2.2.1)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 2 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7686.05 : 7693.09 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 16.09* |  |  |
| 200 | 16.08 | 5.30 | 2.94 |
| 1000 | 16.02 | 4.75 | 2.64 |
| 2000 | 15.96 | 4.24 | 2.36 |
| 3500 | 15.88 | 3.73 | 2.07 |
| 4839 | 15.81 | 3.45 | 1.92 |
| 5200 | 15.79 | 3.39 | 1.88 |

Table (2.2.2)

*    Base porosity

Figure (2.2.2)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 3 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7687.07 : 7694.11 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 13.95* |  |  |
| 200 | 13.92 | 13.22 | 7.34 |
| 1000 | 13.80 | 10.82 | 6.01 |
| 2000 | 13.69 | 8.59 | 4.77 |
| 3500 | 13.55 | 6.37 | 3.54 |
| 4839 | 13.46 | 5.13 | 2.85 |
| 5200 | 13.44 | 4.88 | 2.71 |

Table (2.2.3)

*    Base porosity

Figure (2.2.3)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 24 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7710.06 : 7717.10 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 9.18* |  |  |
| 200 | 9.15 | 19.24 | 10.69 |
| 1000 | 9.03 | 15.43 | 8.57 |
| 2000 | 8.91 | 11.90 | 6.61 |
| 3500 | 8.78 | 8.38 | 4.65 |
| 4839 | 8.70 | 6.42 | 3.57 |
| 5200 | 8.68 | 6.02 | 3.35 |

Table (2.2.4)

*    Base porosity

Figure (2.2.4)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 4 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7660.00 : 7667.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 21.44* |  |  |
| 200 | 21.29 | 41.14 | 22.85 |
| 1000 | 20.80 | 32.20 | 17.89 |
| 2500 | 20.15 | 20.70 | 11.50 |
| 4000 | 19.74 | 13.71 | 7.62 |
| 5182 | 19.51 | 10.20 | 5.67 |
| 6000 | 19.39 | 8.46 | 4.70 |

Table (2.2.5)

*    Base porosity

Figure (2.2.5)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 9 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 7837.00 : 7844.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 11.84* |  |  |
| 200 | 11.79 | 22.37 | 12.43 |
| 1000 | 11.61 | 17.83 | 9.91 |
| 2500 | 11.37 | 11.98 | 6.65 |
| 4000 | 11.21 | 8.42 | 4.68 |
| 5182 | 11.12 | 6.64 | 3.69 |
| 6000 | 11.06 | 5.75 | 3.19 |

Table (2.2.6)

*    Base porosity

Figure (2.2.6)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 16 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8150.00 : 8157.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 19.76* |  |  |
| 200 | 19.70 | 18.88 | 10.49 |
| 1000 | 19.48 | 15.15 | 8.42 |
| 2500 | 19.18 | 10.35 | 5.75 |
| 4000 | 18.97 | 7.44 | 4.13 |
| 5182 | 18.84 | 5.97 | 3.32 |
| 6000 | 18.77 | 5.24 | 2.91 |

Table (2.2.7)

*    Base porosity

Figure (2.2.7)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 19 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8250.00 : 8257.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 19.21* |  |  |
| 200 | 19.09 | 33.64 | 18.69 |
| 1000 | 18.69 | 26.46 | 14.70 |
| 2500 | 18.16 | 17.21 | 9.56 |
| 4000 | 17.81 | 11.60 | 6.44 |
| 5182 | 17.62 | 8.78 | 4.88 |
| 6000 | 17.52 | 7.38 | 4.10 |

Table (2.2.8)

*    Base porosity

Figure (2.2.8)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 20 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8300.00 : 8307.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 16.62* |  |  |
| 200 | 16.55 | 23.90 | 13.28 |
| 1000 | 16.31 | 19.00 | 10.56 |
| 2500 | 15.99 | 12.69 | 7.05 |
| 4000 | 15.77 | 8.85 | 4.92 |
| 5182 | 15.65 | 6.93 | 3.85 |
| 6000 | 15.57 | 5.97 | 3.32 |

Table (2.2.9)

*    Base porosity

Figure (2.2.9)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 24 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8566.00 : 8573.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 11.63* |  |  |
| 200 | 11.54 | 42.04 | 23.36 |
| 1000 | 11.23 | 32.89 | 18.27 |
| 2500 | 10.82 | 21.11 | 11.73 |
| 4000 | 10.57 | 13.97 | 7.76 |
| 5182 | 10.43 | 10.37 | 5.76 |
| 6000 | 10.35 | 8.59 | 4.77 |

Table (2.2.10)

*    Base porosity

Figure (2.2.10)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 29 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8840.00 : 8847.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 17.11* |  |  |
| 200 | 17.03 | 27.10 | 15.06 |
| 1000 | 16.75 | 21.45 | 11.92 |
| 2500 | 16.38 | 14.18 | 7.88 |
| 4000 | 16.13 | 9.76 | 5.42 |
| 5182 | 15.99 | 7.53 | 4.19 |
| 6000 | 15.91 | 6.43 | 3.57 |

Table (2.2.11)

*    Base porosity

Figure (2.2.11)

PORE VOLUME COMPRESSIBLITYAS

A FUNCTION OF OVERBURDEN PRESSURE

Sidewall Plugs

| Client | : Waha Oil Company | Formation | : Gir |
| --- | --- | --- | --- |
| Well # | : T1-31 | Concession | : 31 |
| Sample # | : 30 | Sample Depth (ft.in) = Shifted Depth(ft.in) = | : 8902.00 : 8909.04 |

| Pressure (Psi) | Porosity (%) | Pore Volume Compressibility |  |
| --- | --- | --- | --- |
|  |  | Hydrostatic | Uniaxial |
|  |  | pv/pv/psi x 10-6 |  |
| 0 | 14.50* |  |  |
| 200 | 14.45 | 20.50 | 11.39 |
| 1000 | 14.27 | 16.39 | 9.11 |
| 2500 | 14.04 | 11.11 | 6.17 |
| 4000 | 13.87 | 7.89 | 4.39 |
| 5182 | 13.78 | 6.28 | 3.49 |
| 6000 | 13.72 | 5.48 | 3.04 |

Table (2.2.12)

*    Base porosity

Figure (2.2.12)

Section Three

Specific Water Permeability as Function of Throughput

Water Permeability as a Function of Throughput under Ambient and Overburden Pressure:-

Four of one and half inch diameter plugs, and eight of one-inch diameter sidewall plugs from the Gir formation to investigate the water permeability as function of throughput under ambient and overburden pressure. The clean, dry samples were loaded in a stainless-steel pressurized cell and evacuated for 24 hours. A solution of salinity (60,000 ppm) (formation brine) was introduced at the end of the period, followed by pressurising the system at 2500 psig for 24 hours to assist penetration. Gravimetric checks were made to ensure complete saturation of pore spaces. The samples were now ready for loading up to required conditions.

The saturated sample was individually placed in a hydrostatic core holder and confined at the average overburden pressure, that equal 4839 psi for one and half inch diameter plug and 5182 for one-inch diameter sidewall plugs. Filtered brine (to 0.45 microns) was flowed through the top of the sample at a constant rate for each sample (0.10 to 1.0 cc/min). Liquid permeability versus throughput volume was measured through the tests (Figures 2.3.1 to 2.3.24 and Tables 2.3.1 to 2.3.12).

Water permeability as a function of throughput

| Client | : Waha Oil Company | Sample# | : 1 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 16.86 | Gas Permeability | : 0.640 |
| Length (cm) | : 5.161 | Avg. OBP | : 4839 psig |
| Pore Volume (cc) | : 9.56 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 7685.02 | Shifted Depth (ft.in) | : 7692.06 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 10 | 1 | 0.275 |
| 20 | 2 | 0.257 |
| 30 | 3 | 0.254 |
| 40 | 4 | 0.242 |
| 50 | 5 | 0.254 |
| 60 | 6 | 0.240 |
| 70 | 7 | 0.240 |
| 80 | 8 | 0.252 |
| 90 | 9 | 0.265 |
| 100 | 10 | 0.259 |
| 110 | 12 | 0.264 |
| 120 | 13 | 0.248 |
| 130 | 14 | 0.247 |
| 140 | 15 | 0.257 |
| 150 | 16 | 0.243 |
| 160 | 17 | 0.241 |
| 170 | 18 | 0.240 |
| 180 | 19 | 0.250 |
| 190 | 20 | 0.248 |
| 200 | 21 | 0.243 |

Table (2.3.1)

Figure (2.3.1)

Figure (2.3.2)

Water permeability as a function of throughput

| Client | : Waha Oil Company | Sample# | : 2 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 16.09 | Gas Permeability | : 0.561 |
| Length (cm) | : 5.167 | Avg. OBP | : 4839 psig |
| Pore Volume (cc) | : 9.14 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 7686.05 | Shifted Depth (ft.in) | : 7693.09 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 10 | 1 | 0.191 |
| 20 | 2 | 0.198 |
| 30 | 3 | 0.202 |
| 40 | 4 | 0.210 |
| 50 | 5 | 0.201 |
| 60 | 7 | 0.200 |
| 70 | 8 | 0.190 |
| 80 | 9 | 0.194 |
| 90 | 10 | 0.188 |
| 100 | 11 | 0.188 |
| 110 | 12 | 0.195 |
| 120 | 13 | 0.184 |
| 130 | 14 | 0.193 |
| 140 | 15 | 0.186 |
| 150 | 16 | 0.186 |
| 160 | 18 | 0.182 |
| 170 | 19 | 0.183 |
| 180 | 20 | 0.180 |
| 190 | 21 | 0.168 |
| 200 | 22 | 0.172 |

Table (2.3.2)

Figure (2.3.3)

Figure (2.3.4)

Water permeability as a function of throughput

| Client | : Waha Oil Company | Sample# | : 3 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 13.95 | Gas Permeability | : 0.223 |
| Length (cm) | : 5.116 | Avg. OBP | : 4839 psig |
| Pore Volume (cc) | : 7.93 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 7687.07 | Shifted Depth (ft.in) | : 7694.11 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 8 | 1 | 0.070 |
| 16 | 2 | 0.069 |
| 24 | 3 | 0.071 |
| 32 | 4 | 0.072 |
| 40 | 5 | 0.066 |
| 48 | 6 | 0.068 |
| 56 | 7 | 0.077 |
| 64 | 8 | 0.070 |
| 72 | 9 | 0.070 |
| 80 | 10 | 0.073 |
| 88 | 11 | 0.073 |
| 96 | 12 | 0.073 |
| 104 | 13 | 0.072 |
| 112 | 14 | 0.072 |
| 120 | 15 | 0.072 |
| 128 | 16 | 0.072 |
| 136 | 17 | 0.071 |
| 144 | 18 | 0.071 |
| 152 | 19 | 0.071 |
| 160 | 20 | 0.071 |

Table (2.3.3)

Figure (2.3.5)

Figure (2.3.6)

Water permeability as a function of throughput

| Client | : Waha Oil Company | Sample# | : 24 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 9.18 | Gas Permeability | : 2.352 |
| Length (cm) | : 5.28 | Avg. OBP | : 4839 psig |
| Pore Volume (cc) | : 5.36 | Flow Rate (cc/min) | : 0.1 |
| Depth (ft.in) | : 7710.06 | Shifted Depth (ft.in) | : 7717.10 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 6 | 1 | 0.017 |
| 12 | 2 | 0.017 |
| 18 | 3 | 0.016 |
| 24 | 4 | 0.016 |
| 30 | 6 | 0.015 |
| 36 | 7 | 0.014 |
| 42 | 8 | 0.013 |
| 48 | 9 | 0.014 |
| 54 | 10 | 0.014 |
| 60 | 11 | 0.014 |
| 66 | 12 | 0.015 |
| 72 | 13 | 0.016 |
| 78 | 15 | 0.015 |
| 84 | 16 | 0.015 |
| 90 | 17 | 0.014 |
| 96 | 18 | 0.013 |
| 102 | 19 | 0.013 |
| 108 | 20 | 0.014 |
| 114 | 21 | 0.014 |
| 120 | 22 | 0.014 |

Table (2.3.4)

Figure (2.3.7)

Figure (2.3.8)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 4 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 21.44 | Gas Permeability | : 1.177 |
| Length (cm) | : 2.13 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 1.87 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 7660.00 | Shifted Depth (ft.in) | : 7667.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.228 |
| 4 | 2 | 0.220 |
| 6 | 3 | 0.209 |
| 8 | 4 | 0.195 |
| 10 | 5 | 0.180 |
| 12 | 6 | 0.178 |
| 14 | 7 | 0.182 |
| 16 | 9 | 0.168 |
| 18 | 10 | 0.161 |
| 20 | 11 | 0.161 |
| 22 | 12 | 0.156 |
| 24 | 13 | 0.152 |
| 26 | 14 | 0.147 |
| 28 | 15 | 0.146 |
| 30 | 16 | 0.138 |
| 32 | 17 | 0.134 |
| 34 | 18 | 0.131 |
| 36 | 19 | 0.125 |
| 38 | 20 | 0.124 |
| 40 | 21 | 0.120 |

Table (2.3.5)

Figure (2.3.9)

Figure (2.3.10)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 9 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 11.84 | Gas Permeability | : 0.880 |
| Length (cm) | : 3.14 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 1.54 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 7837.00 | Shifted Depth (ft.in) | : 7844.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.319 |
| 4 | 3 | 0.320 |
| 6 | 4 | 0.314 |
| 8 | 5 | 0.304 |
| 10 | 6 | 0.284 |
| 12 | 8 | 0.272 |
| 14 | 9 | 0.264 |
| 16 | 10 | 0.258 |
| 18 | 12 | 0.260 |
| 20 | 13 | 0.254 |
| 22 | 14 | 0.236 |
| 24 | 16 | 0.227 |
| 26 | 17 | 0.221 |
| 28 | 18 | 0.214 |
| 30 | 19 | 0.217 |
| 32 | 21 | 0.201 |
| 34 | 22 | 0.194 |
| 36 | 23 | 0.195 |
| 38 | 25 | 0.182 |
| 40 | 26 | 0.175 |

Table (2.3.6)

Figure (2.3.11)

Figure (2.3.12)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 16 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 19.76 | Gas Permeability | : 0.997 |
| Length (cm) | : 3.1 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 2.52 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 8150.00 | Shifted Depth (ft.in) | : 8157.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 3 | 1 | 0.270 |
| 6 | 2 | 0.271 |
| 9 | 4 | 0.264 |
| 12 | 5 | 0.252 |
| 15 | 6 | 0.239 |
| 18 | 7 | 0.226 |
| 21 | 8 | 0.201 |
| 24 | 10 | 0.186 |
| 27 | 11 | 0.182 |
| 30 | 12 | 0.169 |
| 33 | 13 | 0.161 |
| 36 | 14 | 0.133 |
| 39 | 15 | 0.122 |
| 42 | 17 | 0.110 |
| 45 | 18 | 0.101 |
| 48 | 19 | 0.095 |
| 51 | 20 | 0.089 |
| 54 | 21 | 0.083 |
| 57 | 23 | 0.084 |
| 60 | 24 | 0.082 |

Table (2.3.7)

Figure (2.3.13)

Figure (2.3.14)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 19 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 19.21 | Gas Permeability | : 1.366 |
| Length (cm) | : 2.8 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 2.42 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 8250.00 | Shifted Depth (ft.in) | : 8257.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.305 |
| 4 | 2 | 0.280 |
| 6 | 2 | 0.273 |
| 8 | 3 | 0.266 |
| 10 | 4 | 0.250 |
| 12 | 5 | 0.222 |
| 14 | 6 | 0.215 |
| 16 | 7 | 0.196 |
| 18 | 7 | 0.170 |
| 20 | 8 | 0.158 |
| 22 | 9 | 0.143 |
| 24 | 10 | 0.130 |
| 26 | 11 | 0.120 |
| 28 | 12 | 0.109 |
| 30 | 12 | 0.100 |
| 32 | 13 | 0.092 |
| 34 | 14 | 0.089 |
| 36 | 15 | 0.079 |
| 38 | 16 | 0.078 |
| 40 | 17 | 0.070 |

Table (2.3.8)

Figure (2.3.15)

Figure (2.3.16)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 20 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 16.62 | Gas Permeability | : 0.781 |
| Length (cm) | : 3.3 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 2.3 | Flow Rate (cc/min) | : 1.0 |
| Depth (ft.in) | : 8300.00 | Shifted Depth (ft.in) | : 8307.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.078 |
| 4 | 2 | 0.069 |
| 6 | 3 | 0.076 |
| 8 | 3 | 0.078 |
| 10 | 4 | 0.078 |
| 12 | 5 | 0.075 |
| 14 | 6 | 0.073 |
| 16 | 7 | 0.071 |
| 18 | 8 | 0.068 |
| 20 | 9 | 0.070 |
| 22 | 10 | 0.067 |
| 24 | 10 | 0.065 |
| 26 | 11 | 0.060 |
| 28 | 12 | 0.058 |
| 30 | 13 | 0.056 |
| 32 | 14 | 0.054 |
| 34 | 15 | 0.053 |

Table (2.3.9)

Figure (2.3.17)

Figure (2.3.18)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 24 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 11.63 | Gas Permeability | : 0.471 |
| Length (cm) | : 1.9 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 0.95 | Flow Rate (cc/min) | : 0.5 |
| Depth (ft.in) | : 8566.00 | Shifted Depth (ft.in) | : 8573.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 1 | 1 | 0.077 |
| 2 | 2 | 0.068 |
| 3 | 3 | 0.061 |
| 4 | 4 | 0.057 |
| 5 | 5 | 0.054 |
| 6 | 6 | 0.052 |
| 7 | 7 | 0.051 |
| 8 | 8 | 0.049 |
| 9 | 9 | 0.047 |
| 10 | 11 | 0.046 |
| 11 | 12 | 0.045 |
| 12 | 13 | 0.043 |
| 13 | 14 | 0.042 |
| 14 | 15 | 0.040 |
| 15 | 16 | 0.038 |
| 16 | 17 | 0.036 |
| 17 | 18 | 0.034 |
| 18 | 19 | 0.033 |
| 19 | 20 | 0.031 |
| 20 | 21 | 0.029 |

Table (2.3.10)

Figure (2.3.19)

Figure (2.3.20)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 29 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 17.11 | Gas Permeability | : 0.726 |
| Length (cm) | : 2.35 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 1.77 | Flow Rate (cc/min) | : 0.5 |
| Depth (ft.in) | : 8840.00 | Shifted Depth (ft.in) | : 8847.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.208 |
| 4 | 2 | 0.206 |
| 6 | 3 | 0.181 |
| 8 | 5 | 0.176 |
| 10 | 6 | 0.172 |
| 12 | 7 | 0.164 |
| 14 | 8 | 0.157 |
| 16 | 9 | 0.147 |
| 18 | 10 | 0.136 |
| 20 | 11 | 0.129 |
| 22 | 12 | 0.116 |
| 24 | 14 | 0.107 |
| 26 | 15 | 0.098 |
| 28 | 16 | 0.088 |
| 30 | 17 | 0.079 |
| 32 | 18 | 0.070 |
| 34 | 19 | 0.070 |
| 36 | 20 | 0.057 |
| 38 | 21 | 0.051 |
| 40 | 23 | 0.046 |

Table (2.3.11)

Figure (2.3.21)

Figure (2.3.22)

Water permeability as a function of throughput

Sidewall Plugs

| Client | : Waha Oil Company | Sample# | : 30 |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Salinity | : 60,000 ppm |
| Porosity (%) | : 14.50 | Gas Permeability | : 0.453 |
| Length (cm) | : 2.70 | Avg. OBP | : 5182 psig |
| Pore Volume (cc) | : 1.65 | Flow Rate (cc/min) | : 0.5 |
| Depth (ft.in) | : 8902.00 | Shifted Depth (ft.in) | : 8909.04 |

| Throughput (cc) | Throughput (PV) | Permeability (mD) |
| --- | --- | --- |
| 2 | 1 | 0.089 |
| 4 | 2 | 0.081 |
| 6 | 4 | 0.076 |
| 8 | 5 | 0.068 |
| 10 | 6 | 0.065 |
| 12 | 7 | 0.063 |
| 14 | 8 | 0.061 |
| 16 | 10 | 0.059 |
| 18 | 11 | 0.056 |
| 20 | 12 | 0.052 |
| 22 | 13 | 0.048 |
| 24 | 15 | 0.043 |
| 26 | 16 | 0.039 |
| 28 | 17 | 0.035 |
| 30 | 18 | 0.031 |
| 32 | 19 | 0.029 |
| 34 | 21 | 0.028 |
| 36 | 22 | 0.024 |
| 38 | 23 | 0.022 |
| 40 | 24 | 0.017 |

Table (2.3.12)

Figure (2.3.23)

Figure (2.3.24)

Section Four

Formation Resistivity Factor

(Ambient Conditions)

FORMATION RESISTIVITY FACTOR (at ambient conditions):

Four of one and half inch diameter plugs, and eight of one-inch diameter sidewall plugs were selected from the well T1-31, Gir formation, to perform formation resistivity factor measurements at ambient conditions. The clean, dry samples were loaded in a stainless-steel pressurized cell and evacuated for 24 hours. A sodium chloride solution of (60,000 ppm) was introduced at the end of the period, followed by pressurising the system at 2500 psig for 24 hours to assist penetration. Gravimetric checks were made to ensure complete saturation of pore spaces.

The brine saturated plugs were placed in turn between electrodes, and their electrical resistances were measured in consecutive days until ionic equilibrium was achieved between the fluid and the rock samples.

A four-point electrical device was used to measure the resistance, and the observed phase angel was set to zero.

Finally, the resistance values were used to calculate specific resistivity (Ro) of the plug samples, and formation factor was obtained by dividing the specific resistivity (Ro) of the plug sample by water resistivity (Rw= 0.1062 ohm.m @ 77oF) for the solution mentioned above. The generated data for the well T1-31 are presented in table 2.4.1 & 2.4.2 and in figure 2.4.1 & 2.4.2.

Formation Resistivity Factor(at ambient conditions)

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 0 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Air Permeability (mD) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.86 | 0.640 | 25.96 | 1.81 |
| 2 | 7686.05 | 7693.09 | 16.09 | 0.561 | 28.20 | 1.83 |
| 3 | 7687.07 | 7694.11 | 13.95 | 0.223 | 40.15 | 1.86 |
| 24 | 7710.06 | 7717.10 | 9.18 | 2.352 | 58.03 | 1.69 |

Table (2.4.1)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 0 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.80 |
| m = | 1.79 | 1.28 |

Figure 2.4.1

Formation Resistivity Factor (at ambient conditions)

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 0 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Air Permeability (mD) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 7660.00 | 7667.04 | 21.44 | 1.177 | 15.73 | 1.79 |
| 9 | 7837.00 | 7844.04 | 11.84 | 0.880 | 53.15 | 1.86 |
| 16 | 8150.00 | 8157.04 | 19.76 | 0.997 | 17.09 | 1.75 |
| 19 | 8250.00 | 8257.04 | 19.21 | 1.366 | 16.92 | 1.71 |
| 20 | 8300.00 | 8307.04 | 16.62 | 0.781 | 22.88 | 1.74 |
| 24 | 8566.00 | 8573.04 | 11.63 | 0.471 | 38.92 | 1.70 |
| 29 | 8840.04 | 8847.04 | 17.11 | 0.726 | 22.83 | 1.77 |
| 30 | 8902.00 | 8909.04 | 14.50 | 0.453 | 31.59 | 1.79 |

Table (2.4.2)

Formation Factor-Porosity Correlation

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 0 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.80 |
| m = | 1.77 | 1.86 |

Figure 2.4.2

Section Five

Formation Resistivity Index

FORMATION RESISTIVITY INDEX

Four of one and half inch diameter plugs, and eight of one-inch diameter sidewall plugs were selected from the well T1-31, Gir formation to perform formation resistivity index measurements. This test was conducted after formation resistivity factor measurements. The fully saturated samples were placed in Ultracentrifuge drainage vessels under air. With samples in place, the temperature was set to ambient temperature. The rotor speed was increased until the first designated RPM value, equivalent to 1 psi, was achieved. The centrifuge was maintained at the required speed for a minimum of 6 hours to ensure equilibrium. The samples were off-loaded after each speed, and the weight of the samples was recorded and their electrical resistances were measured by using a four-point electrical device and the observed phase angel was set to zero. This process was repeated until reached to bring the samples to lower saturation.

Sample dry weight, the fully saturated weight and the weight after each de-saturation step were used to calculate the average saturation of the sample. The resistance was used to calculate the sample resistivity (Rt) and the resistivity was divided by the sample resistivity at one hundred percent liquid saturation (Ro) which yield resistivity index (RI).

The results of all the samples are presented in tables 2.5.1 through 2.5.12 and figures 2.5.1 through 2.5.14.

*Note:  Three samples of sidewall plugs (sample 4, sample 19, sample 29) have been broken at the end of the test that mean after the last speed these samples could not tolerate the high speed due the nature of these samples, so these samples will be exclude in the next experiments except the capillary pressure and pore size distribution test by mercury injection.

Formation Resistivity Index

| Sample # = 1 | Porosity (%) = 16.86 | Permeability(mD) = 0.640 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7685.02 Shifted Depth(ft.in) = 7692.06 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.012; 0.990. 1.019; 0.984. 1.025; 0.981. 1.056; 0.981. 1.074; 0.981. 1.117; 0.979. 1.134; 0.859. 2.638; 0.561. 2.904; 0.511. 3.184; 0.460. 3.628; 0.385. Table 2.5.1: Laboratory Results |

Figure (2.5.1)

Formation Resistivity Index

| Sample # = 2 | Porosity (%) = 16.09 | Permeability(mD) = 0.561 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7686.05 Shifted Depth(ft.in) = 7693.09 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.016; 0.992. 1.033; 0.988. 1.036; 0.982. 1.042; 0.982. 1.094; 0.982. 1.098; 0.979. 1.120; 0.912. 2.254; 0.601. 3.081; 0.521. 3.622; 0.480. 3.858; 0.381. Table 2.5.2: Laboratory Results |

Figure (2.5.2)

Formation Resistivity Index

| Sample # = 3 | Porosity (%) = 13.95 | Permeability(mD) = 0.223 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7687.07 Shifted Depth(ft.in) = 7694.11 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.004; 0.997. 1.017; 0.986. 1.059; 0.979. 1.071; 0.978. 1.115; 0.976. 1.127; 0.971. 1.183; 0.957. 1.568; 0.730. 1.966; 0.647. 2.350; 0.578. 2.906; 0.507. Table 2.5.3: Laboratory Results |

Figure (2.5.3)

Formation Resistivity Index

| Sample # = 24 | Porosity (%) = 9.18 | Permeability(mD) = 2.352 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7710.06 Shifted Depth(ft.in) = 7717.10 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.008; 0.982. 1.017; 0.974. 1.063; 0.969. 1.073; 0.968. 1.077; 0.966. 1.080; 0.958. 1.106; 0.948. 1.115; 0.943. 1.217; 0.918. 1.228; 0.885. 1.244; 0.848. Table 2.5.4: Laboratory Results |

Figure (2.5.4)

Formation Resistivity Index

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Rw @ 77 deg. F | : 0.1062 ohm-m |
| No of Samples | : 4 | Saturation exponent(n) | : 1.50 |

Composite Plot

Figure (2.5.5)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 4 | Porosity (%) = 21.44 | Permeability(mD) = 1.177 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7660.00 Shifted Depth(ft.in) = 7667.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.009; 0.987. 1.015; 0.981. 1.077; 0.969. 1.092; 0.964. 1.121; 0.964. 1.137; 0.956. 1.269; 0.889. 2.471; 0.524. 2.722; 0.511. Table 2.5.5: Laboratory Results |

Figure (2.5.6)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 9 | Porosity (%) = 11.84 | Permeability(mD) = 0.880 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 7837.00 Shifted Depth(ft.in) = 7844.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.004; 0.993. 1.068; 0.981. 1.115; 0.971. 1.123; 0.966. 1.139; 0.955. 1.762; 0.718. 2.649; 0.494. 5.544; 0.318. 6.486; 0.269. Table 2.5.6: Laboratory Results |

Figure (2.5.7)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 16 | Porosity (%) = 19.76 | Permeability(mD) = 0.997 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8150.00 Shifted Depth(ft.in) = 8157.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.009; 0.993. 1.016; 0.990. 1.085; 0.989. 1.086; 0.986. 1.091; 0.986. 1.140; 0.971. 1.511; 0.767. 2.923; 0.481. 3.999; 0.388. Table 2.5.7: Laboratory Results |

Figure (2.5.8)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 19 | Porosity (%) = 19.21 | Permeability(mD) = 1.366 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8250.00 Shifted Depth(ft.in) = 8257.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.058; 0.971. 1.111; 0.932. 1.179; 0.924. 1.182; 0.921. 1.255; 0.896. 1.305; 0.892. 1.667; 0.695. 2.585; 0.565. 2.633; 0.551. Table 2.5.8: Laboratory Results |

Figure (2.5.9)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 20 | Porosity (%) = 16.62 | Permeability(mD) = 0.781 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8300.00 Shifted Depth(ft.in) = 8307.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.008; 0.964. 1.053; 0.950. 1.095; 0.947. 1.135; 0.944. 1.213; 0.941. 1.239; 0.936. 1.637; 0.778. 2.782; 0.476. 3.486; 0.397. Table 2.5.9: Laboratory Results |

Figure (2.5.10)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 24 | Porosity (%) = 11.63 | Permeability(mD) = 0.471 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8566.00 Shifted Depth(ft.in) = 8573.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.115; 0.896. 1.151; 0.890. 1.167; 0.886. 1.209; 0.880. 1.287; 0.877. 1.277; 0.875. 1.328; 0.873. 1.854; 0.672. 2.100; 0.583. Table 2.5.10: Laboratory Results |

Figure (2.5.11)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 29 | Porosity (%) = 17.11 | Permeability(mD) = 0.726 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8840.00 Shifted Depth(ft.in) = 8847.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.013; 0.967. 1.037; 0.941. 1.055; 0.940. 1.125; 0.939. 1.178; 0.936. 1.251; 0.924. 1.353; 0.868. 2.459; 0.539. 2.821; 0.504. Table 2.5.11: Laboratory Results |

Figure (2.5.12)

Formation Resistivity Index

Sidewall Plugs

| Sample # = 30 | Porosity (%) = 14.50 | Permeability(mD) = 0.453 |
| --- | --- | --- |
| Client: Waha Oil Company | Well: T1-31 | Sample Depth (ft.in) = 8902.00 Shifted Depth(ft.in) = 8909.04 |
| Resistivity Index (RI); Brine Saturation (Sw), fraction. 1.000; 1.000. 1.004; 0.977. 1.010; 0.973. 1.044; 0.938. 1.066; 0.938. 1.133; 0.927. 1.233; 0.918. 1.339; 0.854. 1.345; 0.840. 1.607; 0.829. Table 2.5.12: Laboratory Results |

Figure (2.5.13)

Formation Resistivity Index

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Concession | : 31 |
| Formation | : Gir | Rw @ 77 deg. F | : 0.1062 ohm-m |
| No of Samples | : 8 | Saturation exponent(n) | : 1.44 |

Composite Plot

Figure (2.5.14)

Section Six

Formation Resistivity Factor

(Overburden pressure)

FORMATION RESISTIVITY FACTOR (at overburden pressure): -

Samples previously used for the formation factor measurements at ambient conditions were scheduled for formation factor testing at overburden conditions.

The fully saturated samples were individually placed between stainless steel end pieces, in a rubber sleeve and immersed in oil within a hydrostatic core holder.

All connecting pipe work and contact surfaces were fully saturated with simulated formation brine to ensure electrical continuity.

The overburden pressure was applied on the cores up to 400 psig in the manner described in Nieto paper (Reference 3).

The volume of brine expelled being monitored to determine the reduction in pore volume. The resistance values were measured at each requested pressure (400, 800, 1500, 2500,3500 and 4839 psig) for one and half inch diameter plugs, and (400, 1000, 2500, 4000,4500 and 5182 psig) for one-inch diameter sidewall. The samples were allowed to stabilise before their resistances were measured.  On achieving equilibrium, samples resistances were measured using two electrode systems. The resistance values were used to calculate specific resistivity (Ro) of the plug samples, and formation factor was obtained by dividing the specific resistivity (Ro) of the plug sample by water resistivity (Rw). The generated data are presented in tables 2.6.1 through 2.6.12 and in figures 2.6.1 through 2.6.16.

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 400 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.86 | 25.72 | 1.82 |
| 2 | 7686.05 | 7693.09 | 16.09 | 28.72 | 1.84 |
| 3 | 7687.07 | 7694.11 | 13.95 | 40.27 | 1.88 |
| 24 | 7710.06 | 7717.10 | 9.18 | 58.08 | 1.70 |

Table (2.6.1)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 400 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.79 |
| m = | 1.80 | 1.29 |

Figure (2.6.1)

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 800 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.77 | 26.00 | 1.83 |
| 2 | 7686.05 | 7693.09 | 16.02 | 29.36 | 1.85 |
| 3 | 7687.07 | 7694.11 | 13.83 | 41.79 | 1.89 |
| 24 | 7710.06 | 7717.10 | 9.10 | 59.63 | 1.71 |

Table (2.6.2)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 800 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.78 |
| m = | 1.81 | 1.30 |

Figure (2.6.2)

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 1500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.62 | 26.97 | 1.84 |
| 2 | 7686.05 | 7693.09 | 15.97 | 30.17 | 1.86 |
| 3 | 7687.07 | 7694.11 | 13.70 | 43.64 | 1.90 |
| 24 | 7710.06 | 7717.10 | 8.98 | 64.93 | 1.72 |

Table (2.6.3)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 1500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.75 |
| m = | 1.82 | 1.32 |

Figure (2.6.3)

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 2500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.47 | 27.87 | 1.85 |
| 2 | 7686.05 | 7693.09 | 15.93 | 30.76 | 1.87 |
| 3 | 7687.07 | 7694.11 | 13.59 | 44.79 | 1.91 |
| 24 | 7710.06 | 7717.10 | 8.81 | 66.41 | 1.73 |

Table (2.6.4)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 2500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.71 |
| m = | 1.83 | 1.33 |

Figure (2.6.4)

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 3500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.41 | 29.14 | 1.87 |
| 2 | 7686.05 | 7693.09 | 15.90 | 31.44 | 1.88 |
| 3 | 7687.07 | 7694.11 | 13.56 | 45.89 | 1.92 |
| 24 | 7710.06 | 7717.10 | 8.81 | 69.30 | 1.74 |

Table (2.6.5)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 3500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.68 |
| m = | 1.84 | 1.35 |

Figure (2.6.5)

Formation Resistivity Factor

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 1 | 7685.02 | 7692.06 | 16.30 | 30.01 | 1.88 |
| 2 | 7686.05 | 7693.09 | 15.83 | 32.30 | 1.89 |
| 3 | 7687.07 | 7694.11 | 13.48 | 47.32 | 1.93 |
| 24 | 7710.06 | 7717.10 | 8.72 | 72.41 | 1.76 |

Table (2.6.6)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 2.67 |
| m = | 1.85 | 1.37 |

Figure (2.6.6)

Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Overburden Pressure | 0 psig | 400 psig | 800 psig | 1500 psig | 2500 psig | 3500 psig | 4839 psig |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cementation Factor (Free Fit) | m = 1.79 | m = 1.80 | m = 1.81 | m = 1.82 | m = 1.83 | m = 1.84 | m = 1.85 |

Composite Plot (Forced Fit)

(Figure 2.6.7) Formation Factor-Porosity Correlation

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 4 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Overburden Pressure | 0 psig | 400 psig | 800 psig | 1500 psig | 2500 psig | 3500 psig | 4839 psig |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cementation Factor (Forced Fit) | m = 1.28 | m = 1.29 | m =1.30 | m = 1.32 | m = 1.33 | m = 1.35 | m = 1.37 |

Composite Plot (Free Fit)

(Figure 2.6.8)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 400 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.84 | 53.71 | 1.87 |
| 16 | 8150.00 | 8157.04 | 19.76 | 17.47 | 1.76 |
| 20 | 8300.00 | 8307.04 | 16.62 | 23.24 | 1.75 |
| 24 | 8566.00 | 8573.04 | 11.63 | 39.89 | 1.71 |
| 30 | 8902.00 | 8909.04 | 14.50 | 32.60 | 1.80 |

Table (2.6.7)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 400 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.83 |
| m = | 1.78 | 1.88 |

Figure (2.6.9)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 1000 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.51 | 58.77 | 1.88 |
| 16 | 8150.00 | 8157.04 | 19.60 | 18.04 | 1.78 |
| 20 | 8300.00 | 8307.04 | 16.16 | 23.24 | 1.77 |
| 24 | 8566.00 | 8573.04 | 11.05 | 45.23 | 1.73 |
| 30 | 8902.00 | 8909.04 | 14.12 | 34.57 | 1.83 |

Table (2.6.8)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 1000 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.82 |
| m = | 1.79 | 1.89 |

Figure (2.6.10)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 2500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.35 | 64.46 | 1.91 |
| 16 | 8150.00 | 8157.04 | 19.47 | 18.77 | 1.79 |
| 20 | 8300.00 | 8307.04 | 15.98 | 26.33 | 1.78 |
| 24 | 8566.00 | 8573.04 | 10.82 | 49.42 | 1.75 |
| 30 | 8902.00 | 8909.04 | 14.00 | 35.31 | 1.85 |

Table (2.6.9)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 2500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.78 |
| m = | 1.81 | 1.94 |

Figure (2.6.11)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4000 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.25 | 66.96 | 1.92 |
| 16 | 8150.00 | 8157.04 | 19.44 | 19.15 | 1.80 |
| 20 | 8300.00 | 8307.04 | 15.88 | 27.22 | 1.80 |
| 24 | 8566.00 | 8573.04 | 10.58 | 54.67 | 1.79 |
| 30 | 8902.00 | 8909.04 | 13.95 | 36.75 | 1.87 |

Table (2.6.10)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4000 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.74 |
| m = | 1.83 | 1.98 |

Figure (2.6.12)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.22 | 68.82 | 1.93 |
| 16 | 8150.00 | 8157.04 | 19.43 | 19.57 | 1.82 |
| 20 | 8300.00 | 8307.04 | 15.86 | 28.21 | 1.81 |
| 24 | 8566.00 | 8573.04 | 10.58 | 57.46 | 1.80 |
| 30 | 8902.00 | 8909.04 | 13.80 | 37.43 | 1.88 |

Table (2.6.11)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4500 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.72 |
| m = | 1.84 | 2.01 |

Figure (2.6.13)

Formation Resistivity Factor

Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 5182 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062ohm-m |

| Sample # | Depth (ft.in) | Shifted Depth (ft.in) | Porosity (%) | Formation Factor | Cementation Exponent "m" |
| --- | --- | --- | --- | --- | --- |
| 9 | 7837.00 | 7844.04 | 11.18 | 72.33 | 1.95 |
| 16 | 8150.00 | 8157.04 | 19.42 | 20.17 | 1.83 |
| 20 | 8300.00 | 8307.04 | 15.84 | 29.32 | 1.83 |
| 24 | 8566.00 | 8573.04 | 10.47 | 61.50 | 1.82 |
| 30 | 8902.00 | 8909.04 | 13.80 | 37.43 | 1.88 |

Table (2.6.12)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 5182 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

|  | Forced fit | Free fit |
| --- | --- | --- |
| a = | 1 | 0.71 |
| m = | 1.86 | 2.03 |

Figure (2.6.14)

Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Overburden Pressure | 0 psig | 400 psig | 1000 psig | 2500 psig | 4000 psig | 4500 psig | 5182 psig |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cementation Factor (Free Fit) | m = 1.77 | m = 1.78 | m = 1.79 | m = 1.81 | m = 1.83 | m = 1.84 | m = 1.86 |

Composite Plot (Forced Fit)

(Figure 2.6.15) Formation Factor-Porosity Correlation Sidewall Plugs

| Client | : Waha Oil Company | Salinity | : 60,000 ppm |
| --- | --- | --- | --- |
| Well | : T1-31 | Overburden pressure | : 4839 psig |
| Formation | : Gir | Concession | : 31 |
| No of Samples | : 5 | Rw @ 77 deg. F | : 0.1062 ohm-m |

| Overburden Pressure | 0 psig | 400 psig | 1000 psig | 2500 psig | 4000 psig | 4500 psig | 5182 psig |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cementation Factor (Forced Fit) | m = 1.86 | m = 1.88 | m =1.89 | m = 1.94 | m = 1.98 | m = 2.01 | m = 2.03 |

Composite Plot (Free Fit)

(Figure 2.6.16)

Section Seven

Oil-Water Capillary Pressure by Centrifuge

Oil-Water Drainage Cycle: –

Four of one and half inch diameter plug samples were selected from the well T1-31 to perform centrifuge capillary pressure. The brine saturated samples were placed in Beckman Ultracentrifuge drainage vessels under Oil. With samples in place, the temperature was set to ambient temperature. The rotor speed was increased until the first designated RPM value, equivalent to 1 psi, was achieved. The centrifuge was maintained at the required speed for 6 hours to ensure equilibrium. When water production was ceased, samples were off-loaded, and the volume of water expelled was recorded. This volume and the pore volume of sample were used to calculate the average saturation. This process was repeated until a speed equivalent to 120 psi, was reached.

Speed in RPM, length of sample, density difference between oil and water and the radius of rotation were used to calculate the pressure at the sample inner face. The local saturation at the inner face of the sample was calculated using Hassler technique (Hassler, G.L., Brunner, E. (1945)) (Reference 4).

The results are presented in Tables (2.7.1-2.7.4) and figures (2.7.1-2.8.4)

Note: -

According to what was observed in the resistivity index by the ultra-centrifuges, some of

the sidewall samples were broken at the end of the test, and therefore the oil-water and

water-oil capillary pressure tests were cancelled.

Oil-Water Drainage Cycle

| Company | : W.O.C | Core Porosity, % | : 16.86 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.640 |
| Sample No. | : 1 | Irreducible Water Saturation, % | : 29.89 |
| Salinity, ppm | : 60,000 | Water Density, gm/cc | : 1.04 |
| Shifted sample depth(ft.in) | : 7692.06 | Oil Density, gm/cc | : 0.8451 |

Table (2.7.1)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 100.0 |
| 1 | 96.41 |
| 2 | 91.80 |
| 4 | 81.66 |
| 8 | 69.57 |
| 15 | 58.63 |
| 35 | 45.30 |
| 60 | 37.95 |
| 120 | 29.89 |

Figure (2.7.1)

Oil-Water Drainage Cycle

| Company | : W.O.C | Core Porosity, % | : 16.09 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.561 |
| Sample No. | : 2 | Irreducible Water Saturation, % | : 31.82 |
| Salinity, ppm | : 60,000 | Water Density, gm/cc | : 1.04 |
| Shifted sample depth(ft.in) | : 7693.09 | Oil Density, gm/cc | : 0.8451 |

Table (2.7.2)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 100.00 |
| 1 | 98.00 |
| 2 | 97.76 |
| 4 | 86.96 |
| 8 | 74.06 |
| 15 | 62.43 |
| 35 | 48.23 |
| 60 | 40.49 |
| 120 | 31.82 |

Figure (2.7.2)

Oil-Water Drainage Cycle

| Company | : W.O.C | Core Porosity, % | : 13.95 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.223 |
| Sample No. | : 3 | Irreducible Water Saturation, % | : 68.00 |
| Salinity, ppm | : 60,000 | Water Density, gm/cc | : 1.04 |
| Shifted sample depth(ft.in) | : 7694.11 | Oil Density, gm/cc | : 0.8451 |

Table (2.7.3)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 100.00 |
| 1 | 98.04 |
| 2 | 89.80 |
| 4 | 83.95 |
| 8 | 79.84 |
| 15 | 77.16 |
| 35 | 73.00 |
| 60 | 71.00 |
| 120 | 68.00 |

Figure (2.7.3)

Oil-Water Drainage Cycle

| Company | : W.O.C | Core Porosity, % | : 9.18 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 2.352 |
| Sample No. | : 24 | Irreducible Water Saturation, % | : 71.00 |
| Salinity, ppm | : 60,000 | Water Density, gm/cc | : 1.04 |
| Shifted sample depth(ft.in) | : 7717.10 | Oil Density, gm/cc | : 0.8451 |

Table (2.7.4)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 100.0 |
| 1 | 99.81 |
| 2 | 99.53 |
| 4 | 98.79 |
| 8 | 91.15 |
| 15 | 86.16 |
| 35 | 79.00 |
| 60 | 75.00 |
| 120 | 71.00 |

Figure (2.7.4)

Section Eight

Water-Oil Capillary Pressure by Centrifuge

Water - Oil Imbibition Cycle:

Four of one and half inch diameter plug samples were selected by the company from the well T1-31, Gir formation to perform centrifuge capillary pressure. The Samples at irreducible water saturation and initial oil saturation were placed in Ultracentrifuge imbibition vessel under simulated formation brine. With samples in place, the temperature was set to ambient temperature. The rotor speed was increased until the first designated RPM value, equivalent to 1 psi, was achieved. The centrifuge was maintained at the required speed for 6 hours to ensure equilibrium. When oil production was ceased, Samples were off – loaded and the volume of oil expelled was recorded. This volume and the pore volume of sample were used to calculate the average saturation. This process was repeated until a speed equivalent to 120 psi, was reached.

Speed in RPM, length of sample, density difference between oil and water and the radius of rotation were used to calculate the pressure at the sample inner face. The local saturation at the sample inner face of the sample was calculated using Hassler-Brunner technique (Hassler, G.L., Brunner, E. (1945) (Reference 4).

The generated data is presented in tables 2.8.1 through 2.8.4 and in figures 2.8.1 through 2.8.4.

*Note: -

According to what was observed in the resistivity index by the ultra-centrifuges, some of

the sidewall samples were broken at the end of the test, and therefore the oil-water and

water-oil capillary pressure tests were cancelled.

Water-Oil Imbibition Cycle

| Company | : W.O.C | Core Porosity, % | : 16.86 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.640 |
| Sample No. | : 1 | Irreducible Water Saturation, % | : 29.89 |
| Salinity, ppm | : 60,000 | Residual Oil Saturation, % | : 41.70 |
| Sample Depth(ft.in) | : 7692.06 | Brine Density, gm/cc | : 1.04 |

Table (2.8.1)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 29.89 |
| - 1 | 33.18 |
| - 2 | 41.23 |
| - 4 | 46.99 |
| - 8 | 51.04 |
| - 15 | 53.68 |
| - 35 | 56.15 |
| - 60 | 57.26 |
| - 120 | 58.30 |

Figure (2.8.1)

Water-Oil Imbibition Cycle

| Company | : W.O.C | Core Porosity, % | : 16.09 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.561 |
| Sample No. | : 2 | Irreducible Water Saturation, % | : 31.82 |
| Salinity, ppm | : 60,000 | Residual Oil Saturation, % | : 38.48 |
| Sample Depth(ft.in) | : 7693.09 | Brine Density, gm/cc | : 1.04 |

Table (2.8.2)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 31.82 |
| - 1 | 35.41 |
| - 2 | 43.83 |
| - 4 | 49.76 |
| - 8 | 53.97 |
| - 15 | 56.72 |
| - 35 | 59.28 |
| - 60 | 60.43 |
| - 120 | 61.52 |

Figure (2.8.2)

Water-Oil Imbibition Cycle

| Company | : W.O.C | Core Porosity, % | : 13.95 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 0.223 |
| Sample No. | : 3 | Irreducible Water Saturation, % | : 68.00 |
| Salinity, ppm | : 60,000 | Residual Oil Saturation, % | : 25.49 |
| Sample Depth(ft.in) | : 7694.11 | Brine Density, gm/cc | : 1.04 |

Table (2.8.3)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 68.00 |
| - 1 | 69.53 |
| - 2 | 71.13 |
| - 4 | 72.26 |
| - 8 | 73.07 |
| - 15 | 73.59 |
| - 35 | 74.08 |
| - 60 | 74.30 |
| - 120 | 74.51 |

Figure (2.8.3)

Water-Oil Imbibition Cycle

| Company | : W.O.C | Core Porosity, % | : 9.18 |
| --- | --- | --- | --- |
| Well No. | : T1-31 | Core Air Permeability, (mD) | : 2.352 |
| Sample No. | : 24 | Irreducible Water Saturation, % | : 71.00 |
| Salinity, ppm | : 60,000 | Residual Oil Saturation, % | : 19.00 |
| Sample Depth(ft.in) | : 7717.10 | Brine Density, gm/cc | : 1.04 |

Table (2.8.1)

| Ambient Conditions Capillary Pressure, psi | Liquid Saturation (%) |
| --- | --- |
| 0 | 71.00 |
| - 1 | 73.13 |
| - 2 | 75.44 |
| - 4 | 77.07 |
| - 8 | 78.23 |
| - 15 | 78.98 |
| - 35 | 79.69 |
| - 60 | 80.00 |
| - 120 | 81.00 |

Figure (2.8.1)

Section Nine

Mercury Injection Capillary Pressure

MERCURY INJECTION CAPILLARY PRESSURE AND PORE

SIZE DISTRIBUTION:

Five of one and half inch diameter plugs, and eight of one-inch diameter sidewall plugs (total=13 core sample) were selected from the well T1-31, Gir formation to perform mercury injection capillary pressure measurements Using Micromeritics Autopore IV 9510 Mercury Porosimeter with windows software.

Prior to testing, the routine petrophysical properties of porosity, grain density and permeability to air were re-measured.

Each sample was individually weighted and a sample information file that describes the sample and gives the analysis conditions and other parameters for the analysis was created. The instrument performs semi- automatically injection of mercury at programmed pressure steps from less than one psi to 60,000 psia.

To begin the test, each sample was individually loaded into a calibrated glass penetrometer. The penetrometer consists of a sample chamber and precision-bore glass capillary whose volume was selected to be just larger than the measured routine pore volume. The outside of the glass capillary is plated with metal which acts as one plate of a capacitor. The mercury within the capillary acts as the other capacitance plate and then installs the loaded penetrometer into the low-pressure port. The first phase of the low-pressure analysis is the evacuation of gases from the penetrometer for 20-30 minutes, and then the penetrometer is backfilled automatically with Mercury.

For pressure up to 30 psia, air pressure is used, when the low-pressure analysis is complete, the penetrometer is removed from the low-pressure port and installed in a high-pressure port in which hydraulic pressure oil is used, pore volume data are calculated by determining the volume of mercury remaining in the penetrometer. As pressure increases, mercury moves into the sample’s pores, vacating the stem (Drainage or Intrusion), the greater the pressure, the smaller the pore diameter into which the mercury can be forced, then the pressure is decreased gradually in steps to withdraw the mercury from the pores in pressure steps from 60,000 psia to 14 psia (Imbibition or Extrusion).

Mercury volumes are expressed in milligram per gram of dry weight of rock sample, the output data is presented in both tabular and graphical format. Graphical presentation of the data includes:

Cumulative Mercury volume versus air–Mercury capillary pressure to determine the threshold pressure. Air-Mercury capillary pressure versus wetting phase saturation for the two cycles. An accumulative volume injection curve and the incremental volume curve which is particularly useful for interpreting interconnecting pore dimensions are plotted against pore radii. Pore size is computed using La Place equation for cylindrical pores (B.F Swanson, 1985, Burdine et al; 1950, and John Shafer1 & John Neasham2; SCA2000-21) (References 5, 6 & 7).

Conformance/Closure Corrections

The MICP test measures and records Hg intrusion data for each pressure step. The first pressure step is the pressure increase from the “mercury fill pressure” to the first pressure point (e.g., 1.6 - 1.8 psia). As each pressure step is sequentially performed, mercury progressively fills (i.e., “conforms to” or “closes around”) any sample surface irregularities such as micro-fractures and is, hence, recorded as Hg intrusion. At some higher pressure step, however, sufficient pressure is achieved to cause mercury (i.e., the non-wetting phase) to intrude the largest pore throats controlling the sample’s PV. The pressure at which mercury commences to occupy the actual pore system of the sample being tested is called the “Threshold Pressure” or “closure pressure”. All intrusion data recorded up to this initial entry pressure is subtracted from the MICP raw data output as the “closure correction”.

Selection of the closure pressure for a given MICP test is determined by generating a X-Y plot of “percent sample pore volume (PV) occupied by mercury” vs. “Hg intrusion pressure”. This semi-log plot allows identification of an “inflection point” indicating a “sudden” increase in the amount of mercury intrusion. This inflection point, termed the “Threshold/closure pressure”, identifies the pressure at which mercury begins to occupy sample pore volume bounded by the largest pore throats; Pittman (1971) (Reference 8).

Baseline Error Correction Methods:

During MICP test, the penetrometer parts, the mercury, and the sample are all affected differently by the pressures exerted on them during analysis. Each material compresses to a different rate.

Also, the increasing pressure within the high pressure port can cause the temperature of one or more of these materials to rise. As a result, analysis data may show intrusion where none actually

exists. For example, if the sample compresses sharply at a given pressure (compared to the compression of the mercury at that pressure), mercury moves from the penetrometer stem into the sample bulb to fill the space vacated by the shrinking sample. This reduction in the amount of mercury in the stem is interpreted by the software as intrusion.

MICP instrumentation software allows for incorporation of a “Blank Correction File” into the sample test “raw data”, which adjusts the generated MICP raw data output for equipment related compressibility effects but not for sample grain compressibility.

These baseline errors can be reduced or eliminated through one of two correction methods:

1- A “blank correction by formula” file contained in the MICP instrumentation software.

2- An “operator generated” blank correction file.

The first method (Blank Correction by Formula) was used for the entire samples selected from well T1-31.

Formula Correction

Through large number of blank runs using a variety of pressure tables and penetrometers filled with mercury, Micromeritics has developed a formula for correction of each penetrometer type. The AutoPore software provides this formula, so it can be easily incorporated into the raw generated raw data during the measurements procedures.

Note:

- The core samples selected for mercury injection measurements were re-cut to approximately 1”X1” or less to fit into the sample cup of the Autopore instrumentation and in some cases pieces can be used.

- The first pressure value in all tables below is the Threshold Pressure.

- The routine core analysis was re-measured before conducting the MICP measurements.

The generated data is presented in tables 2.9.1 through 2.9.13 and in figures 2.9.1 through 2.9.54.

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 1 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.739 | Threshold Pressure, (psia) | : 217.498 |
| Porosity, (%) | : 16.86 | Dry Weight, (gm) | : 9.615 |
| Air Perm., (mD) | : 0.640 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7685.02 | Sample Shifted Depth, (ft.in) | : 7692.06 |

Table (2.9.1)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 217.498 | D | 0.000 | 100.000 | 0.000 | 0.490 |
| 268.034 | D | 2.920 | 97.080 | 2.920 | 0.398 |
| 327.173 | D | 26.078 | 73.922 | 23.158 | 0.326 |
| 417.593 | D | 60.287 | 39.713 | 34.209 | 0.255 |
| 515.792 | D | 71.211 | 28.789 | 10.924 | 0.207 |
| 636.569 | D | 78.376 | 21.624 | 7.165 | 0.168 |
| 795.721 | D | 83.479 | 16.521 | 5.103 | 0.134 |
| 991.075 | D | 86.861 | 13.139 | 3.382 | 0.108 |
| 1201.17 | D | 89.130 | 10.870 | 2.270 | 0.089 |
| 1495.71 | D | 91.098 | 8.902 | 1.968 | 0.0713 |
| 1897.72 | D | 92.627 | 7.373 | 1.528 | 0.0562 |
| 2344.83 | D | 93.767 | 6.233 | 1.141 | 0.0455 |
| 2895.05 | D | 94.613 | 5.387 | 0.846 | 0.0368 |
| 3592.59 | D | 95.271 | 4.729 | 0.658 | 0.0297 |
| 4485.77 | D | 95.798 | 4.202 | 0.527 | 0.0238 |
| 5582.65 | D | 96.312 | 3.688 | 0.515 | 0.0191 |
| 6883.97 | D | 96.692 | 3.308 | 0.380 | 0.0155 |
| 8585.07 | D | 96.909 | 3.0914 | 0.216 | 0.0124 |
| 10584.5 | D | 96.985 | 3.0154 | 0.076 | 0.0101 |
| 13183.6 | D | 96.985 | 3.0154 | 0.000 | 0.0081 |
| 14782.1 | D | 96.985 | 3.0154 | 0.000 | 0.0072 |
| 16383.9 | D | 96.985 | 3.0154 | 0.000 | 0.0065 |
| 19978.6 | D | 96.985 | 3.0154 | 0.000 | 0.0053 |
| 24992.0 | D | 96.985 | 3.0154 | 0.000 | 0.0043 |
| 29988.2 | D | 96.985 | 3.0154 | 0.000 | 0.0036 |
| 34992.4 | D | 96.985 | 3.0154 | 0.000 | 0.0030 |
| 39990.9 | D | 96.985 | 3.0154 | 0.000 | 0.0027 |
| 44987.4 | D | 96.985 | 3.0154 | 0.000 | 0.0024 |

Table (2.9.1-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 49987.4 | D | 96.985 | 3.0154 | 0.000 | 0.0021 |
| 54991.7 | D | 96.985 | 3.0154 | 0.000 | 0.0019 |
| 59981.7 | D | 96.985 | 3.0154 | 0.000 | 0.0018 |
| 46104.8 | I | 96.985 | 3.0154 | - | - |
| 35510.3 | I | 96.985 | 3.0154 | - | - |
| 27299.1 | I | 96.985 | 3.0154 | - | - |
| 21008.0 | I | 96.985 | 3.0154 | - | - |
| 16007.1 | I | 96.985 | 3.0154 | - | - |
| 12410.7 | I | 96.985 | 3.0154 | - | - |
| 9611.5 | I | 96.985 | 3.0154 | - | - |
| 7310.2 | I | 96.985 | 3.0154 | - | - |
| 5713.6 | I | 96.985 | 3.0154 | - | - |
| 4230.5 | I | 96.985 | 3.0154 | - | - |
| 3303.1 | I | 96.985 | 3.0154 | - | - |
| 2595.6 | I | 96.985 | 3.0154 | - | - |
| 2005.8 | I | 96.985 | 3.0154 | - | - |
| 1500.8 | I | 96.985 | 3.0154 | - | - |
| 1199.4 | I | 96.985 | 3.0154 | - | - |
| 901.7 | I | 96.985 | 3.0154 | - | - |
| 698.0 | I | 96.985 | 3.0154 | - | - |
| 503.0 | I | 95.987 | 4.013 | - | - |
| 400.0 | I | 94.850 | 5.150 | - | - |
| 303.8 | I | 92.780 | 7.220 | - | - |
| 243.9 | I | 90.577 | 9.423 | - | - |
| 193.8 | I | 87.619 | 12.381 | - | - |
| 148.2 | I | 82.662 | 17.338 | - | - |
| 113.6 | I | 76.030 | 23.970 | - | - |
| 88.4 | I | 68.816 | 31.184 | - | - |
| 68.2 | I | 62.933 | 37.067 | - | - |
| 53.3 | I | 60.620 | 39.380 | - | - |
| 33.0 | I | 60.096 | 39.904 | - | - |
| 18.4 | I | 60.091 | 39.909 | - | - |

Figure (2.9.1)-sample #1

Figure (2.9.2)-sample #1

Figure (2.9.3)-sample # 1

Figure (2.9.4)-Sample # 1

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 2 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.681 | Threshold Pressure, (psia) | : 267.923 |
| Porosity, (%) | : 16.09 | Dry Weight, (gm) | : 9.173 |
| Air Perm., (mD) | : 0.561 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7686.05 | Sample Shifted Depth, (ft.in) | : 7693.09 |

Table (2.9.2)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 267.923 | D | 0.000 | 100.000 | 0.000 | 0.398 |
| 328.376 | D | 13.946 | 86.054 | 13.946 | 0.325 |
| 416.982 | D | 53.160 | 46.840 | 39.214 | 0.256 |
| 516.519 | D | 69.296 | 30.704 | 16.136 | 0.207 |
| 636.887 | D | 77.613 | 22.387 | 8.316 | 0.167 |
| 795.822 | D | 83.149 | 16.851 | 5.537 | 0.134 |
| 988.981 | D | 86.777 | 13.223 | 3.628 | 0.108 |
| 1199.71 | D | 89.056 | 10.944 | 2.279 | 0.089 |
| 1496.07 | D | 90.916 | 9.084 | 1.860 | 0.0713 |
| 1898.15 | D | 92.426 | 7.574 | 1.510 | 0.0562 |
| 2348.63 | D | 93.530 | 6.470 | 1.104 | 0.0454 |
| 2894.83 | D | 94.491 | 5.509 | 0.961 | 0.0368 |
| 3592.64 | D | 95.290 | 4.710 | 0.799 | 0.0297 |
| 4492.36 | D | 95.845 | 4.155 | 0.555 | 0.0237 |
| 5581.68 | D | 96.381 | 3.619 | 0.536 | 0.0191 |
| 6884.75 | D | 96.628 | 3.372 | 0.247 | 0.0155 |
| 8584.20 | D | 96.784 | 3.2157 | 0.156 | 0.0124 |
| 10584.6 | D | 96.814 | 3.1863 | 0.029 | 0.0101 |
| 13183.5 | D | 96.836 | 3.1640 | 0.022 | 0.0081 |
| 14781.9 | D | 96.836 | 3.1640 | 0.000 | 0.0072 |
| 16381.1 | D | 96.836 | 3.1640 | 0.000 | 0.0065 |
| 19980.4 | D | 96.836 | 3.1640 | 0.000 | 0.0053 |
| 24989.0 | D | 96.836 | 3.1640 | 0.000 | 0.0043 |
| 29993.7 | D | 96.836 | 3.1640 | 0.000 | 0.0036 |
| 34990.7 | D | 96.836 | 3.1640 | 0.000 | 0.0030 |
| 39988.1 | D | 96.836 | 3.1640 | 0.000 | 0.0027 |
| 44986.9 | D | 96.836 | 3.1640 | 0.000 | 0.0024 |
| 49986.6 | D | 96.836 | 3.1640 | 0.000 | 0.0021 |

Table (2.9.2-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 54986.6 | D | 96.836 | 3.1640 | 0.000 | 0.0019 |
| 59983.9 | D | 96.836 | 3.1640 | 0.000 | 0.0018 |
| 46109.1 | I | 96.836 | 3.1640 | - | - |
| 35501.2 | I | 96.836 | 3.1640 | - | - |
| 27303.5 | I | 96.836 | 3.1640 | - | - |
| 21006.5 | I | 96.836 | 3.1640 | - | - |
| 16007.0 | I | 96.836 | 3.1640 | - | - |
| 12408.2 | I | 96.836 | 3.1640 | - | - |
| 9613.7 | I | 96.836 | 3.1640 | - | - |
| 7304.7 | I | 96.836 | 3.1640 | - | - |
| 5710.4 | I | 96.836 | 3.1640 | - | - |
| 4313.4 | I | 96.836 | 3.1640 | - | - |
| 3302.3 | I | 96.836 | 3.1640 | - | - |
| 2612.4 | I | 96.836 | 3.1640 | - | - |
| 2006.1 | I | 96.836 | 3.1640 | - | - |
| 1498.8 | I | 96.836 | 3.1640 | - | - |
| 1201.0 | I | 96.836 | 3.1640 | - | - |
| 903.7 | I | 96.836 | 3.1640 | - | - |
| 702.5 | I | 96.836 | 3.1640 | - | - |
| 503.6 | I | 96.241 | 3.759 | - | - |
| 401.7 | I | 94.700 | 5.300 | - | - |
| 303.7 | I | 92.518 | 7.482 | - | - |
| 242.1 | I | 89.640 | 10.360 | - | - |
| 190.8 | I | 85.279 | 14.721 | - | - |
| 146.7 | I | 77.904 | 22.096 | - | - |
| 112.0 | I | 68.101 | 31.899 | - | - |
| 87.3 | I | 64.026 | 35.974 | - | - |
| 69.2 | I | 63.394 | 36.606 | - | - |
| 53.5 | I | 63.275 | 36.725 | - | - |

Figure (2.9.5)-sample # 2

Figure (2.9.6)-sample # 2

Figure (2.9.7)-sample # 2

Figure (2.9.8)-Sample # 2

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 3 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.367 | Threshold Pressure, (psia) | : 326.344 |
| Porosity, (%) | : 13.95 | Dry Weight, (gm) | : 6.080 |
| Air Perm., (mD) | : 0.223 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7687.07 | Sample Shifted Depth, (ft.in) | : 7694.11 |

Table (2.9.3)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 326.344 | D | 0.000 | 100.000 | 0.0000 | 0.3268 |
| 415.516 | D | 8.613 | 91.387 | 8.6127 | 0.2567 |
| 517.165 | D | 38.690 | 61.310 | 30.0770 | 0.2062 |
| 636.386 | D | 64.617 | 35.383 | 25.9272 | 0.1676 |
| 796.484 | D | 73.202 | 26.798 | 8.5855 | 0.1339 |
| 985.753 | D | 78.489 | 21.511 | 5.2870 | 0.1082 |
| 1199.18 | D | 81.693 | 18.307 | 3.2038 | 0.0889 |
| 1497.75 | D | 84.491 | 15.509 | 2.7976 | 0.0712 |
| 1897.53 | D | 86.756 | 13.2437 | 2.2656 | 0.0562 |
| 2348.13 | D | 88.326 | 11.6743 | 1.5694 | 0.0454 |
| 2894.43 | D | 89.678 | 10.3222 | 1.3522 | 0.0369 |
| 3594.81 | D | 90.713 | 9.2872 | 1.0350 | 0.0297 |
| 4501.47 | D | 91.771 | 8.2294 | 1.0578 | 0.0237 |
| 5581.53 | D | 92.742 | 7.2576 | 0.9718 | 0.0191 |
| 6884.05 | D | 93.379 | 6.6209 | 0.6367 | 0.0155 |
| 8585.49 | D | 93.896 | 6.1038 | 0.5171 | 0.0124 |
| 10583.3 | D | 94.253 | 5.7468 | 0.3570 | 0.0101 |
| 13181.9 | D | 94.701 | 5.2994 | 0.4473 | 0.0081 |
| 14781.6 | D | 94.701 | 5.2994 | 0.0000 | 0.0072 |
| 16380.3 | D | 94.701 | 5.2994 | 0.0000 | 0.0065 |
| 19981.3 | D | 94.701 | 5.2994 | 0.0000 | 0.0053 |
| 24990.9 | D | 94.701 | 5.2994 | 0.0000 | 0.0043 |
| 29987.1 | D | 94.701 | 5.2994 | 0.0000 | 0.0036 |
| 34990.9 | D | 94.701 | 5.2994 | 0.0000 | 0.0030 |
| 39988.2 | D | 94.701 | 5.2994 | 0.0000 | 0.0027 |
| 44990.0 | D | 94.701 | 5.2994 | 0.0000 | 0.0024 |
| 49988.4 | D | 94.701 | 5.2994 | 0.0000 | 0.0021 |
| 54986.4 | D | 94.701 | 5.2994 | 0.0000 | 0.0019 |

Table (2.9.3-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 59985.1 | D | 94.701 | 5.2994 | 0.000 | 0.0018 |
| 46103.3 | I | 94.701 | 5.2994 | - | - |
| 35509.4 | I | 94.701 | 5.2994 | - | - |
| 27305.0 | I | 94.701 | 5.2994 | - | - |
| 21004.2 | I | 94.701 | 5.2994 | - | - |
| 16007.0 | I | 94.701 | 5.2994 | - | - |
| 12409.1 | I | 94.701 | 5.2994 | - | - |
| 9618.2 | I | 94.701 | 5.2994 | - | - |
| 7312.6 | I | 94.701 | 5.2994 | - | - |
| 5716.5 | I | 94.701 | 5.2994 | - | - |
| 4066.7 | I | 94.701 | 5.2994 | - | - |
| 3308.4 | I | 94.701 | 5.2994 | - | - |
| 2611.9 | I | 94.701 | 5.2994 | - | - |
| 1989.1 | I | 94.555 | 5.4451 | - | - |
| 1503.9 | I | 94.097 | 5.9027 | - | - |
| 1205.1 | I | 93.476 | 6.5244 | - | - |
| 904.1 | I | 92.669 | 7.3307 | - | - |
| 703.0 | I | 91.934 | 8.0661 | - | - |
| 499.8 | I | 90.021 | 9.9793 | - | - |
| 399.8 | I | 87.744 | 12.256 | - | - |
| 302.4 | I | 82.603 | 17.397 | - | - |
| 240.5 | I | 75.782 | 24.218 | - | - |
| 193.8 | I | 67.351 | 32.649 | - | - |
| 147.1 | I | 66.660 | 33.340 | - | - |
| 113.8 | I | 66.364 | 33.636 | - | - |

Figure (2.9.9)-sample # 3

Figure (2.9.10)-sample # 3

Figure (2.9.11)-sample # 3

Figure (2.9.12)-Sample # 3

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 10 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.565 | Threshold Pressure, (psia) | : 172.069 |
| Porosity, (%) | : 9.67 | Dry Weight, (gm) | : 13.160 |
| Air Perm., (mD) | : 0.980 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7696.09 | Sample Shifted Depth, (ft.in) | : 7704.01 |

Table (2.9.4)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 172.069 | D | 0.000 | 100.000 | 0.000 | 0.620 |
| 219.950 | D | 0.465 | 99.535 | 0.465 | 0.485 |
| 266.792 | D | 1.357 | 98.643 | 0.893 | 0.400 |
| 328.066 | D | 3.669 | 96.331 | 2.312 | 0.325 |
| 415.450 | D | 8.809 | 91.191 | 5.140 | 0.257 |
| 518.856 | D | 18.885 | 81.115 | 10.076 | 0.206 |
| 635.607 | D | 34.107 | 65.893 | 15.222 | 0.168 |
| 798.008 | D | 59.344 | 40.656 | 25.237 | 0.134 |
| 989.588 | D | 73.722 | 26.278 | 14.377 | 0.108 |
| 1200.73 | D | 79.575 | 20.425 | 5.853 | 0.089 |
| 1497.77 | D | 84.209 | 15.791 | 4.635 | 0.0712 |
| 1896.24 | D | 87.540 | 12.460 | 3.331 | 0.0562 |
| 2345.06 | D | 89.856 | 10.144 | 2.315 | 0.0455 |
| 2893.42 | D | 91.495 | 8.505 | 1.640 | 0.0369 |
| 3595.29 | D | 92.991 | 7.009 | 1.496 | 0.0297 |
| 4487.73 | D | 94.044 | 5.956 | 1.053 | 0.0238 |
| 5583.13 | D | 94.990 | 5.010 | 0.945 | 0.0191 |
| 6883.04 | D | 95.758 | 4.242 | 0.768 | 0.0155 |
| 8585.27 | D | 96.401 | 3.5990 | 0.643 | 0.0124 |
| 10583.5 | D | 96.902 | 3.0981 | 0.501 | 0.0101 |
| 13181.1 | D | 97.149 | 2.8511 | 0.247 | 0.0081 |
| 14783.6 | D | 97.192 | 2.8082 | 0.043 | 0.0072 |
| 16380.5 | D | 97.192 | 2.8082 | 0.000 | 0.0065 |
| 19981.1 | D | 97.192 | 2.8082 | 0.000 | 0.0053 |
| 24991.6 | D | 97.192 | 2.8082 | 0.000 | 0.0043 |
| 29990.1 | D | 97.192 | 2.8082 | 0.000 | 0.0036 |
| 34990.2 | D | 97.192 | 2.8082 | 0.000 | 0.0030 |
| 39989.8 | D | 97.192 | 2.8082 | 0.000 | 0.0027 |

Table (2.9.4-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 44987.2 | D | 97.192 | 2.8082 | 0.000 | 0.0024 |
| 49992.2 | D | 97.192 | 2.8082 | 0.000 | 0.0021 |
| 54986.2 | D | 97.192 | 2.8082 | 0.000 | 0.0019 |
| 59982.0 | D | 97.192 | 2.8082 | 0.000 | 0.0018 |
| 46097.5 | I | 97.192 | 2.8082 | - | - |
| 35506.9 | I | 97.192 | 2.8082 | - | - |
| 27304.1 | I | 97.192 | 2.8082 | - | - |
| 21009.3 | I | 97.192 | 2.8082 | - | - |
| 16004.2 | I | 97.192 | 2.8082 | - | - |
| 12405.5 | I | 97.192 | 2.8082 | - | - |
| 9613.0 | I | 97.192 | 2.8082 | - | - |
| 7312.4 | I | 97.192 | 2.8082 | - | - |
| 5713.2 | I | 97.192 | 2.8082 | - | - |
| 4310.6 | I | 97.192 | 2.8082 | - | - |
| 3305.3 | I | 97.192 | 2.8082 | - | - |
| 2604.5 | I | 97.192 | 2.8082 | - | - |
| 2004.3 | I | 97.192 | 2.8082 | - | - |
| 1499.5 | I | 97.192 | 2.8082 | - | - |
| 1199.6 | I | 97.192 | 2.808 | - | - |
| 899.6 | I | 96.854 | 3.146 | - | - |
| 701.4 | I | 95.622 | 4.378 | - | - |
| 502.9 | I | 93.139 | 6.861 | - | - |
| 403.7 | I | 90.887 | 9.113 | - | - |
| 301.0 | I | 87.085 | 12.915 | - | - |
| 244.1 | I | 83.141 | 16.859 | - | - |
| 190.5 | I | 78.342 | 21.658 | - | - |
| 146.3 | I | 72.851 | 27.149 | - | - |
| 113.4 | I | 68.941 | 31.059 | - | - |
| 86.1 | I | 66.424 | 33.576 | - | - |
| 68.9 | I | 65.589 | 34.411 | - | - |
| 52.8 | I | 65.131 | 34.869 | - | - |
| 32.8 | I | 64.656 | 35.344 | - | - |
| 18.1 | I | 64.225 | 35.775 | - | - |

Figure (2.9.13)-sample # 10

Figure (2.9.14)-sample # 10

Figure (2.9.15)-sample # 10

Figure (2.9.16)-Sample # 10

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 20 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.262 | Threshold Pressure, (psia) | : 216.916 |
| Porosity, (%) | : 7.81 | Dry Weight, (gm) | : 9.006 |
| Air Perm., (mD) | : 5.511 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7706.11 | Sample Shifted Depth, (ft.in) | : 7714.03 |

Table (2.9.5)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 216.916 | D | 0.000 | 100.000 | 0.0000 | 0.4917 |
| 265.632 | D | 0.452 | 99.548 | 0.452 | 0.4015 |
| 329.522 | D | 2.286 | 97.714 | 1.833 | 0.3237 |
| 418.460 | D | 10.307 | 89.693 | 8.0216 | 0.2549 |
| 515.721 | D | 22.148 | 77.852 | 11.8413 | 0.2068 |
| 635.537 | D | 42.891 | 57.109 | 20.7424 | 0.1678 |
| 795.019 | D | 58.899 | 41.101 | 16.0079 | 0.1342 |
| 988.116 | D | 68.939 | 31.061 | 10.0399 | 0.1079 |
| 1200.904 | D | 75.356 | 24.644 | 6.4175 | 0.0888 |
| 1497.80 | D | 79.634 | 20.366 | 4.2779 | 0.0712 |
| 1895.77 | D | 82.996 | 17.004 | 3.3617 | 0.0563 |
| 2348.09 | D | 85.438 | 14.562 | 2.4424 | 0.0454 |
| 2894.87 | D | 87.358 | 12.6419 | 1.9200 | 0.0368 |
| 3596.80 | D | 88.905 | 11.0954 | 1.5465 | 0.0297 |
| 4487.07 | D | 90.126 | 9.8745 | 1.2210 | 0.0238 |
| 5585.06 | D | 90.775 | 9.2251 | 0.6494 | 0.0191 |
| 6884.60 | D | 91.317 | 8.6825 | 0.5426 | 0.0155 |
| 8586.73 | D | 91.800 | 8.1995 | 0.4830 | 0.0124 |
| 10583.5 | D | 92.100 | 7.8997 | 0.2999 | 0.0101 |
| 13184.3 | D | 92.323 | 7.6770 | 0.2227 | 0.0081 |
| 14780.9 | D | 92.776 | 7.2242 | 0.4528 | 0.0072 |
| 16381.3 | D | 92.776 | 7.2242 | 0.0000 | 0.0065 |
| 19981.3 | D | 92.776 | 7.2242 | 0.0000 | 0.0053 |
| 24991.6 | D | 92.776 | 7.2242 | 0.0000 | 0.0043 |
| 29990.4 | D | 92.776 | 7.2242 | 0.0000 | 0.0036 |
| 34987.8 | D | 92.776 | 7.2242 | 0.0000 | 0.0030 |
| 39991.3 | D | 92.776 | 7.2242 | 0.0000 | 0.0027 |
| 44987.0 | D | 92.776 | 7.2242 | 0.0000 | 0.0024 |

Table (2.9.5-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 49989.7 | D | 92.776 | 7.2242 | 0.000 | 0.0021 |
| 54990.8 | D | 92.776 | 7.2242 | 0.000 | 0.0019 |
| 59982.8 | D | 92.776 | 7.2242 | 0.000 | 0.0018 |
| 46096.2 | I | 92.776 | 7.2242 | - | - |
| 35503.8 | I | 92.776 | 7.2242 | - | - |
| 27304.5 | I | 92.776 | 7.2242 | - | - |
| 21008.5 | I | 92.776 | 7.2242 | - | - |
| 16007.8 | I | 92.776 | 7.2242 | - | - |
| 12411.6 | I | 92.776 | 7.2242 | - | - |
| 9608.0 | I | 92.776 | 7.2242 | - | - |
| 7310.5 | I | 92.776 | 7.2242 | - | - |
| 5717.3 | I | 92.776 | 7.2242 | - | - |
| 4137.3 | I | 92.776 | 7.2242 | - | - |
| 3307.0 | I | 92.776 | 7.2242 | - | - |
| 2597.1 | I | 92.563 | 7.4372 | - | - |
| 1995.9 | I | 91.512 | 8.4877 | - | - |
| 1501.9 | I | 89.978 | 10.0222 | - | - |
| 1203.5 | I | 88.272 | 11.7276 | - | - |
| 902.0 | I | 86.040 | 13.9605 | - | - |
| 699.9 | I | 83.801 | 16.1986 | - | - |
| 500.3 | I | 79.987 | 20.0127 | - | - |
| 399.2 | I | 77.575 | 22.425 | - | - |
| 302.9 | I | 75.926 | 24.074 | - | - |
| 242.1 | I | 75.059 | 24.941 | - | - |
| 194.0 | I | 74.443 | 25.557 | - | - |
| 147.6 | I | 74.268 | 25.732 | - | - |
| 112.5 | I | 74.035 | 25.965 | - | - |
| 87.7 | I | 73.841 | 26.159 | - | - |
| 68.2 | I | 73.663 | 26.337 | - | - |

Figure (2.9.17)-sample # 20

Figure (2.9.18)-sample # 20

Figure (2.9.19)-sample # 20

Figure (2.9.20)-Sample # 20

Composite plot

Figure (2.9.21)

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 4 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 1.578 | Threshold Pressure, (psia) | : 216.723 |
| Porosity, (%) | : 21.44 | Dry Weight, (gm) | : 15.929 |
| Air Perm., (mD) | : 1.177 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7660.00 | Sample Shifted Depth, (ft.in) | : 7667.04 |

Table (2.9.6)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 216.723 | D | 0.000 | 100.000 | 0.000 | 0.492 |
| 265.821 | D | 4.830 | 95.170 | 4.830 | 0.401 |
| 327.173 | D | 28.327 | 71.673 | 23.496 | 0.326 |
| 418.216 | D | 60.692 | 39.308 | 32.366 | 0.255 |
| 517.394 | D | 72.663 | 27.337 | 11.971 | 0.206 |
| 637.254 | D | 79.404 | 20.596 | 6.740 | 0.167 |
| 799.552 | D | 84.289 | 15.711 | 4.885 | 0.133 |
| 990.469 | D | 87.472 | 12.528 | 3.183 | 0.108 |
| 1196.74 | D | 89.668 | 10.332 | 2.196 | 0.089 |
| 1493.67 | D | 91.674 | 8.326 | 2.007 | 0.0714 |
| 1894.13 | D | 93.242 | 6.758 | 1.568 | 0.0563 |
| 2345.71 | D | 94.434 | 5.566 | 1.192 | 0.0455 |
| 2893.63 | D | 95.278 | 4.722 | 0.843 | 0.0369 |
| 3593.51 | D | 96.027 | 3.973 | 0.749 | 0.0297 |
| 4484.09 | D | 96.544 | 3.456 | 0.517 | 0.0238 |
| 5584.16 | D | 97.023 | 2.977 | 0.479 | 0.0191 |
| 6884.58 | D | 97.293 | 2.707 | 0.270 | 0.0155 |
| 8584.45 | D | 97.424 | 2.576 | 0.131 | 0.0124 |
| 10582.1 | D | 97.478 | 2.522 | 0.054 | 0.0101 |
| 13182.2 | D | 97.478 | 2.522 | 0.000 | 0.0081 |
| 14782.3 | D | 97.478 | 2.522 | 0.000 | 0.0072 |
| 16379.8 | D | 97.478 | 2.522 | 0.000 | 0.0065 |
| 19980.7 | D | 97.478 | 2.522 | 0.000 | 0.0053 |
| 24991.7 | D | 97.478 | 2.522 | 0.000 | 0.0043 |
| 29989.6 | D | 97.478 | 2.522 | 0.000 | 0.0036 |
| 34987.6 | D | 97.478 | 2.522 | 0.000 | 0.0030 |
| 39991.1 | D | 97.478 | 2.522 | 0.000 | 0.0027 |
| 44988.7 | D | 97.478 | 2.522 | 0.000 | 0.0024 |

Table (2.9.6-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 49987.3 | D | 97.478 | 2.522 | 0.000 | 0.0021 |
| 54980.4 | D | 97.478 | 2.522 | 0.000 | 0.0019 |
| 59981.3 | D | 97.478 | 2.522 | 0.000 | 0.0018 |
| 46103.6 | I | 97.478 | 2.522 | - | - |
| 35503.2 | I | 97.478 | 2.522 | - | - |
| 27307.2 | I | 97.478 | 2.522 | - | - |
| 20999.1 | I | 97.478 | 2.522 | - | - |
| 16003.7 | I | 97.478 | 2.522 | - | - |
| 12411.7 | I | 97.478 | 2.522 | - | - |
| 9605.4 | I | 97.478 | 2.522 | - | - |
| 7310.1 | I | 97.478 | 2.522 | - | - |
| 5713.5 | I | 97.478 | 2.522 | - | - |
| 4171.2 | I | 97.478 | 2.522 | - | - |
| 3304.9 | I | 97.478 | 2.522 | - | - |
| 2601.9 | I | 97.478 | 2.522 | - | - |
| 1999.5 | I | 97.478 | 2.522 | - | - |
| 1502.2 | I | 97.478 | 2.522 | - | - |
| 1199.2 | I | 97.478 | 2.522 | - | - |
| 902.2 | I | 97.478 | 2.522 | - | - |
| 701.9 | I | 96.693 | 3.307 | - | - |
| 503.3 | I | 95.153 | 4.847 | - | - |
| 402.4 | I | 93.599 | 6.401 | - | - |
| 302.3 | I | 91.186 | 8.814 | - | - |
| 242.0 | I | 88.388 | 11.612 | - | - |
| 191.4 | I | 84.807 | 15.193 | - | - |
| 148.3 | I | 79.281 | 20.719 | - | - |
| 112.8 | I | 71.865 | 28.135 | - | - |
| 88.3 | I | 64.439 | 35.561 | - | - |
| 68.0 | I | 60.057 | 39.943 | - | - |
| 53.4 | I | 58.193 | 41.807 | - | - |
| 32.7 | I | 57.880 | 42.120 | - | - |
| 18.5 | I | 57.797 | 42.203 | - | - |

Figure (2.9.22)-sample # 4

Figure (2.9.23)-sample # 4

Figure (2.9.24)-sample # 4

Figure (2.9.25)-Sample # 4

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 9 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.483 | Threshold Pressure, (psia) | : 112.217 |
| Porosity, (%) | : 11.84 | Dry Weight, (gm) | : 15.929 |
| Air Perm., (mD) | : 0.880 | Formation | : Gir |
| Sample Depth, (ft.in) | : 7837.00 | Sample Shifted Depth, (ft.in) | : 7844.04 |

Table (2.9.7)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 112.217 | D | 0.000 | 100.000 | 0.000 | 0.950 |
| 136.559 | D | 3.898 | 96.102 | 3.898 | 0.781 |
| 172.416 | D | 25.028 | 74.972 | 21.130 | 0.619 |
| 215.908 | D | 49.954 | 50.046 | 24.926 | 0.494 |
| 265.711 | D | 58.833 | 41.167 | 8.879 | 0.401 |
| 329.539 | D | 64.867 | 35.133 | 6.033 | 0.324 |
| 417.880 | D | 69.652 | 30.348 | 4.785 | 0.255 |
| 515.399 | D | 72.409 | 27.591 | 2.757 | 0.207 |
| 635.467 | D | 74.385 | 25.615 | 1.976 | 0.168 |
| 795.573 | D | 76.340 | 23.660 | 1.956 | 0.134 |
| 986.506 | D | 77.097 | 22.903 | 0.757 | 0.108 |
| 1196.20 | D | 78.914 | 21.086 | 1.817 | 0.089 |
| 1497.72 | D | 80.289 | 19.711 | 1.375 | 0.0712 |
| 1899.52 | D | 81.675 | 18.325 | 1.386 | 0.0562 |
| 2348.67 | D | 82.901 | 17.099 | 1.226 | 0.0454 |
| 2896.27 | D | 84.055 | 15.945 | 1.154 | 0.0368 |
| 3592.39 | D | 84.786 | 15.214 | 0.731 | 0.0297 |
| 4485.28 | D | 85.378 | 14.622 | 0.592 | 0.0238 |
| 5581.75 | D | 85.861 | 14.139 | 0.484 | 0.0191 |
| 6885.44 | D | 86.118 | 13.882 | 0.257 | 0.0155 |
| 8585.72 | D | 86.362 | 13.638 | 0.243 | 0.0124 |
| 10583.7 | D | 86.651 | 13.349 | 0.289 | 0.0101 |
| 13182.7 | D | 86.723 | 13.277 | 0.072 | 0.0081 |
| 14782.5 | D | 86.723 | 13.277 | 0.000 | 0.0072 |
| 16381.4 | D | 86.723 | 13.277 | 0.000 | 0.0065 |
| 19981.9 | D | 86.723 | 13.277 | 0.000 | 0.0053 |
| 24994.0 | D | 86.723 | 13.277 | 0.000 | 0.0043 |
| 29992.1 | D | 86.723 | 13.277 | 0.000 | 0.0036 |

Table (2.9.7-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 34994.7 | D | 86.723 | 13.277 | 0.000 | 0.0030 |
| 39990.1 | D | 86.723 | 13.277 | 0.000 | 0.0027 |
| 44989.1 | D | 86.723 | 13.277 | 0.000 | 0.0024 |
| 49989.2 | D | 86.723 | 13.277 | 0.000 | 0.0021 |
| 54988.5 | D | 86.723 | 13.277 | 0.000 | 0.0019 |
| 59986.1 | D | 86.723 | 13.277 | 0.000 | 0.0018 |
| 46102.5 | I | 86.723 | 13.277 | - | - |
| 35507.8 | I | 86.723 | 13.277 | - | - |
| 27305.0 | I | 86.723 | 13.277 | - | - |
| 21012.2 | I | 86.723 | 13.277 | - | - |
| 16008.2 | I | 86.723 | 13.277 | - | - |
| 12407.9 | I | 86.723 | 13.277 | - | - |
| 9617.5 | I | 86.723 | 13.277 | - | - |
| 7315.6 | I | 86.723 | 13.277 | - | - |
| 5710.9 | I | 86.723 | 13.277 | - | - |
| 4306.7 | I | 86.723 | 13.277 | - | - |
| 3312.5 | I | 86.723 | 13.277 | - | - |
| 2610.3 | I | 86.723 | 13.277 | - | - |
| 1995.6 | I | 86.723 | 13.277 | - | - |
| 1502.1 | I | 86.723 | 13.277 | - | - |
| 1205.9 | I | 86.723 | 13.277 | - | - |
| 899.5 | I | 86.723 | 13.277 | - | - |
| 703.9 | I | 86.723 | 13.277 | - | - |
| 502.9 | I | 86.723 | 13.277 | - | - |
| 399.7 | I | 86.723 | 13.277 | - | - |
| 302.4 | I | 84.563 | 15.437 | - | - |
| 242.2 | I | 81.233 | 18.767 | - | - |
| 191.8 | I | 74.147 | 25.853 | - | - |
| 148.6 | I | 60.767 | 39.233 | - | - |
| 113.2 | I | 58.854 | 41.146 | - | - |

Figure (2.9.26)-sample # 9

Figure (2.9.27)-sample # 9

Figure (2.9.28)-sample # 9

Figure (2.9.29)-Sample # 9

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 16 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.737 | Threshold Pressure, (psia) | : 170.910 |
| Porosity, (%) | : 19.76 | Dry Weight, (gm) | : 7.894 |
| Air Perm., (mD) | : 0.997 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8150.00 | Sample Shifted Depth, (ft.in) | : 8157.04 |

Table (2.9.8)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 170.910 | D | 0.000 | 100.000 | 0.0000 | 0.6241 |
| 216.556 | D | 1.582 | 98.418 | 1.5821 | 0.4925 |
| 267.472 | D | 21.219 | 78.781 | 19.6371 | 0.3988 |
| 328.058 | D | 55.829 | 44.171 | 34.6097 | 0.3251 |
| 418.057 | D | 69.001 | 30.999 | 13.1725 | 0.2551 |
| 515.415 | D | 75.975 | 24.025 | 6.9740 | 0.2069 |
| 636.476 | D | 81.171 | 18.829 | 5.1953 | 0.1676 |
| 796.637 | D | 85.015 | 14.985 | 3.8441 | 0.1339 |
| 987.814 | D | 87.830 | 12.170 | 2.8152 | 0.1080 |
| 1200.26 | D | 89.852 | 10.148 | 2.0220 | 0.0889 |
| 1497.14 | D | 91.776 | 8.224 | 1.9236 | 0.0712 |
| 1898.31 | D | 93.332 | 6.6678 | 1.5565 | 0.0562 |
| 2349.38 | D | 94.074 | 5.9260 | 0.7418 | 0.0454 |
| 2894.67 | D | 94.339 | 5.6606 | 0.2654 | 0.0368 |
| 3591.74 | D | 94.533 | 5.4674 | 0.1932 | 0.0297 |
| 4484.96 | D | 94.533 | 5.4674 | 0.0000 | 0.0238 |
| 5583.26 | D | 94.533 | 5.4674 | 0.0000 | 0.0191 |
| 6884.24 | D | 94.533 | 5.4674 | 0.0000 | 0.0155 |
| 8586.17 | D | 94.533 | 5.4674 | 0.0000 | 0.0124 |
| 10584.0 | D | 94.533 | 5.4674 | 0.0000 | 0.0101 |
| 13183.8 | D | 94.533 | 5.4674 | 0.0000 | 0.0081 |
| 14781.5 | D | 94.533 | 5.4674 | 0.0000 | 0.0072 |
| 16381.8 | D | 94.533 | 5.4674 | 0.0000 | 0.0065 |
| 19980.5 | D | 94.533 | 5.4674 | 0.0000 | 0.0053 |
| 24992.1 | D | 94.533 | 5.4674 | 0.0000 | 0.0043 |
| 29991.8 | D | 94.533 | 5.4674 | 0.0000 | 0.0036 |
| 34993.6 | D | 94.533 | 5.4674 | 0.0000 | 0.0030 |
| 39992.4 | D | 94.533 | 5.4674 | 0.0000 | 0.0027 |

Table (2.9.8-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 44988.9 | D | 94.533 | 5.4674 | 0.000 | 0.0024 |
| 49989.1 | D | 94.533 | 5.4674 | 0.000 | 0.0021 |
| 54988.7 | D | 94.533 | 5.4674 | 0.000 | 0.0019 |
| 59981.9 | D | 94.533 | 5.4674 | 0.000 | 0.0018 |
| 46106.0 | I | 94.533 | 5.4674 | - | - |
| 35510.8 | I | 94.533 | 5.4674 | - | - |
| 27298.7 | I | 94.533 | 5.4674 | - | - |
| 21008.8 | I | 94.533 | 5.4674 | - | - |
| 16003.2 | I | 94.533 | 5.4674 | - | - |
| 12407.9 | I | 94.533 | 5.4674 | - | - |
| 9616.9 | I | 94.533 | 5.4674 | - | - |
| 7309.5 | I | 94.533 | 5.4674 | - | - |
| 5712.1 | I | 94.533 | 5.4674 | - | - |
| 4304.2 | I | 94.533 | 5.4674 | - | - |
| 3299.9 | I | 94.533 | 5.4674 | - | - |
| 2598.6 | I | 94.533 | 5.4674 | - | - |
| 2003.2 | I | 94.533 | 5.4674 | - | - |
| 1501.3 | I | 94.533 | 5.4674 | - | - |
| 1204.5 | I | 94.533 | 5.4674 | - | - |
| 900.8 | I | 94.533 | 5.4674 | - | - |
| 704.3 | I | 94.533 | 5.4674 | - | - |
| 501.6 | I | 93.181 | 6.8189 | - | - |
| 398.2 | I | 90.379 | 9.621 | - | - |
| 302.1 | I | 85.606 | 14.394 | - | - |
| 240.5 | I | 80.702 | 19.298 | - | - |
| 191.9 | I | 74.375 | 25.625 | - | - |
| 147.4 | I | 64.936 | 35.064 | - | - |
| 111.5 | I | 56.908 | 43.092 | - | - |
| 88.6 | I | 53.678 | 46.322 | - | - |
| 66.5 | I | 53.161 | 46.839 | - | - |
| 53.3 | I | 52.913 | 47.087 |  |  |
| 31.8 | I | 52.779 | 47.221 |  |  |
| 18.5 | I | 52.404 | 47.596 |  |  |

Figure (2.9.30)-sample # 16

Figure (2.9.31)-sample # 16

Figure (2.9.32)-sample # 16

Figure (2.9.33)-Sample # 16

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 19 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.335 | Threshold Pressure, (psia) | : 172.712 |
| Porosity, (%) | : 19.21 | Dry Weight, (gm) | : 3.355 |
| Air Perm., (mD) | : 1.366 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8250.00 | Sample Shifted Depth, (ft.in) | : 8257.04 |

Table (2.9.9)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 172.712 | D | 0.000 | 100.000 | 0.000 | 0.618 |
| 216.157 | D | 2.144 | 97.856 | 2.144 | 0.493 |
| 269.226 | D | 24.554 | 75.446 | 22.410 | 0.396 |
| 328.525 | D | 50.959 | 49.041 | 26.405 | 0.325 |
| 416.889 | D | 65.086 | 34.914 | 14.127 | 0.256 |
| 516.812 | D | 71.872 | 28.128 | 6.786 | 0.206 |
| 638.303 | D | 78.729 | 21.271 | 6.856 | 0.167 |
| 797.252 | D | 83.263 | 16.737 | 4.535 | 0.134 |
| 987.944 | D | 86.952 | 13.048 | 3.689 | 0.108 |
| 1198.83 | D | 89.165 | 10.835 | 2.212 | 0.089 |
| 1496.96 | D | 90.889 | 9.111 | 1.724 | 0.0713 |
| 1893.68 | D | 92.349 | 7.651 | 1.460 | 0.0563 |
| 2345.28 | D | 93.455 | 6.545 | 1.106 | 0.0455 |
| 2895.07 | D | 94.567 | 5.433 | 1.113 | 0.0368 |
| 3592.40 | D | 95.056 | 4.944 | 0.488 | 0.0297 |
| 4488.25 | D | 95.354 | 4.646 | 0.298 | 0.0238 |
| 5581.44 | D | 95.354 | 4.646 | 0.000 | 0.0191 |
| 6881.36 | D | 95.354 | 4.646 | 0.000 | 0.0155 |
| 8583.40 | D | 95.354 | 4.6461 | 0.000 | 0.0124 |
| 10583.4 | D | 95.354 | 4.6461 | 0.000 | 0.0101 |
| 13183.8 | D | 95.354 | 4.6461 | 0.000 | 0.0081 |
| 14782.7 | D | 95.354 | 4.6461 | 0.000 | 0.0072 |
| 16379.1 | D | 95.354 | 4.6461 | 0.000 | 0.0065 |
| 19978.3 | D | 95.354 | 4.6461 | 0.000 | 0.0053 |
| 24990.9 | D | 95.354 | 4.6461 | 0.000 | 0.0043 |
| 29992.3 | D | 95.354 | 4.6461 | 0.000 | 0.0036 |
| 34987.9 | D | 95.354 | 4.6461 | 0.000 | 0.0030 |
| 39986.2 | D | 95.354 | 4.6461 | 0.000 | 0.0027 |

Table (2.9.9-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 44986.1 | D | 95.354 | 4.6461 | 0.000 | 0.0024 |
| 49988.4 | D | 95.354 | 4.6461 | 0.000 | 0.0021 |
| 54987.7 | D | 95.354 | 4.6461 | 0.000 | 0.0019 |
| 59983.2 | D | 95.354 | 4.6461 | 0.000 | 0.0018 |
| 46095.2 | I | 95.354 | 4.6461 | - | - |
| 35500.7 | I | 95.354 | 4.6461 | - | - |
| 27303.0 | I | 95.354 | 4.6461 | - | - |
| 21007.0 | I | 95.354 | 4.6461 | - | - |
| 16012.2 | I | 95.354 | 4.6461 | - | - |
| 12409.3 | I | 95.354 | 4.6461 | - | - |
| 9609.0 | I | 95.354 | 4.6461 | - | - |
| 7310.1 | I | 95.354 | 4.6461 | - | - |
| 5710.5 | I | 95.354 | 4.6461 | - | - |
| 4304.7 | I | 95.343 | 4.6567 | - | - |
| 3306.0 | I | 95.343 | 4.6567 | - | - |
| 2608.3 | I | 95.343 | 4.6567 | - | - |
| 2001.0 | I | 95.343 | 4.6567 | - | - |
| 1500.3 | I | 95.256 | 4.7437 | - | - |
| 1203.9 | I | 95.121 | 4.879 | - | - |
| 902.1 | I | 94.499 | 5.501 | - | - |
| 702.9 | I | 94.255 | 5.745 | - | - |
| 498.1 | I | 92.979 | 7.021 | - | - |
| 398.0 | I | 91.216 | 8.784 | - | - |
| 301.2 | I | 89.168 | 10.832 | - | - |
| 242.0 | I | 87.324 | 12.676 | - | - |
| 188.0 | I | 83.433 | 16.567 | - | - |
| 145.9 | I | 77.050 | 22.950 | - | - |
| 111.4 | I | 67.116 | 32.884 | - | - |
| 86.4 | I | 56.678 | 43.322 | - | - |
| 67.2 | I | 51.964 | 48.036 | - | - |
| 53.3 | I | 50.525 | 49.475 | - | - |

Figure (2.9.34)-sample # 19

Figure (2.9.35)-sample # 19

Figure (2.9.36)-sample # 19

Figure (2.9.37)-Sample # 19

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 20 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 1.042 | Threshold Pressure, (psia) | : 269.757 |
| Porosity, (%) | : 16.62 | Dry Weight, (gm) | : 13.549 |
| Air Perm., (mD) | : 0.781 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8300.00 | Sample Shifted Depth, (ft.in) | : 8307.04 |

Table (2.9.10)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 269.757 | D | 0.000 | 100.000 | 0.000 | 0.3954 |
| 328.987 | D | 1.603 | 98.397 | 1.603 | 0.3242 |
| 417.422 | D | 21.770 | 78.230 | 20.1678 | 0.2555 |
| 519.039 | D | 58.177 | 41.823 | 36.4063 | 0.2055 |
| 636.941 | D | 72.083 | 27.917 | 13.9062 | 0.1675 |
| 795.999 | D | 78.044 | 21.956 | 5.9610 | 0.1340 |
| 988.620 | D | 81.286 | 18.714 | 3.2416 | 0.1079 |
| 1198.409 | D | 83.682 | 16.318 | 2.3963 | 0.0890 |
| 1495.54 | D | 85.742 | 14.258 | 2.0606 | 0.0713 |
| 1897.29 | D | 87.481 | 12.519 | 1.7387 | 0.0562 |
| 2345.91 | D | 88.933 | 11.067 | 1.4517 | 0.0455 |
| 2895.89 | D | 90.046 | 9.9538 | 1.1134 | 0.0368 |
| 3594.21 | D | 91.010 | 8.9895 | 0.9643 | 0.0297 |
| 4486.31 | D | 92.176 | 7.8238 | 1.1658 | 0.0238 |
| 5581.88 | D | 93.104 | 6.8964 | 0.9274 | 0.0191 |
| 6886.67 | D | 93.767 | 6.2333 | 0.6631 | 0.0155 |
| 8581.82 | D | 94.347 | 5.6529 | 0.5804 | 0.0124 |
| 10582.9 | D | 94.671 | 5.3291 | 0.3238 | 0.0101 |
| 13182.2 | D | 94.844 | 5.1556 | 0.1735 | 0.0081 |
| 14780.7 | D | 94.852 | 5.1478 | 0.0077 | 0.0072 |
| 16378.5 | D | 94.852 | 5.1478 | 0.0000 | 0.0065 |
| 19979.0 | D | 94.852 | 5.1478 | 0.0000 | 0.0053 |
| 24990.2 | D | 94.852 | 5.1478 | 0.0000 | 0.0043 |
| 29989.6 | D | 94.852 | 5.1478 | 0.0000 | 0.0036 |
| 34987.5 | D | 94.852 | 5.1478 | 0.0000 | 0.0030 |
| 39987.6 | D | 94.852 | 5.1478 | 0.0000 | 0.0027 |
| 44986.8 | D | 94.852 | 5.1478 | 0.0000 | 0.0024 |
| 49988.8 | D | 94.852 | 5.1478 | 0.0000 | 0.0021 |

Table (2.9.10-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 54990.7 | D | 94.852 | 5.1478 | 0.000 | 0.0019 |
| 59980.7 | D | 94.852 | 5.1478 | 0.000 | 0.0018 |
| 46106.8 | I | 94.852 | 5.1478 | - | - |
| 35508.8 | I | 94.852 | 5.1478 | - | - |
| 27301.1 | I | 94.852 | 5.1478 | - | - |
| 21003.4 | I | 94.852 | 5.1478 | - | - |
| 16009.7 | I | 94.852 | 5.1478 | - | - |
| 12411.8 | I | 94.852 | 5.1478 | - | - |
| 9612.3 | I | 94.852 | 5.1478 | - | - |
| 7317.0 | I | 94.852 | 5.1478 | - | - |
| 5711.3 | I | 94.852 | 5.1478 | - | - |
| 4305.1 | I | 94.852 | 5.1478 | - | - |
| 3299.2 | I | 94.852 | 5.1478 | - | - |
| 2608.8 | I | 94.852 | 5.1478 | - | - |
| 2004.0 | I | 94.852 | 5.1478 | - | - |
| 1505.0 | I | 94.852 | 5.1478 | - | - |
| 1204.3 | I | 94.852 | 5.1478 | - | - |
| 901.9 | I | 94.474 | 5.5258 | - | - |
| 701.9 | I | 93.347 | 6.6533 | - | - |
| 504.4 | I | 91.775 | 8.2247 | - | - |
| 402.2 | I | 90.251 | 9.749 | - | - |
| 301.7 | I | 87.631 | 12.369 | - | - |
| 242.4 | I | 84.683 | 15.317 | - | - |
| 191.2 | I | 81.397 | 18.603 | - | - |
| 147.7 | I | 76.146 | 23.854 | - | - |
| 112.5 | I | 70.917 | 29.083 | - | - |
| 85.5 | I | 68.019 | 31.981 | - | - |
| 68.3 | I | 67.132 | 32.868 | - | - |
| 52.8 | I | 66.654 | 33.346 | - | - |
| 34.0 | I | 66.215 | 33.785 | - | - |

Figure (2.9.38)-sample # 20

Figure (2.9.39)-sample # 20

Figure (2.9.40)-Sample # 20

Figure (2.9.41)-Sample # 20

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 24 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.875 | Threshold Pressure, (psia) | : 266.236 |
| Porosity, (%) | : 11.63 | Dry Weight, (gm) | : 18.021 |
| Air Perm., (mD) | : 0.471 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8566.00 | Sample Shifted Depth, (ft.in) | : 8573.04 |

Table (2.9.11)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 266.236 | D | 0.000 | 100.000 | 0.000 | 0.401 |
| 326.320 | D | 3.070 | 96.930 | 3.070 | 0.327 |
| 417.459 | D | 24.716 | 75.284 | 21.645 | 0.256 |
| 517.697 | D | 54.118 | 45.882 | 29.403 | 0.206 |
| 636.474 | D | 68.810 | 31.190 | 14.692 | 0.168 |
| 799.297 | D | 77.056 | 22.944 | 8.245 | 0.133 |
| 989.045 | D | 81.364 | 18.636 | 4.309 | 0.108 |
| 1199.79 | D | 84.345 | 15.655 | 2.981 | 0.089 |
| 1499.44 | D | 86.512 | 13.488 | 2.166 | 0.0711 |
| 1894.73 | D | 87.896 | 12.104 | 1.384 | 0.0563 |
| 2345.73 | D | 89.079 | 10.921 | 1.183 | 0.0455 |
| 2894.90 | D | 90.105 | 9.895 | 1.027 | 0.0368 |
| 3593.31 | D | 90.793 | 9.207 | 0.688 | 0.0297 |
| 4483.59 | D | 91.369 | 8.631 | 0.576 | 0.0238 |
| 5584.58 | D | 91.941 | 8.059 | 0.571 | 0.0191 |
| 6883.94 | D | 91.941 | 8.059 | 0.000 | 0.0155 |
| 8582.42 | D | 91.941 | 8.059 | 0.000 | 0.0124 |
| 10581.5 | D | 91.941 | 8.059 | 0.000 | 0.0101 |
| 13182.1 | D | 91.941 | 8.059 | 0.000 | 0.0081 |
| 14779.2 | D | 91.941 | 8.059 | 0.000 | 0.0072 |
| 16380.2 | D | 91.941 | 8.059 | 0.000 | 0.0065 |
| 19977.7 | D | 91.941 | 8.059 | 0.000 | 0.0053 |
| 24989.8 | D | 91.941 | 8.059 | 0.000 | 0.0043 |
| 29987.2 | D | 91.941 | 8.059 | 0.000 | 0.0036 |
| 34989.6 | D | 91.941 | 8.059 | 0.000 | 0.0030 |
| 39991.6 | D | 91.941 | 8.059 | 0.000 | 0.0027 |
| 44987.2 | D | 91.941 | 8.059 | 0.000 | 0.0024 |
| 49986.0 | D | 91.941 | 8.059 | 0.000 | 0.0021 |

Table (2.9.11-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 54994.6 | D | 91.941 | 8.059 | 0.000 | 0.0019 |
| 59976.1 | D | 91.941 | 8.059 | 0.000 | 0.0018 |
| 46103.9 | I | 91.941 | 8.059 | - | - |
| 35504.2 | I | 91.941 | 8.059 | - | - |
| 27309.4 | I | 91.941 | 8.059 | - | - |
| 21009.2 | I | 91.941 | 8.059 | - | - |
| 16005.0 | I | 91.941 | 8.059 | - | - |
| 12410.0 | I | 91.941 | 8.059 | - | - |
| 9614.8 | I | 91.941 | 8.059 | - | - |
| 7311.7 | I | 91.941 | 8.059 | - | - |
| 5716.2 | I | 91.941 | 8.059 | - | - |
| 4172.4 | I | 91.941 | 8.059 | - | - |
| 3306.9 | I | 91.941 | 8.059 | - | - |
| 2601.4 | I | 91.941 | 8.059 | - | - |
| 2005.3 | I | 91.941 | 8.059 | - | - |
| 1504.6 | I | 91.941 | 8.059 | - | - |
| 1200.9 | I | 91.941 | 8.059 | - | - |
| 900.5 | I | 91.941 | 8.059 | - | - |
| 702.2 | I | 91.076 | 8.924 | - | - |
| 503.0 | I | 88.046 | 11.954 | - | - |
| 402.3 | I | 85.151 | 14.849 | - | - |
| 301.6 | I | 80.076 | 19.924 | - | - |
| 242.6 | I | 74.771 | 25.229 | - | - |
| 193.6 | I | 68.433 | 31.567 | - | - |
| 148.6 | I | 61.331 | 38.669 | - | - |
| 113.8 | I | 58.516 | 41.484 | - | - |
| 87.5 | I | 57.608 | 42.392 | - | - |
| 67.8 | I | 57.192 | 42.808 | - | - |
| 53.1 | I | 56.980 | 43.020 | - | - |
| 32.8 | I | 56.829 | 43.171 | - | - |
| 18.2 | I | 56.609 | 43.391 | - | - |

Figure (2.9.42)-sample # 24

Figure (2.9.43)-sample # 24

Figure (2.9.44)-Sample # 24

Figure (2.9.45)-Sample # 24

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 29 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.351 | Threshold Pressure, (psia) | : 220.164 |
| Porosity, (%) | : 17.11 | Dry Weight, (gm) | : 4.362 |
| Air Perm., (mD) | : 0.726 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8840.00 | Sample Shifted Depth, (ft.in) | : 8847.04 |

Table (2.9.12)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 220.164 | D | 0.000 | 100.000 | 0.000 | 0.484 |
| 265.870 | D | 2.520 | 97.480 | 2.520 | 0.401 |
| 329.074 | D | 22.049 | 77.951 | 19.529 | 0.324 |
| 418.489 | D | 53.117 | 46.883 | 31.067 | 0.255 |
| 517.340 | D | 63.932 | 36.068 | 10.815 | 0.206 |
| 636.505 | D | 71.179 | 28.821 | 7.247 | 0.168 |
| 798.798 | D | 76.621 | 23.379 | 5.442 | 0.134 |
| 988.537 | D | 80.811 | 19.189 | 4.190 | 0.108 |
| 1200.08 | D | 83.534 | 16.466 | 2.723 | 0.089 |
| 1498.92 | D | 86.046 | 13.954 | 2.512 | 0.0712 |
| 1898.06 | D | 88.387 | 11.613 | 2.342 | 0.0562 |
| 2345.33 | D | 90.028 | 9.972 | 1.640 | 0.0455 |
| 2897.38 | D | 91.344 | 8.656 | 1.316 | 0.0368 |
| 3592.57 | D | 92.435 | 7.565 | 1.091 | 0.0297 |
| 4486.45 | D | 93.583 | 6.417 | 1.148 | 0.0238 |
| 5582.75 | D | 94.032 | 5.968 | 0.449 | 0.0191 |
| 6881.49 | D | 94.476 | 5.524 | 0.444 | 0.0155 |
| 8584.03 | D | 94.799 | 5.2005 | 0.324 | 0.0124 |
| 10584.2 | D | 94.836 | 5.1643 | 0.036 | 0.0101 |
| 13182.9 | D | 95.028 | 4.9723 | 0.192 | 0.0081 |
| 14778.8 | D | 95.028 | 4.9723 | 0.000 | 0.0072 |
| 16380.2 | D | 95.028 | 4.9723 | 0.000 | 0.0065 |
| 19982.3 | D | 95.028 | 4.9723 | 0.000 | 0.0053 |
| 24992.8 | D | 95.028 | 4.9723 | 0.000 | 0.0043 |
| 29991.3 | D | 95.028 | 4.9723 | 0.000 | 0.0036 |
| 34989.9 | D | 95.028 | 4.9723 | 0.000 | 0.0030 |
| 39989.5 | D | 95.028 | 4.9723 | 0.000 | 0.0027 |
| 44991.9 | D | 95.028 | 4.9723 | 0.000 | 0.0024 |

Table (2.9.12-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 49989.6 | D | 95.028 | 4.9723 | 0.000 | 0.0021 |
| 54984.0 | D | 95.028 | 4.9723 | 0.000 | 0.0019 |
| 59985.9 | D | 95.028 | 4.9723 | 0.000 | 0.0018 |
| 46106.7 | I | 95.028 | 4.9723 | - | - |
| 35504.5 | I | 95.028 | 4.9723 | - | - |
| 27304.5 | I | 95.028 | 4.9723 | - | - |
| 21010.0 | I | 95.028 | 4.9723 | - | - |
| 16006.7 | I | 95.028 | 4.9723 | - | - |
| 12406.5 | I | 95.028 | 4.9723 | - | - |
| 9613.5 | I | 95.028 | 4.9723 | - | - |
| 7306.8 | I | 95.028 | 4.9723 | - | - |
| 5711.9 | I | 95.028 | 4.9723 | - | - |
| 4310.0 | I | 95.028 | 4.9723 | - | - |
| 3300.8 | I | 95.028 | 4.9723 | - | - |
| 2604.5 | I | 95.028 | 4.9723 | - | - |
| 2004.2 | I | 95.028 | 4.9723 | - | - |
| 1504.4 | I | 95.028 | 4.972 | - | - |
| 1201.0 | I | 95.028 | 4.972 | - | - |
| 903.2 | I | 95.028 | 4.972 | - | - |
| 699.7 | I | 94.588 | 5.412 | - | - |
| 503.3 | I | 92.146 | 7.854 | - | - |
| 400.6 | I | 88.179 | 11.821 | - | - |
| 303.8 | I | 82.936 | 17.064 | - | - |
| 240.7 | I | 76.550 | 23.450 | - | - |
| 192.5 | I | 73.410 | 26.590 | - | - |
| 146.7 | I | 72.416 | 27.584 | - | - |
| 112.8 | I | 71.862 | 28.138 | - | - |
| 85.8 | I | 71.583 | 28.417 | - | - |
| 68.3 | I | 71.419 | 28.581 | - | - |
| 52.9 | I | 71.317 | 28.683 | - | - |
| 32.7 | I | 71.177 | 28.823 | - | - |

Figure (2.9.46)-sample # 29

Figure (2.9.47)-sample # 29

Figure (2.9.48)-Sample # 29

Figure (2.9.49)-Sample # 29

MERCURY INJECTION CAPILLARY PRESSURE AND PORE SIZE DISTRIBUTION

Sidewall Plugs

| Client | : Waha Oil Company |
| --- | --- |
| Well | : T1-31 |
| Sample # | : 30 |

| Core Sample Data and Experimental Values |  |  |  |
| --- | --- | --- | --- |
| Hg Pore volume, (cc) | : 0.778 | Threshold Pressure, (psia) | : 326.363 |
| Porosity, (%) | : 14.50 | Dry Weight, (gm) | : 11.782 |
| Air Perm., (mD) | : 0.453 | Formation | : Gir |
| Sample Depth, (ft.in) | : 8902.00 | Sample Shifted Depth, (ft.in) | : 8909.04 |

Table (2.9.13)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 326.363 | D | 0.000 | 100.000 | 0.0000 | 0.3268 |
| 416.860 | D | 7.915 | 92.085 | 7.9151 | 0.2559 |
| 516.867 | D | 28.399 | 71.601 | 20.4836 | 0.2064 |
| 637.847 | D | 53.791 | 46.209 | 25.3927 | 0.1672 |
| 799.277 | D | 66.893 | 33.107 | 13.1016 | 0.1334 |
| 990.181 | D | 71.978 | 28.022 | 5.0846 | 0.1077 |
| 1201.34 | D | 75.532 | 24.468 | 3.5548 | 0.0888 |
| 1497.98 | D | 78.510 | 21.490 | 2.9775 | 0.0712 |
| 1897.30 | D | 81.166 | 18.8343 | 2.6558 | 0.0562 |
| 2345.68 | D | 82.985 | 17.0147 | 1.8197 | 0.0455 |
| 2895.86 | D | 84.475 | 15.5249 | 1.4898 | 0.0368 |
| 3594.84 | D | 85.833 | 14.1667 | 1.3582 | 0.0297 |
| 4482.05 | D | 86.867 | 13.1331 | 1.0336 | 0.0238 |
| 5580.29 | D | 87.766 | 12.2337 | 0.8994 | 0.0191 |
| 6881.99 | D | 88.471 | 11.5286 | 0.7051 | 0.0155 |
| 8583.96 | D | 89.151 | 10.8493 | 0.6793 | 0.0124 |
| 10585.4 | D | 89.457 | 10.5434 | 0.3059 | 0.0101 |
| 13181.6 | D | 89.686 | 10.3143 | 0.2291 | 0.0081 |
| 14782.9 | D | 89.686 | 10.3143 | 0.0000 | 0.0072 |
| 16382.5 | D | 89.686 | 10.3143 | 0.0000 | 0.0065 |
| 19980.0 | D | 89.686 | 10.3143 | 0.0000 | 0.0053 |
| 24991.2 | D | 89.686 | 10.3143 | 0.0000 | 0.0043 |
| 29990.8 | D | 89.686 | 10.3143 | 0.0000 | 0.0036 |
| 34987.5 | D | 89.686 | 10.3143 | 0.0000 | 0.0030 |
| 39986.8 | D | 89.686 | 10.3143 | 0.0000 | 0.0027 |
| 44988.1 | D | 89.686 | 10.3143 | 0.0000 | 0.0024 |
| 49986.5 | D | 89.686 | 10.3143 | 0.0000 | 0.0021 |
| 54988.3 | D | 89.686 | 10.3143 | 0.0000 | 0.0019 |

Table (2.9.13-Continue)

| Pressure (psia) | Cycle | Cum. Hg Sat. (Pore Vol., %) | Wetting Phase (Sw) (Pore Vol., %) | Incr. Hg Sat. (Pore Vol., %) | Ri (microns) |
| --- | --- | --- | --- | --- | --- |
| 59980.9 | D | 89.686 | 10.3143 | 0.000 | 0.0018 |
| 46108.4 | I | 89.686 | 10.3143 | - | - |
| 35499.2 | I | 89.686 | 10.3143 | - | - |
| 27311.5 | I | 89.686 | 10.3143 | - | - |
| 21011.4 | I | 89.686 | 10.3143 | - | - |
| 16005.6 | I | 89.686 | 10.3143 | - | - |
| 12413.0 | I | 89.686 | 10.3143 | - | - |
| 9613.1 | I | 89.686 | 10.3143 | - | - |
| 7318.8 | I | 89.686 | 10.3143 | - | - |
| 5712.5 | I | 89.686 | 10.3143 | - | - |
| 4311.4 | I | 89.686 | 10.3143 | - | - |
| 3298.8 | I | 89.686 | 10.3143 | - | - |
| 2606.6 | I | 89.686 | 10.3143 | - | - |
| 2000.4 | I | 89.686 | 10.3143 | - | - |
| 1498.7 | I | 89.686 | 10.3143 | - | - |
| 1203.3 | I | 89.686 | 10.3143 | - | - |
| 900.8 | I | 88.676 | 11.3235 | - | - |
| 701.5 | I | 87.613 | 12.3871 | - | - |
| 500.6 | I | 85.606 | 14.3943 | - | - |
| 401.9 | I | 83.816 | 16.184 | - | - |
| 302.1 | I | 80.607 | 19.393 | - | - |
| 243.1 | I | 77.506 | 22.494 | - | - |
| 193.0 | I | 73.237 | 26.763 | - | - |
| 147.1 | I | 67.637 | 32.363 | - | - |
| 112.5 | I | 63.576 | 36.424 | - | - |
| 87.4 | I | 61.441 | 38.559 | - | - |
| 68.1 | I | 60.127 | 39.873 | - | - |

Figure (2.9.50)-sample # 30

Figure (2.9.51)-sample # 30

Figure (2.9.52)-Sample # 30

Figure (2.9.53)-Sample # 30

Composite plot

Sidewall Plugs

Figure (2.9.54)

Section Ten

Unsteady State Water-Oil Relative Permeability

(At Reservoir Conditions)

LABORATORY PROCEDURES

- Unsteady-State Water-Oil Relative Permeability for 1.5” Plugs

- Two of one and half inch diameter plugs were selected by Waha Oil Company from the well T1-31, Gir formation to perform unsteady-state water-oil relative permeability test at reservoir conditions by using X-Ray system.

- Samples underwent solvent extraction of hydrocarbons to ensure they were reverted to a water-wet wettability preference, then leached of salts with methanol and dried to a stable weight. Basic properties including grain density, pore volume, and permeability to air were measured at reservoir net confining stress.

3. Simulated formation brine containing (60,000 ppm) sodium chloride was used as the synthetic formation brine based on information provided by W.O.C. The brine was mixed and filtered to 0.45 microns and degassed. Fluid parameters including viscosity and density were measured at ambient and elevated temperature.

4. The samples were evacuated of air and pressure saturated with synthetic formation brine.

5. The brine saturated samples were then loaded into a centrifuge in an air- displacing-brine configuration and spun at a capillary pressure equivalent to 120 psi to establish initial water saturation. The samples were unloaded from the centrifuge and weighed.

6. Teflon tape was used to rap and stabilize each sample.

7. Each sample was loaded into an X-ray core holder, and the appropriate net confining stress (4839 psi) was applied. The sample was scanned prior to connecting injection lines. Injection lines were attached, and a pore pressure was established by injecting tagged laboratory oil through the system and through the sample against a backpressure until differential pressure stabilized.

8. Tagged laboratory oil, filtered to 0.45 microns, and tagged with 12.5% by weight   1-Iododecane, was injected through the sample at a constant rate. Differential pressure was monitored until equilibrium was achieved. Effective permeability to oil at initial water saturation was determined at two injection rates while X-ray scanning the sample. The measurements were conducted at ambient and reservoir temperature (213 OF).

9.   Synthetic formation brine was injected at a constant rate. X-ray scans of the sample were taken, and injection was continued until a 99.95% water-cut was achieved. Differential pressure was recorded as a function of elapsed time throughout the test process. Finally, effective permeability to water at residual oil saturation was determined at two injection rates while scanning the sample.

10. Pore pressure and overburden pressure were slowly released. The sample was unloaded, weighed, and submitted to Dean-Stark and soxhlet extraction with toluene as a check on residual fluid volumes, leached of salts with methanol, and dried to a constant weight in a convection oven.

11.  Each sample was extracted of any final fluid, leached of salts, and dried in a convection oven until the weight stabilized.

12. Unsteady-State water-oil relative permeability relationships at reservoir conditions were calculated from collected data, sample and fluid parameters by using Sendra software, which is a two-phase 1D black-oil simulation model. It is used for analyzing single SCAL experiments as well as several SCAL experiments simultaneously (Reference 9).

The analysis can be performed by either simulation or estimation. Simulation is forward simulations of a multi-phase flow experiment, while Estimation utilizes an optimization routine to history match the experimental data obtained in the laboratory. (Sendra Software User Guide version 2018) uses different types of correlations to history match the experimental data, the type of correlation to be used may vary with which type of experiments that is analyzed as well as which type of experimental data that is used throughout the estimation. The available correlations are as follows: Corey, LET, Chierici, Burdine and Sigmund – McCaffery.

The results of water-oil relative permeability for 1.5 inches plug samples are presented in tables 2.10.1 through 2.10.2 and in figures 2.10.1 through 2.10.6.

- Unsteady-State Water-Oil Relative Permeability for Side Wall Plugs

Five sidewall samples were selected perform unsteady-state water-oil relative permeability test at reservoir conditions. The samples were fully saturated by brine with salinity of (60,000 ppm).  The prepared sample was individually placed in a hydrostatic core holder, an overburden pressure of (5182 psig) and reservoir temperature of (213 OF) were applied.

laboratory oil, filtered to 0.45 microns was prepared in the laboratory to match the viscosity of reservoir oil. This oil was injected into the top of the sample and the water production at the effluent of the sample was monitored. When the effluent was brine free and the flow was constant, the oil permeability was measured, and the irreducible water saturation was calculated both gravimetric and volumetric and cross-referenced.

On completion of oil permeability measurements, simulated formation brine was injected at the top end face of the sample at constant flow rate using HPLC pump. The produced volumes, differential pressure, time and confining pressure were all monitored and recorded. The brine flood was only terminated when the water cut reached 99.98%. When oil production was ceased, permeability to water was measured and the test was terminated. The time, oil volume and the total volume were used to calculate relative permeability using Sendra software which is a two-phase 1D black-oil simulation model developed by Weatherford Petroleum Consultants AS. It is used for analyzing single SCAL experiments as well as several SCAL experiments simultaneously. It is tailor made for revealing relative permeability and capillary pressure from two-phase and multi-phase flow experiments performed in the SCAL laboratory. The analysis covers oil-water, oil-gas as well as water-gas experiments and both imbibition and drainage is handled. A third stagnant phase can easily be included. Furthermore, Sendra contains a powerful graphical tool used to plot all results and compare a variety of flow properties. These results can be further analyzed and recommended for field application. The analysis can be performed by either simulation or estimation. Simulation is forward simulations of a multi-phase flow experiment, while Estimation utilizes an optimization routine to history match the experimental data obtained in the laboratory.

The results of water-oil relative permeability for sidewall plugs are presented in tables 2.10.3 through 2.10.7 and in figures 2.10.7 through 2.10.21.

WATER-OIL RELATIVE PERMEABILITY

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.244 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.120 |
| Sample # | : 1 | Perm. To Water at resid. Oil Saturation, mD | : 0.072 |
| Porosity, (%) | : 16.86 | Residual Oil Saturation (Sorw), fraction | : 0.195 |
| Permeability,(mD) | : 0.640 | Net Confining Pressure, psi | : 4839 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 7685.02 | Shifted Depth, ft.in | : 7692.06 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.244 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.255 | 2.124E-09 | 0.930375 | 2.283E-09 |
| 0.267 | 6.797E-08 | 0.864284 | 7.864E-08 |
| 0.278 | 5.162E-07 | 0.801616 | 6.439E-07 |
| 0.290 | 2.175E-06 | 0.742262 | 2.930E-06 |
| 0.301 | 6.638E-06 | 0.686117 | 9.674E-06 |
| 0.313 | 1.652E-05 | 0.633072 | 2.609E-05 |
| 0.324 | 0.000036 | 0.583024 | 6.123E-05 |
| 0.336 | 0.000070 | 0.535867 | 1.299E-04 |
| 0.347 | 0.000125 | 0.491500 | 2.552E-04 |
| 0.358 | 0.000212 | 0.449821 | 4.722E-04 |
| 0.370 | 0.000342 | 0.410730 | 8.329E-04 |
| 0.381 | 0.000529 | 0.374128 | 1.413E-03 |
| 0.393 | 0.000789 | 0.339917 | 2.320E-03 |
| 0.404 | 0.001142 | 0.308001 | 3.709E-03 |
| 0.416 | 0.001613 | 0.278285 | 5.796E-03 |
| 0.427 | 0.002227 | 0.250676 | 8.885E-03 |
| 0.439 | 0.003016 | 0.225081 | 1.340E-02 |
| 0.450 | 0.004014 | 0.201409 | 1.993E-02 |
| 0.462 | 0.005259 | 0.179572 | 2.929E-02 |
| 0.473 | 0.006797 | 0.159480 | 4.262E-02 |
| 0.484 | 0.008675 | 0.141048 | 6.150E-02 |
| 0.496 | 0.010947 | 0.124190 | 8.814E-02 |
| 0.507 | 0.013671 | 0.108823 | 1.256E-01 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.519 | 0.016913 | 0.094865 | 1.783E-01 |
| 0.530 | 0.020743 | 0.082234 | 2.522E-01 |
| 0.542 | 0.025237 | 0.070854 | 3.562E-01 |
| 0.553 | 0.030478 | 0.060645 | 5.026E-01 |
| 0.565 | 0.036556 | 0.051533 | 7.094E-01 |
| 0.576 | 0.043567 | 0.043443 | 1.003E+00 |
| 0.587 | 0.051615 | 0.036304 | 1.422E+00 |
| 0.599 | 0.060811 | 0.030045 | 2.024E+00 |
| 0.610 | 0.071272 | 0.024597 | 2.898E+00 |
| 0.622 | 0.083127 | 0.019895 | 4.178E+00 |
| 0.633 | 0.096509 | 0.015872 | 6.080E+00 |
| 0.645 | 0.111561 | 0.012467 | 8.949E+00 |
| 0.656 | 0.128435 | 0.009619 | 1.335E+01 |
| 0.668 | 0.147292 | 0.007269 | 2.026E+01 |
| 0.679 | 0.168302 | 0.005360 | 3.140E+01 |
| 0.691 | 0.191643 | 0.003840 | 4.991E+01 |
| 0.702 | 0.217506 | 0.002656 | 8.190E+01 |
| 0.713 | 0.246088 | 0.001758 | 1.399E+02 |
| 0.725 | 0.277599 | 0.001102 | 2.519E+02 |
| 0.736 | 0.312258 | 0.000642 | 4.860E+02 |
| 0.748 | 0.350295 | 0.000339 | 1.032E+03 |
| 0.759 | 0.391952 | 0.000155 | 2.522E+03 |
| 0.771 | 0.437482 | 0.000057 | 7.704E+03 |
| 0.782 | 0.487147 | 0.000014 | 3.546E+04 |
| 0.794 | 0.541224 | 0.000001 | 4.457E+05 |
| 0.805 | 0.600000 | 0.000000 | ************ |

Table (2.10.1)

Figure (2.10.1) Cross-plot (Linear scale) of water oil relative permeability for sample # 1

Figure (2.10.2) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 1

Figure (2.10.3) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 1

WATER-OIL RELATIVE PERMEABILITY

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.241 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.107 |
| Sample # | : 2 | Perm. To Water at resid. Oil Saturation, mD | : 0.070 |
| Porosity, (%) | : 16.09 | Residual Oil Saturation (Sorw), fraction | : 0.129 |
| Permeability,(mD) | : 0.561 | Net Confining Pressure, psi | : 4839 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 7686.05 | Shifted Depth, ft.in | : 7693.09 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.241 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.254 | 2.081E-10 | 0.947837 | 2.196E-10 |
| 0.267 | 1.022E-08 | 0.897382 | 1.139E-08 |
| 0.280 | 9.966E-08 | 0.848614 | 1.174E-07 |
| 0.292 | 5.016E-07 | 0.801511 | 6.258E-07 |
| 0.305 | 1.757E-06 | 0.756052 | 2.324E-06 |
| 0.318 | 4.893E-06 | 0.712214 | 6.870E-06 |
| 0.331 | 1.163E-05 | 0.669976 | 1.736E-05 |
| 0.344 | 2.463E-05 | 0.629315 | 3.913E-05 |
| 0.357 | 4.772E-05 | 0.590208 | 8.086E-05 |
| 0.370 | 8.625E-05 | 0.552634 | 1.561E-04 |
| 0.382 | 1.473E-04 | 0.516567 | 2.852E-04 |
| 0.395 | 2.402E-04 | 0.481987 | 4.983E-04 |
| 0.408 | 3.766E-04 | 0.448868 | 8.389E-04 |
| 0.421 | 5.710E-04 | 0.417187 | 1.369E-03 |
| 0.434 | 8.413E-04 | 0.386921 | 2.174E-03 |
| 0.447 | 1.209E-03 | 0.358044 | 3.377E-03 |
| 0.460 | 1.699E-03 | 0.330533 | 5.142E-03 |
| 0.472 | 2.343E-03 | 0.304362 | 7.698E-03 |
| 0.485 | 3.174E-03 | 0.279506 | 1.136E-02 |
| 0.498 | 4.234E-03 | 0.255939 | 1.654E-02 |
| 0.511 | 5.570E-03 | 0.233636 | 2.384E-02 |
| 0.524 | 7.233E-03 | 0.212571 | 3.403E-02 |
| 0.537 | 9.285E-03 | 0.192716 | 4.818E-02 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.550 | 0.011792 | 0.174045 | 6.775E-02 |
| 0.562 | 0.014831 | 0.156531 | 9.475E-02 |
| 0.575 | 0.018487 | 0.140144 | 1.319E-01 |
| 0.588 | 0.022853 | 0.124858 | 1.830E-01 |
| 0.601 | 0.028032 | 0.110643 | 2.534E-01 |
| 0.614 | 0.034140 | 0.097470 | 3.503E-01 |
| 0.627 | 0.041302 | 0.085309 | 4.842E-01 |
| 0.640 | 0.049656 | 0.074128 | 6.699E-01 |
| 0.652 | 0.059351 | 0.063898 | 9.288E-01 |
| 0.665 | 0.070550 | 0.054586 | 1.292E+00 |
| 0.678 | 0.083431 | 0.046159 | 1.807E+00 |
| 0.691 | 0.098185 | 0.038584 | 2.545E+00 |
| 0.704 | 0.115020 | 0.031826 | 3.614E+00 |
| 0.717 | 0.134158 | 0.025850 | 5.190E+00 |
| 0.730 | 0.155839 | 0.020620 | 7.558E+00 |
| 0.742 | 0.180321 | 0.016097 | 1.120E+01 |
| 0.755 | 0.207880 | 0.012242 | 1.698E+01 |
| 0.768 | 0.238811 | 0.009015 | 2.649E+01 |
| 0.781 | 0.273428 | 0.006372 | 4.291E+01 |
| 0.794 | 0.312068 | 0.004269 | 7.310E+01 |
| 0.807 | 0.355087 | 0.002658 | 1.336E+02 |
| 0.820 | 0.402865 | 0.001489 | 2.706E+02 |
| 0.832 | 0.455806 | 0.000705 | 6.465E+02 |
| 0.845 | 0.514337 | 0.000246 | 2.092E+03 |
| 0.858 | 0.578909 | 0.000041 | 1.426E+04 |
| 0.871 | 0.650000 | 0.000000 | ************ |

Table (2.10.2)

Figure (2.10.4) Cross-plot (Linear scale) of water oil relative permeability for sample # 2

Figure (2.10.5) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 2

Figure (2.10.6) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 2

WATER-OIL RELATIVE PERMEABILITY

Sidewall Plugs

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.207 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.344 |
| Sample # | : 9 | Perm. To Water at resid. Oil Saturation, mD | : 0.237 |
| Porosity, (%) | : 11.84 | Residual Oil Saturation (Sorw), fraction | : 0.230 |
| Permeability,(mD) | : 0.880 | Net Confining Pressure, psi | : 5182 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 7837.00 | Shifted Depth, ft.in | : 7844.04 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.207 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.218 | 2.241E-08 | 0.935358 | 2.396E-08 |
| 0.230 | 4.830E-07 | 0.873665 | 5.528E-07 |
| 0.241 | 2.911E-06 | 0.814845 | 3.572E-06 |
| 0.253 | 1.041E-05 | 0.758821 | 1.372E-05 |
| 0.264 | 2.798E-05 | 0.705518 | 3.965E-05 |
| 0.276 | 6.274E-05 | 0.654863 | 9.581E-05 |
| 0.287 | 0.000124 | 0.606779 | 2.047E-04 |
| 0.299 | 0.000224 | 0.561194 | 3.999E-04 |
| 0.310 | 0.000378 | 0.518034 | 7.299E-04 |
| 0.322 | 0.000603 | 0.477225 | 1.264E-03 |
| 0.333 | 0.000920 | 0.438694 | 2.097E-03 |
| 0.345 | 0.001352 | 0.402370 | 3.361E-03 |
| 0.356 | 0.001928 | 0.368181 | 5.236E-03 |
| 0.368 | 0.002677 | 0.336055 | 7.966E-03 |
| 0.379 | 0.003634 | 0.305922 | 1.188E-02 |
| 0.391 | 0.004837 | 0.277710 | 1.742E-02 |
| 0.402 | 0.006327 | 0.251351 | 2.517E-02 |
| 0.414 | 0.008150 | 0.226774 | 3.594E-02 |
| 0.425 | 0.010355 | 0.203911 | 5.078E-02 |
| 0.437 | 0.012997 | 0.182694 | 7.114E-02 |
| 0.448 | 0.016133 | 0.163055 | 9.894E-02 |
| 0.460 | 0.019825 | 0.144926 | 1.368E-01 |
| 0.471 | 0.024140 | 0.128240 | 1.882E-01 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.483 | 0.029149 | 0.112933 | 2.581E-01 |
| 0.494 | 0.034926 | 0.098938 | 3.530E-01 |
| 0.506 | 0.041554 | 0.086191 | 4.821E-01 |
| 0.517 | 0.049116 | 0.074627 | 6.581E-01 |
| 0.529 | 0.057701 | 0.064183 | 8.990E-01 |
| 0.540 | 0.067406 | 0.054795 | 1.230E+00 |
| 0.552 | 0.078329 | 0.046403 | 1.688E+00 |
| 0.563 | 0.090574 | 0.038945 | 2.326E+00 |
| 0.575 | 0.104252 | 0.032359 | 3.222E+00 |
| 0.586 | 0.119478 | 0.026587 | 4.494E+00 |
| 0.598 | 0.136370 | 0.021569 | 6.323E+00 |
| 0.609 | 0.155056 | 0.017247 | 8.990E+00 |
| 0.621 | 0.175665 | 0.013565 | 1.295E+01 |
| 0.632 | 0.198335 | 0.010465 | 1.895E+01 |
| 0.644 | 0.223206 | 0.007894 | 2.828E+01 |
| 0.655 | 0.250426 | 0.005796 | 4.321E+01 |
| 0.667 | 0.280148 | 0.004119 | 6.801E+01 |
| 0.678 | 0.312532 | 0.002812 | 1.111E+02 |
| 0.690 | 0.347740 | 0.001824 | 1.906E+02 |
| 0.701 | 0.385945 | 0.001107 | 3.486E+02 |
| 0.713 | 0.427321 | 0.000613 | 6.970E+02 |
| 0.724 | 0.472052 | 0.000297 | 1.587E+03 |
| 0.736 | 0.520325 | 0.000117 | 4.444E+03 |
| 0.747 | 0.572335 | 0.000031 | 1.819E+04 |
| 0.759 | 0.628282 | 0.000003 | 1.888E+05 |
| 0.770 | 0.688373 | 0.000000 | ************ |

Table (2.10.3)

Figure (2.11.7) Cross-plot (Linear scale) of water oil relative permeability for sample # 9

Figure (2.10.8) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 9

Figure (2.10.9) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 9

WATER-OIL RELATIVE PERMEABILITY

Sidewall Plugs

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.180 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.273 |
| Sample # | : 16 | Perm. To Water at resid. Oil Saturation, mD | : 0.162 |
| Porosity, (%) | : 19.76 | Residual Oil Saturation (Sorw), fraction | : 0.150 |
| Permeability,(mD) | : 0.997 | Net Confining Pressure, psi | : 5182 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 8150.00 | Shifted Depth, ft.in | : 8157.04 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.180 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.194 | 9.137E-10 | 0.930967 | 9.814E-10 |
| 0.207 | 3.392E-08 | 0.865396 | 3.920E-08 |
| 0.221 | 2.810E-07 | 0.803180 | 3.498E-07 |
| 0.235 | 1.259E-06 | 0.744215 | 1.692E-06 |
| 0.248 | 4.031E-06 | 0.688399 | 5.856E-06 |
| 0.262 | 1.043E-05 | 0.635629 | 1.641E-05 |
| 0.276 | 2.330E-05 | 0.585804 | 3.978E-05 |
| 0.289 | 4.675E-05 | 0.538823 | 8.677E-05 |
| 0.303 | 8.641E-05 | 0.494588 | 1.747E-04 |
| 0.317 | 1.497E-04 | 0.453000 | 3.304E-04 |
| 0.330 | 2.460E-04 | 0.413965 | 5.943E-04 |
| 0.344 | 3.873E-04 | 0.377384 | 1.026E-03 |
| 0.358 | 5.879E-04 | 0.343166 | 1.713E-03 |
| 0.371 | 8.652E-04 | 0.311215 | 2.780E-03 |
| 0.385 | 1.240E-03 | 0.281441 | 4.405E-03 |
| 0.399 | 1.736E-03 | 0.253752 | 6.840E-03 |
| 0.412 | 2.381E-03 | 0.228060 | 1.044E-02 |
| 0.426 | 3.208E-03 | 0.204275 | 1.570E-02 |
| 0.440 | 4.253E-03 | 0.182311 | 2.333E-02 |
| 0.453 | 5.557E-03 | 0.162082 | 3.428E-02 |
| 0.467 | 7.167E-03 | 0.143505 | 4.994E-02 |
| 0.481 | 9.134E-03 | 0.126495 | 7.221E-02 |
| 0.494 | 1.152E-02 | 0.110972 | 1.038E-01 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.508 | 0.014378 | 0.096855 | 1.484E-01 |
| 0.522 | 0.017789 | 0.084066 | 2.116E-01 |
| 0.536 | 0.021825 | 0.072527 | 3.009E-01 |
| 0.549 | 0.026572 | 0.062162 | 4.275E-01 |
| 0.563 | 0.032121 | 0.052898 | 6.072E-01 |
| 0.577 | 0.038570 | 0.044661 | 8.636E-01 |
| 0.590 | 0.046028 | 0.037381 | 1.231E+00 |
| 0.604 | 0.054611 | 0.030988 | 1.762E+00 |
| 0.618 | 0.064443 | 0.025414 | 2.536E+00 |
| 0.631 | 0.075659 | 0.020594 | 3.674E+00 |
| 0.645 | 0.088402 | 0.016463 | 5.370E+00 |
| 0.659 | 0.102827 | 0.012958 | 7.935E+00 |
| 0.672 | 0.119097 | 0.010021 | 1.189E+01 |
| 0.686 | 0.137388 | 0.007591 | 1.810E+01 |
| 0.700 | 0.157885 | 0.005613 | 2.813E+01 |
| 0.713 | 0.180786 | 0.004033 | 4.483E+01 |
| 0.727 | 0.206299 | 0.002798 | 7.373E+01 |
| 0.741 | 0.234647 | 0.001860 | 1.262E+02 |
| 0.754 | 0.266064 | 0.001170 | 2.274E+02 |
| 0.768 | 0.300796 | 0.000685 | 4.388E+02 |
| 0.782 | 0.339105 | 0.000364 | 9.312E+02 |
| 0.795 | 0.381263 | 0.000168 | 2.271E+03 |
| 0.809 | 0.427561 | 0.000062 | 6.908E+03 |
| 0.823 | 0.478300 | 0.000015 | 3.154E+04 |
| 0.836 | 0.533798 | 0.000001 | 3.899E+05 |
| 0.850 | 0.594389 | 0.000000 | ************ |

Table (2.10.4)

Figure (2.10.10) Cross-plot (Linear scale) of water oil relative permeability for sample # 16

Figure (2.10.11) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 16

Figure (2.10.12) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 32

WATER-OIL RELATIVE PERMEABILITY

Sidewall Plugs

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.304 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.151 |
| Sample # | : 20 | Perm. To Water at resid. Oil Saturation, mD | : 0.079 |
| Porosity, (%) | : 16.62 | Residual Oil Saturation (Sorw), fraction | : 0.260 |
| Permeability,(mD) | : 0.781 | Net Confining Pressure, psi | : 5182 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 8300.00 | Shifted Depth, ft.in | : 8307.04 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.304 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.313 | 1.856E-09 | 0.930375 | 1.995E-09 |
| 0.322 | 5.940E-08 | 0.864284 | 6.873E-08 |
| 0.331 | 4.511E-07 | 0.801616 | 5.627E-07 |
| 0.340 | 1.901E-06 | 0.742262 | 2.561E-06 |
| 0.348 | 0.000006 | 0.686117 | 0.000008 |
| 0.357 | 0.000014 | 0.633072 | 0.000023 |
| 0.366 | 0.000031 | 0.583024 | 0.000054 |
| 0.375 | 0.000061 | 0.535867 | 0.000114 |
| 0.384 | 0.000110 | 0.491500 | 0.000223 |
| 0.393 | 0.000186 | 0.449821 | 0.000413 |
| 0.402 | 0.000299 | 0.410730 | 0.000728 |
| 0.411 | 0.000462 | 0.374128 | 0.001235 |
| 0.420 | 0.000689 | 0.339917 | 0.002028 |
| 0.429 | 0.000998 | 0.308001 | 0.003242 |
| 0.437 | 0.001410 | 0.278285 | 0.005066 |
| 0.446 | 0.001947 | 0.250676 | 0.007765 |
| 0.455 | 0.002636 | 0.225081 | 0.011710 |
| 0.464 | 0.003508 | 0.201409 | 0.017416 |
| 0.473 | 0.004597 | 0.179572 | 0.025597 |
| 0.482 | 0.005940 | 0.159480 | 0.037249 |
| 0.491 | 0.007582 | 0.141048 | 0.053752 |
| 0.500 | 0.009567 | 0.124190 | 0.077036 |
| 0.509 | 0.011948 | 0.108823 | 0.109796 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.518 | 0.014782 | 0.094865 | 0.155818 |
| 0.526 | 0.018129 | 0.082234 | 0.220452 |
| 0.535 | 0.022056 | 0.070854 | 0.311294 |
| 0.544 | 0.026637 | 0.060645 | 0.439230 |
| 0.553 | 0.031949 | 0.051533 | 0.619976 |
| 0.562 | 0.038076 | 0.043443 | 0.876470 |
| 0.571 | 0.045110 | 0.036304 | 1.242573 |
| 0.580 | 0.053147 | 0.030045 | 1.768914 |
| 0.589 | 0.062290 | 0.024597 | 2.532390 |
| 0.598 | 0.072650 | 0.019895 | 3.651768 |
| 0.607 | 0.084345 | 0.015872 | 5.314061 |
| 0.615 | 0.097501 | 0.012467 | 7.820687 |
| 0.624 | 0.112248 | 0.009619 | 11.669793 |
| 0.633 | 0.128729 | 0.007269 | 17.710386 |
| 0.642 | 0.147090 | 0.005360 | 27.440680 |
| 0.651 | 0.167490 | 0.003840 | 43.618891 |
| 0.660 | 0.190093 | 0.002656 | 71.581940 |
| 0.669 | 0.215073 | 0.001758 | 122.308978 |
| 0.678 | 0.242612 | 0.001102 | 220.168067 |
| 0.687 | 0.272903 | 0.000642 | 424.780841 |
| 0.696 | 0.306147 | 0.000339 | 902.029476 |
| 0.704 | 0.342554 | 0.000155 | 2203.968448 |
| 0.713 | 0.382345 | 0.000057 | 6733.133048 |
| 0.722 | 0.425751 | 1.374E-05 | 30991.206743 |
| 0.731 | 0.473012 | 1.214E-06 | 389544.335280 |
| 0.740 | 0.524381 | 0.000000 | ************ |

Table (2.10.5)

Figure (2.10.13) Cross-plot (Linear scale) of water oil relative permeability for sample # 20

Figure (2.10.14) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 20

Figure (2.10.15) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 20

WATER-OIL RELATIVE PERMEABILITY

Sidewall Plugs

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.353 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.141 |
| Sample # | : 24 | Perm. To Water at resid. Oil Saturation, mD | : 0.086 |
| Porosity, (%) | : 11.63 | Residual Oil Saturation (Sorw), fraction | : 0.279 |
| Permeability,(mD) | : 0.471 | Net Confining Pressure, psi | : 5182 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 8566.00 | Shifted Depth, ft.in | : 8573.04 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.353 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.361 | 1.469E-09 | 0.930665 | 1.578E-09 |
| 0.368 | 5.032E-08 | 0.864828 | 5.819E-08 |
| 0.376 | 3.977E-07 | 0.802381 | 4.956E-07 |
| 0.383 | 1.724E-06 | 0.743219 | 2.319E-06 |
| 0.391 | 0.000005 | 0.687234 | 0.000008 |
| 0.398 | 0.000014 | 0.634324 | 0.000021 |
| 0.406 | 0.000030 | 0.584384 | 0.000051 |
| 0.413 | 0.000059 | 0.537313 | 0.000110 |
| 0.421 | 0.000108 | 0.493010 | 0.000218 |
| 0.428 | 0.000184 | 0.451376 | 0.000408 |
| 0.436 | 0.000299 | 0.412311 | 0.000726 |
| 0.443 | 0.000467 | 0.375720 | 0.001242 |
| 0.451 | 0.000702 | 0.341505 | 0.002055 |
| 0.458 | 0.001024 | 0.309571 | 0.003308 |
| 0.466 | 0.001456 | 0.279827 | 0.005203 |
| 0.473 | 0.002023 | 0.252178 | 0.008022 |
| 0.481 | 0.002756 | 0.226535 | 0.012165 |
| 0.488 | 0.003688 | 0.202808 | 0.018185 |
| 0.496 | 0.004859 | 0.180909 | 0.026857 |
| 0.503 | 0.006311 | 0.160750 | 0.039259 |
| 0.511 | 0.008093 | 0.142246 | 0.056895 |
| 0.518 | 0.010259 | 0.125314 | 0.081870 |
| 0.526 | 0.012869 | 0.109870 | 0.117129 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.533 | 0.015988 | 0.095834 | 0.166824 |
| 0.541 | 0.019686 | 0.083126 | 0.236825 |
| 0.548 | 0.024044 | 0.071668 | 0.335492 |
| 0.556 | 0.029146 | 0.061383 | 0.474813 |
| 0.563 | 0.035083 | 0.052197 | 0.672129 |
| 0.571 | 0.041956 | 0.044035 | 0.952783 |
| 0.578 | 0.049872 | 0.036827 | 1.354217 |
| 0.586 | 0.058947 | 0.030503 | 1.932498 |
| 0.593 | 0.069304 | 0.024994 | 2.772845 |
| 0.601 | 0.081076 | 0.020234 | 4.006929 |
| 0.609 | 0.094405 | 0.016159 | 5.842375 |
| 0.616 | 0.109440 | 0.012705 | 8.613728 |
| 0.624 | 0.126343 | 0.009814 | 12.874342 |
| 0.631 | 0.145284 | 0.007425 | 19.567422 |
| 0.639 | 0.166443 | 0.005483 | 30.357798 |
| 0.646 | 0.190012 | 0.003933 | 48.309650 |
| 0.654 | 0.216192 | 0.002725 | 79.351074 |
| 0.661 | 0.245196 | 0.001807 | 135.670534 |
| 0.669 | 0.277248 | 0.001135 | 244.307958 |
| 0.676 | 0.312586 | 0.000663 | 471.345551 |
| 0.684 | 0.351457 | 0.000351 | 1000.415586 |
| 0.691 | 0.394123 | 0.000161 | 2441.508803 |
| 0.699 | 0.440856 | 0.000059 | 7442.542028 |
| 0.706 | 0.491943 | 1.442E-05 | 34119.111691 |
| 0.714 | 0.547685 | 1.288E-06 | 425274.102373 |
| 0.721 | 0.608395 | 0.000000 | ************ |

Table (2.10.6)

Figure (2.10.16) Cross-plot (Linear scale) of water oil relative permeability for sample # 24

Figure (2.10.17) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 24

Figure (2.10.18) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 24

WATER-OIL RELATIVE PERMEABILITY

Sidewall Plugs

| Client | : WOC | Irreducible Water Saturation, fraction | : 0.370 |
| --- | --- | --- | --- |
| Well | : T1-31 | Perm. To Oil at Irred. Water Saturation, mD | : 0.090 |
| Sample # | : 30 | Perm. To Water at resid. Oil Saturation, mD | : 0.047 |
| Porosity, (%) | : 14.50 | Residual Oil Saturation (Sorw), fraction | : 0.260 |
| Permeability,(mD) | : 0.453 | Net Confining Pressure, psi | : 5182 |
| Formation | : Gir | Temperature, OF | : 213 |
| Depth, ft.in | : 8902.00 | Shifted Depth, ft.in | : 8909.04 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.370 | 0.000E+00 | 1.000000 | 0.000E+00 |
| 0.378 | 5.723E-08 | 0.927913 | 6.168E-08 |
| 0.385 | 9.986E-07 | 0.859667 | 1.162E-06 |
| 0.393 | 5.319E-06 | 0.795133 | 6.689E-06 |
| 0.400 | 1.743E-05 | 0.734183 | 2.373E-05 |
| 0.408 | 0.000044 | 0.676691 | 0.000065 |
| 0.415 | 0.000093 | 0.622533 | 0.000149 |
| 0.423 | 0.000175 | 0.571587 | 0.000307 |
| 0.430 | 0.000304 | 0.523731 | 0.000581 |
| 0.438 | 0.000494 | 0.478847 | 0.001032 |
| 0.446 | 0.000763 | 0.436817 | 0.001748 |
| 0.453 | 0.001131 | 0.397526 | 0.002845 |
| 0.461 | 0.001619 | 0.360862 | 0.004488 |
| 0.468 | 0.002253 | 0.326711 | 0.006896 |
| 0.476 | 0.003059 | 0.294966 | 0.010369 |
| 0.483 | 0.004066 | 0.265516 | 0.015312 |
| 0.491 | 0.005306 | 0.238258 | 0.022269 |
| 0.498 | 0.006813 | 0.213087 | 0.031974 |
| 0.506 | 0.008625 | 0.189900 | 0.045417 |
| 0.513 | 0.010780 | 0.168598 | 0.063938 |
| 0.521 | 0.013320 | 0.149084 | 0.089345 |
| 0.529 | 0.016290 | 0.131260 | 0.124102 |
| 0.536 | 0.019736 | 0.115033 | 0.171565 |
| 0.544 | 0.023708 | 0.100311 | 0.236340 |

| Water Saturation (fraction) | Relative Permeability to Water (fraction) | Relative Permeability to Oil (fraction) | Relative Permeability Ratio |
| --- | --- | --- | --- |
| 0.551 | 0.028257 | 0.087005 | 0.324780 |
| 0.559 | 0.033440 | 0.075026 | 0.445709 |
| 0.566 | 0.039312 | 0.064290 | 0.611480 |
| 0.574 | 0.045935 | 0.054714 | 0.839545 |
| 0.581 | 0.053370 | 0.046216 | 1.154796 |
| 0.589 | 0.061682 | 0.038717 | 1.593154 |
| 0.597 | 0.070941 | 0.032142 | 2.207100 |
| 0.604 | 0.081215 | 0.026416 | 3.074454 |
| 0.612 | 0.092580 | 0.021468 | 4.312395 |
| 0.619 | 0.105110 | 0.017229 | 6.100725 |
| 0.627 | 0.118885 | 0.013632 | 8.721024 |
| 0.634 | 0.133985 | 0.010613 | 12.624611 |
| 0.642 | 0.150496 | 0.008111 | 18.555402 |
| 0.649 | 0.168504 | 0.006066 | 27.777338 |
| 0.657 | 0.188099 | 0.004424 | 42.519010 |
| 0.664 | 0.209374 | 0.003130 | 66.882609 |
| 0.672 | 0.232423 | 0.002136 | 108.818379 |
| 0.680 | 0.257345 | 0.001393 | 184.733608 |
| 0.687 | 0.284241 | 0.000858 | 331.239978 |
| 0.695 | 0.313215 | 0.000490 | 638.582220 |
| 0.702 | 0.344372 | 0.000253 | 1360.541100 |
| 0.710 | 0.377823 | 0.000113 | 3354.371604 |
| 0.717 | 0.413679 | 0.000040 | 10431.108825 |
| 0.725 | 0.452056 | 9.107E-06 | 49637.591645 |
| 0.732 | 0.493071 | 7.364E-07 | 669609.536691 |
| 0.740 | 0.536846 | 0.000000 | ************ |

Table (2.10.7)

Figure (2.10.19) Cross-plot (Linear scale) of water oil relative permeability for sample # 30

Figure (2.10.20) Cross-plot (Semi-Log scale) of water oil relative permeability for sample # 30

Figure (2.10.21) Cross-plot (Semi-Log scale) of relative permeability Ratio for sample # 30

Section Eleven

Effective Oil Permeability at Initial Water Saturation under at Overburden Pressure

EFFECTIVE OIL PERMEABILITY AT INITIAL WATER SATURATION

Two of one and half inch diameter plugs and Five sidewall samples were selected from well T1-31 to perform effective oil permeability. The clean dry samples were loaded in a stainless-steel saturator and evacuated for at least 24 hours. Simulated formation brine containing sodium chloride with salinity 60,000 PPM, was introduced at the end of this period. The process was followed by pressurisation to 2500 psig to assist penetration. Gravimetric checks were made to ensure complete saturation. The saturated sample was individually placed in a hydrostatic core holder and confined at the designed overburden pressure and reservoir temperature (213 OF).

Mineral oil, type B having a viscosity of 40.44 centipoises at 25 Co was injected into the top of the sample individually and water production at the sample effluent was monitored. When the effluent was brine free and the flow was constant, oil permeability was measured, and the irreducible water saturation was calculated both gravimetric and volumetric and cross-referenced.  The generated data is presented in Table 2.11.1 and Figure 2.11.1.

EFFECTIVE OIL PERMEABILITY AT INITIAL WATER SATURATION UNDER AMBIENT AND OVERBURDEN PRESSURE

| Sample # | Shifted Depth (ft-in) | Oil Permeability (mD) | Air Permeability (mD) | Initial Water Sat. (fraction) |
| --- | --- | --- | --- | --- |
| 1 | 7692.06 | 0.120 | 0.640 | 0.244 |
| 2 | 7693.09 | 0.107 | 0.561 | 0.241 |
| 9 | 7844.04 | 0.344 | 0.880 | 0.207 |
| 16 | 8157.04 | 0.273 | 0.997 | 0.180 |
| 20 | 8307.04 | 0.151 | 0.781 | 0.304 |
| 24 | 8573.04 | 0.141 | 0.471 | 0.353 |
| 30 | 8909.04 | 0.09 | 0.453 | 0.370 |

Table 2.11.1

Figure 2.11.1

Section Twelve

Determination of Young’s Modulus, Poisson’s ratio & Acoustic Velocity

INTRODUCION

The objective of this experimental work was to determine the static elastic constants, Young’s modulus and Poisson’s ratio, on three samples from well T1-31, Concession 31.

The following two sets of tests were conducted:

Test one:

Determination of the static Young's Modulus and Poisson's Ratio at Unconfined Stress.

The static test was conducted (at unconfined ambient conditions, minimal pressure) on one sample supplied. The core samples were dry for these tests.

Test Two:

Determination of the static Young's Modulus & Poisson's Ratio at 5 stresses.

The static test was conducted at five effective stress values, (1000, 2000, 3000, 4000 &4839) on the two samples. These samples were fully saturated with brine before testing, using the brine salinity of 60,000 ppm.

Sample Preparation:

Before conducting the rock mechanics tests, the samples have been prepared, the diameter of the tested samples was two inches, and the length is twice the diameter or more.

Three core samples (two inches diameter) were drilled in vertical directions and cut from the preserved cores as requested by Waha Oil Company using diamond core bit with water as the bit coolant and lubricant.

The samples were extracted of hydrocarbons using toluene, leached of salt using methanol, and oven dried at 80 oC. Silver Nitrate solution was used to ensure the removal of salts existing in the pore spaces

Unconfined or Uniaxial Compressive Strength Test:

The unconfined compressive test or uniaxial compressive strength test (UCS) is performed on cylindrical rock sample according to precise standardized procedure (ISRM Suggested Methods) in terms of sample geometry, sample end preparation and moisture conditions. In our laboratories, sample diameters are about two inches. The dimensions of the sample could affect rock strength: in most cases, increasing a sample size determines a decrease in rock strength. The sample length, it has to be at least twice the diameter, that is L/D>2. Sample ends preparation is important, because it affects the stress distribution on the sample surfaces. End plate flatness determines a correct perpendicular loading transmission on rock samples. Moisture conditions are critical in rock strength response. Many experimental results have shown that moisture conditions, together with the nature of the saturating liquid, can change the rock strength because of different physical processes such as surface energy changes, friction reduction and corrosion.

Once the sample preparation has been completed following the above mentioned procedures, the unconfined or uniaxial compressive strength can be performed by increasing the load along the axis of the sample, while monitoring the applied force and the consequent deformation.

EXPERIMENTAL FACILITY

The equipment used for these experiments consists of a servo-controlled stiff testing machine, Hoek cell with a pressure intensifier, strain gauge conditioning equipment and a data logging system.

## Servo-controlled Stiff Testing Machine and Pressure Intensifier

The loads applied to the samples are generated using an RTR-1000 (Rapid triaxial test system. This machine consisted of a straining frame holding a hydraulic ram, platens and a load cell. The ram operated vertically with the load cell located in the crosshead of the straining frame. The position of the ram was monitored by an electronic LVDT and this position signal together with that from the load cell allowed the flow of hydraulic oil to the ram to be controlled. Thus the load and rate of loading were controlled.

Hoek Cell

This cell provided a means of applying confining pressure to the sample. It consisted of a steel cell rated to 68.9MPa (10000psi) within which a polyurethane sleeve was located. Confining pressure was transmitted to the radial surface of the sample via hydraulic oil contained within the annulus between the body of the cell and the sleeve. The samples were located within the sleeve and the hydraulic oil introduced or removed from the cell by the connection to the pressure intensifier. The pressure and volume of oil introduced or removed from the cell were monitored electronically.

## Strain Gauge Conditioning Equipment

Strain gauge conditioning equipment was used to determine static elastic constants. This tool, which consists of a series of Wheatstone bridges and amplifiers, creates amplified electronic signals that are fed to LED meters. The two vertical gauges on the sample became the active arm of a Wheatstone bridge. The other two arms of the bridge were formed from passive gauges located on a separate dummy sandstone core that was not loaded, in effect a half bridge was formed. The conditioning electronics excited the bridge and amplified the signal produced. The amplified signals were fed to LED meters and the data logged during the tests. This set up was identical for the horizontal gauges.

## Data Logging Equipment

This consisted of a PC with data logging hardware and software. The parameters of interest during the tests (axial load, axial deformation) were recorded at appropriate time intervals.  The files produced were compatible with Excel spreadsheet software.

EXPERIMENTAL PROCEDURE AND RESULTS

## Static Elastic Constants at Increasing Stress States

One electrical strain gauge aligned horizontally and two other gauges aligned vertically were bonded to the outside of the samples to measure the sample length and diameter changes with stress to determine the static elastic constants. These data were used to determine the static elastic constants. The electrical connections of the gauges were soldered to plastic coated electrical foil and the whole assembly coated with epoxy resin to protect the gauges and connections for use in the test cell.

The samples were loaded between the platens within the stiff testing machine. The axial load was then increased by using a constant rate to produce a response in the vertical and horizontal gauges. The confining pressure was kept constant during this variation in load. The data from the tests were used to determine the change in axial and radial strain at each stress level.

The modulus of elasticity, E, is calculated as the ratio of the change in axial stress to the change in axial strain (i.e., change in the sample length divided by the length, L/L), which occurs, using Equation 1.

,	(1)

Where 1 is the change in axial stress and 1 is the change in axial strain.

Poisson’s ratio, , is calculated as the ratio of the accompanying change in radial strain (i.e., change in the sample diameter divided by the diameter, D/D) to the change in axial strain as follows:

,	(2)

Where 3 is the change in radial strain.

A summary of the test results including the derived constants for each sample is shown in Table (2.12.1). The data in Figures (2.12.1) to (2.12.2) were used to determine the change in axial and radial strain.

Static Elastic Constants at Increasing Stress

The samples were loaded into the Hoek cell and the stress increased to the first stress level hydrostatically.  The axial load was increased by using a constant rate to produce a response in the vertical and horizontal gauges.  The confining pressure was kept constant during this variation in load.  This procedure was repeated at each of the stress levels and the data from the tests were used

to determine the change in axial and radial strain at each stress level.  The elastic constants were calculated following the procedure described above.

A summary of the test results including the derived constants for each sample is shown in Table (2.12.2). The data in Figures (2.12.3) to (2.12.22) was used to determine the change in axial and radial strain at each of the test stresses. The measurements were conducted at five different values for the overburden pressure; these pressures are 1000, 2000, 3000, 4000 and 4839 psi.

## Compression (P) and Shear (S) wave velocity system

At each stress level, the compression and shear waves velocities are calculated based on the elastic constants

Vp= K+4G/3ρ                       Vs=  Gρ

Where K is bulk modulus, G shear modulus and ρ density of a rock.

Summary of static elastic constants at unconfined stress

| Sample No. | Depth Interval (ft.in) | Length (in) | Diameter (in) | Bulk Density (g/cm3) | Poisson’s Ratio | Young's Modulus (Gpa) | Bulk Modulus (Gpa) | Shear Modulus (Gpa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 7707.02 – 7707.07 | 4.27 | 1.98 | 2.51 | 0.25 | 39.08 | 23.88 | 14.47 |

Table (2.12.1)

Note:

All samples were tested with an axial loading strain rate of 0.05% / min.

Pore pressure = 0 psi for all tests.

Figure (2.12.1)                                                                                                                  Figure (2.12.2)

Summary of static elastic constants testing at increasing stress

| Sample No. | Depth Interval (ft.in) | Effective Confining Pressure (psi) | Length (in) | Diameter (in) | Bulk Density (g/cm3) | Poisson’s Ratio | Young's Modulus (Gpa) | Bulk Modulus (Gpa) | Shear Modulus (Gpa) | P Wave Velocity (m/s) | S Wave Velocity (m/s) | vp/vs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7686.10 – 7687.03 | 1000 | 4.28 | 1.98 | 2.43 | 0.21 | 24.58 | 13.92 | 10.19 | 3364.5 | 2047.8 | 1.64 |
|  |  | 2000 |  |  |  | 0.22 | 30.26 | 17.92 | 12.41 | 3766.1 | 2259.9 | 1.67 |
|  |  | 3000 |  |  |  | 0.22 | 32.85 | 19.85 | 13.41 | 3940.4 | 2349.2 | 1.68 |
|  |  | 4000 |  |  |  | 0.21 | 34.35 | 19.93 | 14.15 | 3995.7 | 2413.1 | 1.66 |
|  |  | 4839 |  |  |  | 0.27 | 37.94 | 26.79 | 14.83 | 4377.4 | 2470.4 | 1.77 |
| 4 | 7712.08 – 7713.02 | 1000 | 4.27 | 1.99 | 2.60 | 0.28 | 34.99 | 27.10 | 13.61 | 4171.6 | 2287.9 | 1.82 |
|  |  | 2000 |  |  |  | 0.25 | 40.21 | 26.57 | 16.10 | 4298.3 | 2488.4 | 1.73 |
|  |  | 3000 |  |  |  | 0.25 | 48.76 | 25.79 | 15.74 | 4241.6 | 2460.5 | 1.72 |
|  |  | 4000 |  |  |  | 0.24 | 51.82 | 25.96 | 16.49 | 4294.3 | 2518.4 | 1.71 |
|  |  | 4839 |  |  |  | 0.29 | 69.21 | 53.92 | 26.88 | 5875.7 | 3215.5 | 1.83 |

Table (2.12.2)

Note:  All samples were tested with an axial loading strain rate of 0.05% / min & Pore pressure = 0 psi for all tests.

Figure (2.12.3)                                                                                                                       Figure (2.12.4)

Figure (2.12.5)                                                                                                                       Figure (2.12.6)

Figure (2.12.7)                                                                                                                       Figure (2.12.8)

Figure (2.12.9)                                                                                                                       Figure (2.12.10)

Figure (2.12.11)                                                                                                                       Figure (2.12.12)

Figure (2.12.13)                                                                                                                       Figure (2.12.14)

Figure (2.12.15)                                                                                                                       Figure (2.12.16)

Figure (2.12.17)                                                                                                                       Figure (2.12.18)

Figure (2.12.19)                                                                                                                       Figure (2.12.20)

Figure (2.12.21)                                                                                                                       Figure (2.12.22)

Conclusions

Conclusions

In conclusion, after doing all the required analysis, the obtained results show the following:

- Porosity and Permeability for Core Plug Samples:

- In this study the controls on porosity and permeability of Gir formation of well No. T1-31, regarding its texture and cementation, petrophysical classification and the effect of subdivision of the petrophysical rock types. Horizontal permeability range between (0.001 – 4.152) mD, mostly ranges (0.001 – 0.01) mD, the qualitative classification is poor.

- The porosity values (2.18% – 16.86%), mostly ranges from 6 % to 12 %, the qualitative classification is poor to fair.

- Finally, the vertical permeability samples classified as poor as well according to the values obtained (0.002 – 0.069) mD.

- Porosity and Permeability for Sidewall Plug Samples:

- Horizontal permeability range between (0.017 – 2.1) mD, mostly ranges (0.04 – 0.32) mD, the qualitative classification is poor.

- The porosity values (1.02% – 21.4%), mostly ranges from 10 % to 18 %, the qualitative classification is fair to good.

- In general, all plug samples were low permeability / or fractured. Therefore, the hydraulic flow unit was not performed.

- The results of porosity and horizontal permeability measurements for twelve samples selected from well T1-31 as a function of overburden pressure are presented in tables (2.1.1 – 2.1.12) and in figures (2.1.1-2.1.12). The data show a decrease in porosity with increasing overburden pressure. The data for core plugs exhibit a reduction in absolute permeability with increasing overburden pressure. These reductions in absolute permeabilities with increasing overburden pressure can be attributed to the decrease in pore size and compressibility of the rock.

- It was observed from the measurement of water permeability as a function of throughput that the water permeability decreases as the pore volume injected increases for all samples.

- In performing laboratory measurements, changes have been observed in the resistivity of fluid filled reservoir rocks as a result of changing overburden pressure conditions. Both formation resistivity factors (FF) and cementation exponent (m) were found to increase with confining pressure for Gir formation. In fact, this relation is very clear in this study in section 4 and section 6 of the second part of this report.  The cementation exponent of four of one and half inch diameter plugs at ambient conditions increased from 1.79 to 1.85 (+3.2%) as a result of increasing the pressure up to 4839 psi, and for one inch side wall plugs increased from 1.77 to 1.84 (+3.8%) as a result of increasing the pressure up to 5182 psi.

- The average value saturation exponent (n)=1.50 for one and half inch diameter plugs and 1.44 for one inch sidewall plugs of Gir formation. For more details see tables (2.5.1-2.5.12) and figures (2.5.1-2.5.14) in sections five of the second part of this report.

- In the present study, centrifuge capillary pressure (sections 7 & 8) show clearly that the low permeability samples gave high irreducible water saturation and residual oil saturation.

- High pressure mercury porosimeter is useful to get accurate capillary pressure profile and in determining the relative proportion of micro porosity in reservoir rocks.

- From High pressure mercury injection test, it observed that most samples have pore size distribution in micropores range.

- Water- Oil relative permeability for 1.5’’ Core Plug Samples:

- The values of relative permeability to oil at Swi for sample 1 & 2 is close where (Kro@Swi= 0.120 for sample 1 and 0.107 for sample 2)

- The values of relative permeability to water at Sor for sample 1 & 2 is approximately equal where (Krw@Sor= 0.072 for sample 1 and 0.070 for sample 2)

- The values of irreducible water saturation (Swi) for sample 1 & 2 is approximately equal where (Swi= 24.4% for sample 1 and 24.1% for sample 2)

- The values of Residual oil saturation (Swi) for sample 1 & 2 is different where (Sor= 19.5% for sample 1 and 12.9% for sample 2)

- Water- Oil relative permeability for 1.0’’ Core Plug Samples:

- The values of relative permeability to oil at Swi for side wall samples range is (0.09-0.344 mD)

- The values of relative permeability to water at Sor for side wall samples range is (0.047-0.237 mD)

- The values of irreducible water saturation (Swi) for for side wall samples range is (18%-36.9%)

- The values of Residual oil saturation (Swi) for for side wall samples range is (15%-27.9%)

- The effective Oil permeability after establishment of initial water saturation ranges from 0.09 mD to 0.344 mD and the permeability profile showed converging trend between air and effective oil permeability

- Poisson’s range was (0.25) at unconfined stress for sample#3 and the range was (0.21 to 0.29) for the samples at increasing stress.

- The Young’s Modulus range was (39.08 Gpa) at unconfined stress for sample#3 and it showed an increase for the samples at increasing stress.

References

References

- “API Recommended Practice 40, February 1998, section 5, 6”.

- Teew, D., “Prediction of Formation Compaction from Laboratory Compressibility Data.”

SPEJ (Sept.1971) 263-7; Trans., AIME, 251.

- Nieto, J.T. et al., “Core Compaction Correction - An approach.” SCA Advances in core evaluation 1990.

- Hassler, G.L., Brunner, E. (1945),"Measurement of capillary pressures in small core samples." Trans. AIME, 160, 114-123.

- B.F Swanson, The log Analyst, November 1985 “Micro porosity in Reservoir rocks: Its          Measurement and Influence on Electrical Resistivity.”

- Burdine, N.T.,Gournay, L.S. and Reichertz, P.P., (1950), “Pore Size Distribution of

Petroleum Reservoirs.” AIME, Vol. 189, pp195-204.

- John Shafer1 & John Neasham2, Mercury Porosimetry Protocol for Rapid Determination of Petrophysical and Reservoir Quality Properties, SCA 2000-21.

- Pittman, E. D., 1971, “Microporosity in Carbonate Rocks” AAPG Bull., Vol. 55, No.10   (October,1971), pp.1873-1881.

- SENDRA Software User Guide version 2018.2, PRORES AS, Norway.